3GPP TSG-RAN WG2 Meeting #128　　　　　　                                    R2-2410702

Orlando, USA, 18th – 22nd Nov, 2024

Agenda item:	8.6.3

Source: 	Sharp

Title: 	L1 event triggered measurement reporting

Document for:	Discussion and Decision

Introduction

WID for mobility enhancement was approved and RAN2 started discussion on the WI. 
Also, RAN2 reached some agreements follows in RAN2#127bis [1].

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this paper, we discussed measurement event evaluation related potential issues.

Discussion

TimeToTrigger

In the last meeting, RAN2 discussed TimeToTrigger function for event triggering measurement for LTM. There are two issues about TimeToTrigger.

TTT operation per beam vs per candidate cell

Regarding the discussion about whether TTT is applied per beam or per candidate cell, the NW needs to know which beam has satisfied the condition for the duration of the TTT in the case of LTM cell switch. If beam level TTT is applied, NW can know which beam has satisfied the condition for duration of the TTT. In addition, basically, because the event triggered measurement for LTM is a beam level mechanism (e.g. beam level evaluation), it is more reasonable to apply TTT at the beam level. Therefore, we propose to apply TTT at the beam level for event triggered measurement.

Proposal 1. TTT is applied at the beam level.

TTT operation on current beam changing

It is needed to discuss whether TTT should be reset or not when the current beam is changed. If TTT is reset when the current beam is changed, there may be instances where reporting is not triggered, even though the quality of the current beam is poor for the duration of the TTT. Therefore, to avoid this situation, TTT should not be reset when the current beam is changed.

Proposal 2. TTT is not reset when current beam is changed.

Contents in MR MAC CE.

In RAN2#127 meeting, it was agreed that MAC CE is used as container of measurement reporting. In RAN2#127bis meeting, we discussed contents in MAC CE, and agreed that MR MAC CE include:

- Beam information

- Beam quantity

- Triggered event information.

But there are a few unclear points, so in this section, we will address these issues related to MR MAC CE.

How to indicate beam information in MR MAC CE ?

In RAN2#127bis, it was agreed that MR MAC CE include beam information, but it is not clear how to indicate beam information. In my understanding, in event triggered measurement reporting, there are two methods to indicate which beam is reported to NW.

Method 1 is that SSBRI/CRI is included in MR MAC CE. In Rel-18 LTM, this method is used to indicate which beam is reported to gNB. In Rel-18, LTM-CSI-ResourceConfigId is included in both LTM-CSI-ResourceConfig and LTM-CSI-ReportConfig. By the structure of this configuration, the gNB can know the resource set by which resource was used to send the MR UCI.

Observation 1. In Rel-18 LTM, SSBRI is included in MR UCI to indicate which beam is reported.

Rel-19 MR MAC CE is event triggered basis but the resource used to send the MR MAC CE has not been determined yet. However, if MR MAC CE includes ReportConfigID, gNB can know the associated resource set. In addition, it is agreed that, for association between measurement resource configuration and measurement reporting configuration, the method in Rel-18 LTM is reused (i.e. one reporting configuration is not associated to multiple resource configuration). Therefore, if ReportConfigID is included in MR MAC CE, we can use method 1.

Observation 2. In Rel-19 LTM, if ReportConfigID is included in MR MAC CE, SSBRI/CRI can be included in MR MAC CE to indicate which beam is reported.

Method 2 is candidate ID and RS id is included in MR MAC CE. gNB can know which beam is reported based on this information. We can use this method even if ReportConfigId is not included in MR MAC CE.

Observation 3. Candidate ID and RS id can be included in MR MAC CE to indicate which beam is reported to gNB.

Proposal 3. MR MAC CE includes SSBRI/CRI or candidate id and RS id to indicate which beam is reported.

Whether the measurement result per cell is included in MR MAC CE or not ?

In RAN2#127, it was agreed that beam level measurement result is used LTM event evaluation and cell level measurement result is not used LTM event evaluation. It is obvious to use same level (i.e. beam level) measurement result for evaluation and reporting. In addition, gNB can configure both L1 measurement and L3 measurement at the same time if needed. Therefore, we propose that MR MAC CE includes only beam level measurement result, but not cell level measurement result.

Proposal 4. MR MAC CE does not include cell level measurement result.

Beam which did not meet event is included in MR MAC CE or not ?

It is needed to discuss whether MR MAC CE can include beam information which beam did not fulfill the event. This issue is related to the TTT issues. As proposed above, If TTT is applied per cell and TTT is not reset when the current beam is changed, there are multiple beams related to one TTT, so these beams should be included in one MR MAC CE and reported to NW.

Proposal 5. When TTT is fulfilled, all beams which belong to the same candidate cell and meet the condition should be included in MR MAC CE.

How to reduce MR size ?

After discussion which contents are included in MR, RAN2 should discuss how to reduce MR size. For example, we compare the size of MR when method 1 (i.e. SSBRI/CRI) is used and method 2(Candidate cell ID + RS ID) is used. When several beams in MR MAC CE belong to the same candidate cell, method 2 can be used to reduce size of the MR MAC CE. On the other hand, method 1 can not be used to reduce size in same situation. Like this, RAN2 discuss how to include information to make the size of MR MAC CE smaller.

Observation 4. Method 2 may reduce MR MAC CE size when several beams in MR MAC CE belong to the same candidate cell.

Proposal 6. RAN2 discuss how to include information to make the size of MR smaller after discussion which contents are included in MR.

Conclusion

Proposal 1. TTT is applied at the beam level.

Proposal 2. TTT is not reset when current beam is changed.

Proposal 3. MR MAC CE include  s SSBRI/CRI or candidate id and RS id to indicate which beam is reported.

Proposal 4. MR MAC CE does not include cell level measurement result.

Proposal 5. When TTT is fulfilled, all beams which belong to the same candidate cell and meet the condition should be included in MR MAC CE.

Proposal 6. RAN2 discuss how to include information to make the size of MR smaller after discussion which contents are included in MR.

Reference

[1] R2_127bis_R19 MOB_Session Notes (Kyeongin Jeong)_EoM.docx