3GPP TSG-RAN-WG2 Meeting #128	R2-2410703

Orlando, USA, 18th– 22nd Nov, 2024

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Discussion on issues for supporting inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. In RAN2#127, RAN2 made agreements as follows [2];

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In this paper, we discuss inter-CU LTM related issues.

Discussion

Inter-CU SCG LTM

RAN2 agreed that source SN initiates inter-CU SCG LTM preparation, source SN generates SCG part configuration and MN includes it into its MN RRC configuration message. So, candidate configurations are sent via MN RRC Reconfiguration message. RAN2 also agreed that the LTM cell switch command MAC CE is sent by source SN for inter-CU SN LTM. Here, in legacy LTM, LTM configuration sent via MCG RRC message is maintained in ltm-Config associated with MCG. It is the same with CHO/CPAC mechanism, i.e. entries of conditional reconfiguration sent via SRB1 is stored in VarConditionalReconfig associated with MCG and entries of conditional reconfiguration sent via SRB3 (or embedded RRCReconfiguration sent via SRB1) is stored in VarConditionalReconfig associated with SCG. Based on this premise, in case of inter-CU SCG LTM, UE needs to apply candidate configuration associated with MCG RRC based on the LTM CSC MAC CE which SCG MAC received. This behaviour is not considered in Rel-18. Therefore, RAN2 needs to discuss how to handle candidate configuration for inter-SCG LTM. For example, UE stores candidate configuration received via MN RRC message in ltm-Config associated with SCG RRC.

If the UE maintains inter-SN LTM candidate configuration in ltm-Config associated with SCG RRC, UE always (i.e. whenever the UE removes/adds/modifies the configuration) needs to determine that the candidate configuration provided via MN RRC format is inter-SN LTM candidate configuration. It means that NW always indicates whether or not the candidate configuration is inter-SN LTM candidate configuration. If the SN configures UE with intra-SN LTM candidate configuration via SN RRC format, the SN cannot ensure the validity of each configuration and that there is no ID collision since the inter-SN LTM candidate configurations are modelled as MN RRC format. Similarly, in the legacy conditional reconfiguration, inter-SN CPAC is maintained in MCG VarConditionalReconfiguration.

If the UE maintains inter-SN LTM candidate configuration in ltm-Config associated with MCG RRC, UE needs to understand whether the LTM is triggered on MCG or SCG based on other than the information whether the candidate configuration indicated by lower layer is included MCG ltm-Config or SCG ltm-Config.

Observation 1. Candidate configuration for inter-SCG LTM is provided by MN RRC message and applied by LTM CSC MAC CE which is sent from source SN.

Proposal 1. RAN2 discuss how to handle candidate configuration for inter-SCG LTM.

And, there can be multiple LTM candidate cells in single target candidate CU. In that case, the multiple LTM candidate cells are intra-CU LTM candidates from the target candidate CU’s perspective. It means that it is managed in ltm-Config associated with SCG in Rel-18 LTM. However, from the perspective of source CU, it should be included in MN RRC message.

Proposal 2. RAN2 assumes that target CU’s intra-CU LTM candidate configuration can be mixed in MN RRC message.

Early DL synchronization

RAN2 agreed that R18 TCI state activation/deactivation MAC CE is reused. However, it is not discussed which node sends the MAC CE. For inter-CU MCG LTM case, the MN-DU should send the MAC CE since MCG receives L1 measurement report for inter-CU MCG LTM and this MAC CE is used for early DL sync with the candidate PCell. For inter-CU SCG LTM case, we consider four cases, as shown in Table 1. TCI state information for early synchronisation is indicated by ltm-TCI-Info which is configured per ltm-Candidate, i.e. this information is provided by MN RRC format.

If we assume that inter-CU SCG LTM configuration is maintained in “MCG” ltm-Config and the “MN” sends the MAC CE for SCG early synchronisation, the MN needs to receive L1 measurement report directly from the UE or indirectly from the SN; Otherwise, the MN sends the MAC CE based on intuition or L3 measurement report.

If we assume that inter-CU SCG LTM configuration is maintained in “SCG” ltm-Config and the “MN” sends the MAC CE for SCG early synchronisation, the MN needs to receive L1 measurement report directly from the UE or indirectly from the SN, and UE needs to decide that the TCI state ID indicated by the received MAC CE is associated with SCG ltm-Config.

If we assume that inter-CU SCG LTM configuration is maintained in “MCG” ltm-Config and the “SN” sends the MAC CE for SCG early synchronisation, UE needs to decide that the TCI state ID indicated by the received MAC CE is associated with MCG ltm-Config.

And maintaining inter-CU SCG LTM configuration sent via MN RRC format in SCG ltm-Config provides complex operation from UE and NW perspectives. We show summary of this analysis as below;

Table 1. Impact Analysis related to TCI state activation/deactivation

|  | MN sends TCI state activation/deactivation MAC CE for SCG early sync | SN sends TCI state activation/deactivation MAC CE for SCG early sync |
| --- | --- | --- |
| MCG ltm-Config maintains inter-CU SCG LTM configuration | - MN receives L1 measurement report directly/indirectly | - UE decides that the TCI state ID included in the MAC CE is associated with MCG ltm-Config. |
| SCG ltm-Config maintains inter-CU SCG LTM configuration | - MN receives L1 measurement report directly/indirectly - UE decides that the TCI state ID included in the MAC CE is associated with SCG ltm-Config - Management by MN/UE becomes difficult | - Management by MN/UE becomes difficult |

According to the above analysis, inter-CU SCG LTM configuration provided by MN RRC format should be maintained in MCG ltm-Config. And, it is a waste that MN receives L1 measurement report directly/indirectly since NW signalling increases. L1 measurement report should also be sent to the source SN since it was agreed that the source SN sends LTM CSC MAC CE for inter-CU SCG LTM. Therefore, in the same way with inter-CU MCG LTM case, the SN-DU should send the TCI state activation/deactivation MAC CE.

Proposal 3.Source MN sends TCI state activation/deactivation MAC CE for inter-CU MCG LTM, and source SN sends TCI state activation/deactivation MAC CE for inter-CU SCG LTM.

To achieve the above functions, UE should send L1 measurement report to the corresponding gNB. In other words, L1 measurement reports for intra/inter-MN LTM should be sent to the source MN and L1 measurement reports for intra/inter-SN LTM should be sent to the source SN.

Proposal 4. L1 measurement reports for inter-MN LTM are sent to the source MN and L1 measurement reports for inter-SN LTM are sent to the source SN.

For inter-SN LTM cell switch, LTM CSC MAC CE is sent from source SN. For early DL synchronisation with candidate PSCell, we proposed that source SN should send the TCI state activation/deactivation MAC CE. And from LTM configuration management perspective, MCG RRC ltm-Config should maintain inter-SN LTM configuration. In this case, UE needs to determine that the MAC CE sent from current SN is to be applied to candidate configuration included in MCG RRC ltm-Config. In this case, there are some solutions to resolve this issue as below;

Alt 1. LTM CSC MAC CE sent from the source SN indicates whether the candidate configuration indicated by the candidate cell ID field is maintained in MCG RRC ltm-Config or SCG RRC ltm-Config.

Alt 2. All SCG LTM configurations are maintained in MCG RRC ltm-Config if at least one inter-SN LTM configuration is configured.

For Alt 1, to achieve this, SN needs to know the ID of candidate configuration contained in MCG RRC ltm-Config. However, only 1-bit information is needed to indicate whether the candidate configuration indicated by the candidate cell ID field is maintained in MCG RRC ltm-Config or SCG RRC ltm-Config. And since UE can determine whether the candidate configuration indicated by the candidate cell ID field is maintained in MCG RRC ltm-Config or SCG RRC ltm-Config, UE can maintain MCG RRC ltm-Config or SCG RRC ltm-Config, apply an appropriate candidate configuration and NW can use 8+8 entries for two ltm-Config. For Alt 2, the number of total LTM candidate configurations is limited to 8 since SCG RRC ltm-Config cannot be used. However, no additional signalling is needed but new UE behaviour is needed. The benefits are the same, but we think Alt 1 has fewer disadvantages.

Proposal 5. Inter-SN LTM candidate configuration provided via MN RRC Reconfiguration message is managed in ltm-Config associated to MCG RRC.

Proposal 6. RAN2 discuss which solution to be used to determine that the MAC CE sent from current SN is to be applied to candidate configuration included in MCG RRC ltm-Config;

- Alt 1. LTM CSC MAC CE sent from SN indicate whether the candidate configuration indicated by candidate cell ID field is maintained in MCG RRC ltm-Config or SCG RRC ltm-Config.

- Alt 2. All SCG LTM configurations are maintained in MCG RRC ltm-Config if at least one inter-SN LTM configuration is configured.

Execution

In the previous meeting, some companies suggest that the PDCP re-establishment should be performed per RB. As we know, legacy PDCP-reestablishment is included in a DRB configuration associated with a DRB ID. That’s why some companies propose it. Here, we consider the case when PDCP re-establishment is NOT needed even if it’s inter-CU LTM. In our view, if the UE is configured with DC, if it is inter-SN LTM and if the DRB is terminated to the MN, PDCP which constitutes the DRB associated with SCG is not needed to perform re-establishment (DRB associated with MCG is already excluded).

Observation 2. If the UE is configured with DC, if it is inter-SN LTM and if the DRB is terminated to the MN, PDCP which constitutes the DRB associated with SCG is not needed to perform re-establishment.

If the PDCP re-establishment is always performed upon inter-SN LTM, MN security key should be refreshed to avoid using same COUNT value for same security key. Such operation is wasteful from NW and UE perspectives. So, we propose that PDCP recovery should be performed for the DRB configured with MN security key upon inter-SN LTM. Otherwise, PDCP re-establishment should be performed.

Proposal 7. PDCP recovery should be performed for the DRB which is associated with SCG and is configured with MN security key upon inter-SN LTM.

Inter-MN LTM with SCG

There are some remaining issues to support DC scenario. For inter-MN LTM case, if intra-SN LTM configuration is configured, intra-SN LTM configuration may not be needed. According to the WID,

- Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

In case that SN is released, it means that SCG configuration is released. Therefore, legacy intra-SN LTM configuration should be released. In case that SN is unchanged, it means that SCG configuration is unchanged or modified but SN is not changed. In this case, intra-SN LTM configuration isn’t released upon inter-MN LTM execution. If intra-SN LTM configuration is reused, it should be stored. And if intra-SN LTM configuration isn’t reused, it can be should be released. Anyway, it is up to SN whether the UE releases or maintains intra-SN LTM configuration, and it can be achieved NW implementation using explicit signalling.

Proposal 8. UE expects SN ensures that intra-SN SCG configuration is valid until it’s released even if inter-MN LTM is triggered.

For inter-MN LTM with PSCell addition, it might be out of the above WID scope. And it needs the handling which is similar with MN-initiated inter-CU PSCell change. Therefore, RAN2 shouldn’t pursue it.

Proposal 9. RAN2 doesn’t pursue inter-MN LTM with PSCell addition.

To avoid inter-MN LTM with PSCell addition, MN should ensure there is no SCG LTM configuration if UE is not configured with SCG.

Proposal 10. NW ensures there is no SCG LTM configuration if UE is not configured with SCG.

Subsequent LTM

The intention of Rel-18 subsequent LTM is to avoid RRC configuration so as to reduce the handover interruption. Thus, subsequent intra-CU LTM was supported in Rel-18. Based on the definition of subsequent LTM [3], LTM preparation phase including RRC reconfiguration (LTM candidate configuration) is neglected in subsequent LTM. In other words, the UE keeps the RRC reconfiguration (LTM candidate configuration) in subsequent LTM.

In RAN2#125bis, it was agreed that Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. In RAN2#127, it was agreed that as in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least an inter-CU LTM cell switch procedure where the UE is not configured with DC, unless the LTM candidate configurations are explicitly released by the network. However, in case that an inter-CU LTM occurs, the gNB-CU is switched from the source gNB-CU to the target gNB-CU, where the candidate CU of the source gNB-CU may not be applicable as the candidate CU of the target gNB-CU. Moreover, the number of LTM candidate configurations is limited at most 8,so it is possible that when subsequent LTM is applied in Rel-19, the RRC configuration transmitted by the source gNB-CU may not be applicable for the target gNB-CU after an inter-CU cell switch in the subsequent LTM.

As a result, it is desire to ensure the accuracy of the RRC configuration content applicable to the target gNB-CU and to avoid RRC configuration overhead during the mixture of subsequent inter-CU LTM and subsequent intra-CU LTM. Hence, it is proposed that the validity of the subsequent LTM should be monitored. For example, the number of subsequent cell switches should be limited. The duration of subsequent cell switches to be allowed should be limited.

Proposal 11. The validity of the subsequent LTM should be monitored.

Proposal 12. The approaches for monitoring the validity of the subsequent LTM can consider the limited number of subsequent cell switches and the limited duration allowed for subsequent cell switches.

LTM and L3 HO

In RAN2#127, it was agreed that L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE while inter-CU LTM is being configured (w/o DC). It was also agreed that when performing L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations. That is, the UE simultaneously keeps the RRC configuration of L3 mobility (HO and CHO) and inter-CU LTM. Depending on the triggering timing, the UE performs either L3 mobility or inter-CU LTM. However, it is the MAC entity of the UE that receives the LTM CSC MAC CE from the source DU to trigger the inter-CU LTM, while the RRC layer of the UE receives the L3 HO command from the source CU to trigger L3 mobility. Based on Rel-18 intra-CU LTM, when the intra-CU LTM is triggered, the MAC entity of the UE subsequently notifies the RRC layer of the UE. Therefore, we propose that when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE further notifies the RRC layer of the UE, by following the rationale in Rel-18. Afterwards, RAN2 further discuss the cases when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and the execution order of L3 HO and inter-CU LTM.

Proposal 13. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 14. RAN2 further discuss

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. Candidate configuration for inter-SCG LTM is provided by MN RRC message and applied by LTM CSC MAC CE which is sent from source SN.

Proposal 1. RAN2 discuss how to handle candidate configuration for inter-SCG LTM.

Proposal 2. RAN2 assumes that target CU’s intra-CU LTM candidate configuration can be mixed in MN RRC message.

Proposal 3.Source MN sends TCI state activation/deactivation MAC CE for inter-CU MCG LTM, and source SN sends TCI state activation/deactivation MAC CE for inter-CU SCG LTM.

Proposal 4. L1 measurement reports for inter-MN LTM are sent to the source MN and L1 measurement reports for inter-SN LTM are sent to the source SN.

Proposal 5. Inter-SN LTM candidate configuration provided via MN RRC Reconfiguration message is managed in ltm-Config associated to MCG RRC.

Proposal 6. RAN2 discuss which solution to be used to determine that the MAC CE sent from current SN is to be applied to candidate configuration included in MCG RRC ltm-Config;

- Alt 1. LTM CSC MAC CE sent from SN indicate whether the candidate configuration indicated by candidate cell ID field is maintained in MCG RRC ltm-Config or SCG RRC ltm-Config.

- Alt 2. All SCG LTM configurations are maintained in MCG RRC ltm-Config if at least one inter-SN LTM configuration is configured.

Observation 2. If the UE is configured with DC, if it is inter-SN LTM and if the DRB is terminated to the MN, PDCP which constitutes the DRB associated with SCG is not needed to perform re-establishment.

Proposal 7. PDCP recovery should be performed for the DRB which is associated with SCG and is configured with MN security key upon inter-SN LTM.

Proposal 8. UE expects SN ensures that intra-SN SCG configuration is valid until it’s released even if inter-MN LTM is triggered.

Proposal 9. RAN2 doesn’t pursue inter-MN LTM with PSCell addition.

Proposal 10. NW ensures there is no SCG LTM configuration if UE is not configured with SCG.

Proposal 11. The validity of the subsequent LTM should be monitored.

Proposal 12. The approaches for monitoring the validity of the subsequent LTM can consider the limited number of subsequent cell switches and the limited duration allowed for subsequent cell switches.

Proposal 13. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 14. RAN2 further discuss

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- R2-24xxxx, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

- TS 38.300 V18.1.0 (2024-05), 5G; NR; NR and NG-RAN Overall description; Stage-2