3GPP TSG-RAN WG2 Meeting #128	R2-2410719

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:		8.2.4

Source:		Samsung

Title:			Further discussion on Ambient IoT random access

Document for:	Discussion & Decision

- 1 Introduction

The agreements that have been made related to A-IoT random access in the last RAN2 meeting is as follows.

Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

Based on the agreements, in this contribution, we discuss remaining issues on A-IoT random access.

- 2 Discussion

In 3-step CBRA, we need to discuss whether the subsequent R2D transmission is always sent by reader, when the reader receives A-IoT Msg3 successfully. For inventory + command use case or command only use case, the command message can be directly sent to the device. Note that it could be used as an implicit indication of the successful reception of A-IoT Msg3 by the reader. For inventory only use case, we think the corresponding R2D transmission needs to be sent to the device to indicate the successful reception of A-IoT Msg3. Otherwise, the device has no idea whether the reader decided to abort the on-going 3-step CBRA procedure or received the A-IoT Msg3 successfully. Hence we think that the corresponding R2D transmission following A-IoT Msg3 is always needed.

Proposal 1: In 3-step CBRA, device considers A-IoT Msg3 transmission as successful if the subsequent R2D transmission to this device is received. The subsequent R2D transmission is of either R2D command message, or ACK-like R2D message if no R2D command message follows.

Given the agreement that reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response”, the same philosophy can be considered for Msg3 retransmission in 3-step CBRA when the Msg3 suffers failure, in order to reduce the latency of the inventory while imporving the reliability. That is, based on Reader implementation, it is allowed to send Msg2 again to trigger Msg3 retransmission, if needed in some cases, e.g., the previous Msg2 or Msg3 has not been delivered successfully.

Proposal 2: In 3-step CBRA, reader can re-send Msg2 to trigger the device to re-send Msg3.

In CBRA, for device access and re-access purpose, we should discuss which information should be provided by which R2D message(s). Firstly, the R2D trigger message(s) could possibly include the information used for determining 1) whether to participate the RA procedure, 2) the total number of access occasions assigned for Msg1 transmissions, 3) scheduled resource in time, and possibly in frequency, domain(s) for Msg1 transmissions.

In our understanding, information 1) and 2) are essential for device to access/re-access the RA procedure.

The remaining issue is, whether R2D trigger message for indicating the scheduling information is needed. One aspect we can consider is that A-IoT devices are supposed to suffer large sampling frequency offset (SFO), and hence, in many cases, the time synchronization among different devices and reader cannot last for a long time. It means such R2D trigger message is needed for better time synchronization. On the other hand, the total number of access occasions assigned by reader can be set within a certain bound to avoid the synchronization issue. Besides, RAN1 is also discussing similar issues. Hence we think RAN2 can first confirm the need of information used for purpose 1) and 2), and further discuss whether 3) is needed or not considering RAN1 progress.

Proposal 3: RAN2 to confirm that R2D trigger message(s) indicates the information for devices to determine 1) whether to access/re-access the RA procedure, and 2) the total number of access occasions, and FFS for 3) the resource scheduling in time, and possibly in frequency, domain(s) for Msg 1 transmissions.

For the issue of whether 2-step CBRA should be supported or not, and if supported, whether it should be a mandatory or optional feature, we prefer a simple and unified solution, where:

- 3-step and 2-step CBRAs are mandatorily supported by device, and

- whether to proceed with 3-step or 2-step CBRA is controlled by reader.

There could be an option that 2-step CBRA is not introduced for A-IoT. While, we see some value delivered by 2-step CBRA in reducing the inventory latency when the inventory should be finished within a certain time limit.

There could be another option that 2-step CBRA is introduced as an optional feature, while it causes some restrictions on the scenarios for applying the 2-step CBRA, e.g., reader can only trigger it with prior knowledge on device capability/types (because no AS-signalling for device capability reporting in A-IoT), within a controllable area.

Proposal 4: A unified CBRA solution is introduced for A-IoT, where

- 3-step and 2-step CBRAs are mandatorily supported by device, and

- whether to proceed with 3-step or 2-step CBRA for the device is controlled by reader.

- 3	Conclusion

Based on above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: In 3-step CBRA, device considers A-IoT Msg3 transmission as successful if the subsequent R2D transmission to this device is received. The subsequent R2D transmission is of either R2D command message, or ACK-like R2D message if no R2D command message follows.

Proposal 2: In 3-step CBRA, reader can re-send Msg2 to trigger the device to re-send Msg3.

Proposal 3: RAN2 to confirm that R2D trigger message(s) indicates the information for devices to determine 1) whether to access/re-access the RA procedure, and 2) the total number of access occasions, and FFS for 3) the resource scheduling in time, and possibly in frequency, domain(s) for Msg 1 transmissions.

Proposal 4: A unified CBRA solution is introduced for A-IoT, where

- 3-step and 2-step CBRAs are mandatorily supported by device, and

- whether to proceed with 3-step or 2-step CBRA for the device is controlled by reader.