3GPP TSG-RAN WG2 Meeting #128 	   R2-2410728

# Orlando, USA, Nov 18th – 22th, 2024

Source:	TCL

Title:	Discussion on DSR enhancement

Agenda item:	8.7.4.2

Document for:	Discussion and decision

- Introduction

In RAN2 #127bis meeting, DSR enhancement has been discussed extensively and some agreements has been reached as follows:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution we will further discuss the open issues related to DSR enhancement.

- Discussion

  - The thresholds configuration for DSR triggering and reporting

In the discussion on thresholds configuration for DSR triggering and reporting, Some companies believe that if the triggering threshold is smaller than the reporting threshold(s), after a DSR is triggered, reporting data with a remaining time greater than the triggering threshold may trigger a new DSR and duplicated report partial DSR information. However as illustrated in the following Figure 1, Regardless of whether the triggering threshold is greater than or equal to the reporting threshold(s), the problem of partial duplicated report still exists. One potential solution is to restrict the duplicated report in DSR. Moreover, duplicated report could be avoided completely by restricting the data reported in a DSR to trigger another DSR.

Therefore, a triggering threshold lower than the largest reporting threshold in the DSR procedure does not bring any additional impact. On the contrary, reporting more information in the DSR will reduce the number of DSR reports, decrease signaling overhead, and improve system efficiency. So we propose that the triggering threshold could be configured to be lower than the reporting threshold(s), with the constraint that the data reported in a DSR does not trigger another DSR.

Figure 1

Proposal 1: It is suggested that the triggering threshold could be configured to be lower than the reporting threshold(s), with the constraint that the data reported in a DSR does not trigger another DSR.

  - The usage of old and new DSR

Based on the agreement regarding the enhanced DSR format from the last meeting, and with reference to the legacy DSR format, one possible format is illustrated in Figure 2: The only change is that the "R" bit preceding each pair of remaining time and buffer size in the legacy DSR format has been changed to an "E" bit. This "E" bit is used to indicate whether there is another one following a pair of remaining time and buffer size for an LCG.

Figure 2

Basically, in the case that there is at least one LCG configured with multiple reporting thresholds and all LCGs only include one pair of remaining time and buffer size, both legacy and enhanced DSR is applicable. However, the co-existence could cause unnecessary confusion and no benefit compared to only using the enhance DSR. So we propose to always use a new enhanced DSR in case there is at least one LCG configured with multiple reporting thresholds.

Proposal 2: It is suggested to use a new enhanced DSR in case there is at least one LCG configured with multiple reporting thresholds.

- Conclusion

In this contribution, we discussed the DSR enhancement for XR service and provided our proposals as follows:

Proposal 1: It is suggested that the triggering threshold could be configured to be lower than the reporting threshold(s), with the constraint that the data reported in a DSR does not trigger another DSR.

Proposal 2: It is suggested to use a new enhanced DSR in case there is at least one LCG configured with multiple reporting thresholds.