# 3GPP TSG-WG2 Meeting #128                                            			R2-2410730

Orlando, USA, 18th – 22nd November 2024

Source:                      Qualcomm Incorporated

Title:                          LP-WUS in IDLE and Inactive state

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# Discussion

## 2.1 Inter-frequency bands support

RAN WG1 sent RAN WG2/RAN4 an LS in R2-2409157/R1-2407559) and asked RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

The intention of supporting inter-frequency bands for LR and MR is UE could only support LP-WUS on specific band, and MR is operating on another frequency band, or reduce LR cell overloading. In this case, it is assumed that RRM measurement is offloaded to LR, and then no RRM measurement on MR.

If the UE only has measurement on LR cell but no on MR cell, there could be issue that UE moves and out of sync with MR cell. So when the UE is triggered by LP-WUS, UE has to re-sync and reselect the cell on MR frequency. This needs more UE wake-up time and consumes more UE power consumption. Additionally, it will introduce more access latency since UE needs to re-sync and reselect MR cell before respond paging.

Even in collocated of LR and MR frequency, it cannot guarantee that LR and MR coverage is equivalent due to different frequency propagation characteristics, especially LR cell is in lower band, and MR cell is in higher band, in which case the LR cell coverage could be much larger than MR cell coverage. Then when the UE moves, the UE may also out-of-sync with the MR cell.

Observation 1: UE power consumption and paging response latency could be introduced due to UE needs to re-synchronize and reselect a cell on MR frequency after receiving the paging message.

Another issue is that UE needs to camp on LR cell and MR cell simultaneously so that UE needs to maintain two sets of system information and it is unclear whether UE should perform two location update procedure respectively in both LR and MR when the UE moves outside of location area e.g. TA  or RAN paging area.

Observation 2: UE needs to camp on both LR cell and MR cell and maintain system information for both LR cell and MR cell, which bring complexity to UE implementation.

Observation 3: It is unclear whether UE should perform two location update procedure respectively in both LR and MR when the UE moves outside of location area e.g. TA  or RAN paging area.

Another issue is the deployment restriction the cells o LR frequency and cells on MR frequency has to be in the same paging area, otherwise, the gNB CN may not handle the paging response since currently CN and gNB does not expect receiving paging response outside of the paging area.

Observation 4: There should be deployment registration that LR cell and MR cell have to be in the same paging area.

whether the current gNB or CN can support the case that the gNB receiving UE access for paging response

Based on the above discussion, considering the limited time in Rel-19, it is proposed not to support inter-frequency bands for LR and MR.

- RAN2 does not support the case that LR and MR is operating different frequency bands.

For the frequency bands that UE cannot supports LP-WUS, it is assumed LP-WUS will not be enabled for the UE.

- For the frequency bands that UE cannot supports LP-WUS, RAN2 assumes LP-WUS will not be enabled for the UE.

## 2.1 LP-WUS mode control

There were some discussions in RAN2#127 meeting that whether dedicated LP-WUS configuration can be provided to the UE. It is understood that the LP-WUS configuration here means entry/exit condition, it is unclear what is the motivation for dedicated entry/exit condition to be introduced. And if the dedicated condition is introduced, the UE needs to maintain both the dedicated configuration and common configuration and RAN2 also needs to discuss in what condition the dedicated configuration is invalid.

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

In RAN2#127 meeting, there were proposals and discussion that network can activate or deactivate LP-WUS monitoring using dedicated RRC signalling. If the UE is deactivated LP-WUS monitoring, then the UE will never use LP-WUS to save power consumption as long as the UE stays in IDLE state in this cell. And the motivation is not clear. Then it is proposed not to introduce LP-WUS activation/deactivation in dedicated RRC message.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

Some companies proposed to indicate in SIB whether the UE should skip PEI monitoring when the UE is monitoring LP-WUS. The motivation is to replace PEI with LP-WUS totally, and the network does not need to send indication signalling in both LP-WUS and PEI. But the issue is, when the UE moves outside of LP-WUS coverage, the UE will not indicate to the network as the baseline RAN2 made in the last meeting and the network cannot enable PEI, so the UE cannot use PEI. Another issue is, LP-WUS and PEI can be used together to reduce more false paging. Also, RAN1 has discussed this issue, and the agreement is that it is left to UE implementation to monitor PEI or not.

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Then, it is proposed network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB, and do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

In the RAN2#127 meeting, there were discussion on whether entry/exit condition of LP-WUS monitoring should be mandatory or can be optionally configured.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Since gNB does not know whether the UE enters or leaves LP-WUS mode, then gNB needs to send paging indication in both of LP-WUS and PEI or legacy paging. Whatever the UE is in LP-WUS paging mode or legacy paging mode (i.e. whether the entry or exit condition is satisfied), there is no different gNB behaviors. Then it is not necessary to provide the condition to the UE. Besides, there could be the case that LP-WUS coverage is equivalent with MR coverage, then the condition is not needed. UE can determine whether to enter or exit LP-WUS based on the MR/LR measurement.

- The configuration of the entry/exit condition of LP-WUS monitoring can be optionally configured to the UE. If absent, it is left to UE implementation to determine whether to enter or exit LP-WUS mode.

In RAN2#127 meeting, it has been agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

One issue is, if both of the OFDM-based and OOK-based WUS are configured for the entry/exit threshold, which one of should be used by the UE. It assumes that LP-WUS UE may have different capabilities on OFDM based WUS measurements. In this case, it should allow the UE to select the measurement metrics the UE can support.

- It is up to UE implementation to choose which one is used if both of OFDM-based and OOK-based WUS entry/exit thresholds are configured.

## 2.2 Whether to support dynamic PO

In RAN1 there is some discussion on dynamic PO and has following agreement. Dynamic PO has also impact on higher layer.

Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO

Dynamic paging occasion means UE monitors paging after deterring LP-WUS indicating its subgroup ID and UE does not need to monitor paging on legacy paging occasion.

Currently, network and UE calculate paging occasions and transmit/receive paging message on paging occasion. When DL data arrives to CN, CN will calculate paging occasion, and send paging request to gNB before the paging occasion to reduce gNB burden of saving paging for a long time.

If dynamic PO is introduced, CN will send paging request to gNB when DL data arrives, which will change existing CN behavior.

Observation 5: Currently, CN will calculate paging occasion, and send paging request to gNB before the paging occasion. If dynamic PO is introduced, CN will send paging request to gNB when DL data arrives, which will change existing CN behaviour.

UE in IDLE and Inactive state will not indicate to gNB when the UE enters or leaves LP-WUS mode, then when gNB does not know whether legacy paging on LP-WUS paging will be used. gNB has to send two paging messages on the dynamic PO and legacy PO, then paging message load will be doubled.

Observation 6: gNB needs to send two paging message on both of dynamic PO and legacy PO, which will waste paging resource.

Based on the above observations, it is proposed dynamic PO is not introduced.

- Dynamic PO mechanism is not introduced, and UE should monitor paging in legacy PO mechanism if triggered by LP-WUS.

## 2.3 LP-WUS subgrouping

In last RAN2 meeting, RAN2 agreed that for UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

In last RAN1 meeting, RAN1 had working assumption that the maximum number of subgroups per PO supported in Rel-19 is 32. From RAN2 perspective, RAN can confirm 32 can be supported.

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.

- RAN2 confirms RAN1 working assumption that the maximum number of subgroups per PO supported in Rel-19 is 32.

After UE detects LP-WUS indication, RAN1 agreed that it is left to UE implementation to monitor PEI or not. If the UE is indicated to trigger MR for paging monitoring, UE may continue to monitor PEI and determine whether to further monitor paging message. one additional issue is whether any enhancement is needed to further distributed the UEs mapped to the same LP-WUS subgroup into different PEI subgroups. The joint subgrouping between LP-WUS and PEI improves the subgrouping granularity by a factor of the number of PEI subgroups with no additional signaling cost. Then the subgroup ID in LP-WUS can be independent with the one in PEI. Since the maximum number is 32, then RAN2 can discuss whether enhancement is needed to further distribute the UEs mapped in the same LP-WUS subgroup into different subgroups in PEI.

- For UE-based subgrouping, the subgroup ID in LP-WUS can be independent with the one in PEI.

- RAN2 discusses whether enhancement is needed to further distribute the UEs mapped in the same LP-WUS subgroup into different subgroups in PEI.

## 2.5 UE LP-WUS capability reporting

For CN-assigned subgroup ID, UE should report capability to CN, and then CN can assign CN-assigned subgroup ID to UE.

For UE-based subgroup ID, it is also beneficial to indicate CN UE capability. In existing paging framework, CN can also calculate POs and sends Paging Request to gNB just earlier than the PO. The motivation is to avoid gNB storing paging for a long time. If the UE does not indicate UE-based subgrouping capability to CN, then CN will send Paging Request to gNB as legacy, i.e. just earlier than legacy PO. However, the time of sending Paging Request could be late that LP-WUS occasion, and gNB has to send LP-WUS indication in the next cycle. This will delay paging one paging DRX cycle. This can be shown in the following figure.

RAN1 is discussing that one or more offsets need to be configured to the UE, there is no agreements so far. For this capability, it can be FFS.

- Observation 7: Without UE-based subgrouping capability in CN, the paging could be delayed one paging cycle.

- UE should indicate supporting CN-assigned and/or UE-based subgrouping with LP-WUS in the REGISTRATION REQUEST message. Whether to indicate the capability of offset value(s) waits for RAN1 further progress.

# Conclusion

- Inter-frequency bands support,

Observation 1: UE power consumption and paging response latency could be introduced due to UE needs to re-synchronize and reselect a cell on MR frequency after receiving the paging message.

Observation 2: UE needs to camp on both LR cell and MR cell and maintain system information for both LR cell and MR cell, which bring complexity to UE implementation.

Observation 3: It is unclear whether UE should perform two location update procedure respectively in both LR and MR when the UE moves outside of location area e.g. TA  or RAN paging area.

Observation 4: There should be deployment registration that LR cell and MR cell have to be in the same paging area.

- RAN2 does not support the case that LR and MR is operating different frequency bands.

- For the frequency bands that UE cannot supports LP-WUS, RAN2 assumes LP-WUS will not be enabled for the UE.

- LP-WUS mode control

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- The configuration of the entry/exit condition of LP-WUS monitoring can be optionally configured to the UE. If absent, it is left to UE implementation to determine whether to enter or exit LP-WUS mode.

- It is up to UE implementation to choose which one is used if both of OFDM-based and OOK-based WUS entry/exit thresholds are configured.

- Dynamic PO mechanism

Observation 5: Currently, CN will calculate paging occasion, and send paging request to gNB before the paging occasion. If dynamic PO is introduced, CN will send paging request to gNB when DL data arrives, which will change existing CN behaviour.

Observation 6: gNB needs to send two paging message on both of dynamic PO and legacy PO, which will waste paging resource.

- Dynamic PO mechanism is not introduced, and UE should monitor paging in  legacy PO mechanism if triggered by LP-WUS.

- LP-WUS subgrouping

- RAN2 confirms RAN1 working assumption that the maximum number of subgroups per PO supported in Rel-19 is 32.

- For UE-based subgrouping, the subgroup ID in LP-WUS can be independent with the one in PEI.

- RAN2 discusses whether enhancement is needed to further distribute the UEs mapped in the same LP-WUS subgroup into different subgroups in PEI.

UE LP-WUS capability reporting

- Observation 7: Without UE-based subgrouping capability in CN, the paging could be delayed one paging cycle.

- UE should indicate supporting CN-assigned and/or UE-based subgrouping with LP-WUS in the REGISTRATION REQUEST message. Whether to indicate the capability of offset value(s) waits for RAN1 further progress.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)