3GPP TSG-RAN WG2 Meeting #128	R2-2410731

Orlando, USA, 18th – 22nd November 2024

Source:                Qualcomm Incorporated

Title:	LP-WUS operation in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

• Initiate the discussion on the configuration, activation/deactivation.

• Initiate the discussion on the procedure of using LP-WUS with/without C-DRX.

# Discussion

## 2.1 RAN2 impact on LP-WUS options

In RAN1, there are several options about how UE performs LP-WUS in CONNECTED state. Since these options impacts RAN2 specification, it is worthy to evaluate these options from RAN2 perspective.

### 2.1.1    RAN2 aspects for Option 1-1

In the RAN2#127 meeting, it is agreed to take the existing DCP mechanism as baseline. Based on this agreement.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

In RAN1#118bis meeting, RAN1 agreed that the following operations are assumed.

----------------------------------------------------RAN1 agreements-------------------------------------------------------

Agreement

For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

- LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

- UE is configured with legacy C-DRX configurations as Rel-18

- UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

- No impact on RRM/RLM/BFD measurement requirements is assumed

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

  - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

    - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

-------------------------------------------------------------------------------------------------------------------------------------

Then the following configuration should be provided to the UE as Option 1-1 LP-WUS

- For Option 1-1, the following configurations can be provided to the UE.

- Legacy C-DRX configuration.

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- The following indications similar with the DCP indications can also be configured to the UE.

o	LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case LP-WUS is monitored but not detected;

o	LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case LP-WUS is configured but associated drx-onDurationTimer is not started;

o	LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case LP-WUS is configured but associated drx-onDurationTimer is not started;

### 2.1.2    RAN2 aspects for Option 1-2

For Option 1-2, RAN2 agreed to introduce a new timer for PDCCH monitoring triggered by LP-WUS. Taking this into account, following discusses how Option 1-2 is configured. Option 1-2 could have two approaches.

Alternative 1: Configure LP-WUS occasion periodicity/offset and offset to start drx-onDurationTimer-LPWUS

In this alternative, LP-WUS occasions periodicity and offset are configured to the UE. Additionally, MR-offset is configured to determine the start of the new drx-onDurationTimer-LPWUS, as shown in Figure 1. UE calculates the start of drx-onDurationTimer-LPWUS, and starts drx-onDurationTimer-LPWUS after MR-offset.

Figure 1: Alternative 1

Alternative 2: Configure DRX periodicy/offset and offset of LPWUS monitoring prior to the start of drx-onDurationTimer

This alternative is similar to Option 1-1, LP-WUS periodicity/offset are not configured by RRC. UE is configured C-DRX periodicity/offset, on top of that, UE is additionally configured LP-WUS offset, as shown in Figure 2. UE calculates the start of LP-WUS occasions, and monitors LP-WUS occasions prior to the start of drx-onDurationTimer-LPWUS.

Figure 2: Alternative 2

- For Option 1-2, consider the following alternatives for LP-WUS configuration

Alt 1: Configure LP-WUS occasion periodicity/offset and offset to start drx-onDurationTimer-LPWUS

- LPWUSCycleStartOffset: LP-WUS occasion cycle and drx-StartOffset which defines the subframe where LP-WUS DRX cycle starts;

- MR-offset: The start of drx-onDurationTimer-LPWUS relative to the start of LPWUS monitoring.

Alt 2: Configure DRX cycle and offset of LPWUS monitoring prior to the start of drx-onDurationTimer

- DRX cycle and offset: DRX cycle and offset which defines the subframe where drx-onDurationTimer-LPWUS start if triggered by LP-WUS

- LPWUS-offset: The start of LPWUS monitoring prior to the start of drx-onDurationTimer-LPWUS

## 2.3 Switching between LP-WUS and legacy PDCCH monitoring

As discussed in IDLE/Inactive state, it is possible the LP-WUS coverage is smaller than MR coverage. In this case, UE should be able to fall back to PDCCH monitoring on MR using the configured C-DRX configuration.

Considering different LP-WUS options and different legacy PDCCH there are two fallback cases to be considered.

Case 1: UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured

In this case, the UE is using LP-WUS option 1-1, when the UE detects moving outside of LP-WUS coverage, UE can automatically switch to legacy C-DRX PDCCH monitoring. There is no essential need for the network to know the exact moment that the UE autonomously fallback to PDCCH monitoring using C-DRX configuration. In option 1-1, the network anyway transmits both LP-WUS and PDCCH in the corresponding C-DRX active time. The UE monitors either LP-WUS or PDCCH in the corresponding C-DRX active time, depending on fallback status. Therefore, the network does not need to know the particular moment that the UE switches back to legacy C-DRX operation. However, it would be beneficial for network to know whether the UE is able to detect the LP-WUS or not, then gNB can determine whether to send LP-WUS any more.

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Case 2: UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured

For Option 1-2, if UE autonomous fallback to legacy C-DRX is enabled for Option 1-2, network does not know whether/when the UE leaves LP-WUS mode. There can be misalignment between the UE and the gNB when the UE monitors legacy PDCCH and gNB will continue to send LP-WUS indication and PDCCH in the new active time window, and then PDCCH could be missed. So in this case, UE should report to the gNB before switching to legacy PDCCH monitoring. And gNB should send command to the UE to switch to legacy PDCCH monitoring.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Case 3: UE is using legacy PDCCH monitoring with/without DCP configured and enters LP-WUS mode

In this case, the UE is monitoring PDCCH as legacy, and UE detects entering LP-WUS coverage. Since for LP-WUS, gNB should firstly indicate to UE in LP-WUS and then send PDCCH on active time window. Then UE should report to gNB. And gNB should send switching command to the UE to enable LP-WUS. Then there is no mis-alignment between the gNB and the UE, and there is no potential PDCCH monitoring missed.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

Considering the above cases, UE should indicate to gNB when enters or leave LP-WUS mode. The condition for entering or leaving LP-WUS mode could be based on MR measurement, it can be FFS for other conditions, and it can be FFS on whether the entry/exist condition need to be configured to the UE or can be left UE implementation.

- For both of Option 1-1 and Option 1-2,  UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-1, the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

## 2.4 LP-WUS activation/deactivation

RAN1 decided to study activation/deactivation of LP-WUS monitoring as highlighted below in the RAN1#116 agreement.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

It could be possible that a network configures one or multiple LP-WUS configurations, and one of the LP-WUS configuration is activated by the MAC-CE. The MAC-CE can also deactivate LP-WUS monitoring (i.e., it activates none of the LP-WUS configurations), in which case the UE should switch to legacy C-DRX configuration.

In each of the LP-WUS configurations, various RRC parameters can be provided, then the MAC-CE activation can control the overhead/efficiency, associated beam, etc, in dynamic manner.

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

## 2.3 LP-WUS in CA

For CA, from Rel-16, dual DRX groups has been supported. A UE can be configured with different values of drx-onDurationTimer and drx-InactivityTimer for one DRX group and for the other DRX group. Long DRX cycle is common for both DRX groups.

Dual DRX groups

Rel-16 DCP was enabled only on PCell or PSCell and indicates to start drx-onDurationTimer of all the DRX groups. In other words, it was not possible to trigger PDCCH monitoring of different DRX groups independently. This is not beneficial for UE power saving. For Rel-19 LP-WUS in connected mode, per DRX group PDCCH monitoring triggering should be considered. There are two options to enable this:

- Opt.1: LP-WUS is configured to be monitored per DRX group

- Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

Option 1 is enabled if LP-WUS is monitored in two different serving cells in different DRX groups. The LP-WUS in each serving cell is nothing to do with PDCCH monitoring for the cell(s) in the other DRX group. Option 2 is enabled without LP-WUS monitoring on multiple serving cells. However, it has to reserve a couple of bits in the LP-WUS to indicate DRX group. Further study is necessary on which option to support.

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

    - Opt.1: LP-WUS is configured to be monitored per DRX group

      - For dual DRX groups, LP-WUS monitoring is configured per DRX group

      - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

    - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

# Conclusion

RAN2 impact on LP-WUS options,

- For Option 1-1, the following configurations can be provided to the UE.

- Legacy C-DRX configuration.

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- The following indications similar with the DCP indications can also be configured to the UE.

o	LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case LP-WUS is monitored but not detected;

o	LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case LP-WUS is configured but associated drx-onDurationTimer is not started;

o	LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case LP-WUS is configured but associated drx-onDurationTimer is not started;

- For Option 1-2, consider the following alternatives for LP-WUS configuration

Alt 1: Configure LP-WUS occasion periodicity/offset and offset to start drx-onDurationTimer-LPWUS

- LPWUSCycleStartOffset: LP-WUS occasion cycle and drx-StartOffset which defines the subframe where LP-WUS DRX cycle starts;

- MR-offset: The start of drx-onDurationTimer-LPWUS relative to the start of LPWUS monitoring.

Alt 2: Configure DRX cycle and offset of LPWUS monitoring prior to the start of drx-onDurationTimer

- DRX cycle and offset: DRX cycle and offset which defines the subframe where drx-onDurationTimer-LPWUS start if triggered by LP-WUS

- LPWUS-offset: The start of LPWUS monitoring prior to the start of drx-onDurationTimer-LPWUS

Switching between LP-WUS and legacy PDCCH monitoring

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

- For both of Option 1-1 and Option 1-2,  UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-1, the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

LP-WUS activation/deactivation

- gNB can configure multiple LP-WUS configurations to the UE, and activate one or more of the configurations to the UE via MAC CE.

LP-WUS in CA

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells.

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

    - Opt.1: LP-WUS is configured to be monitored per DRX group

      - For dual DRX groups, LP-WUS monitoring is configured per DRX group

      - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

    - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)