3GPP TSG-RAN WG2 Meeting #128	R2-2410741

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	    8.5.4

Source:	III

Title:	Adaptation of paging signal/channel transmissions

Document for:	Discussion

Introduction

In RAN2#127bis meeting, the agreements of adaption of common signal/channel for paging are as follows:

Agreements on paging adaptation 1. Select option-b as baseline for R19 NES paging enhancement. 2. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) 3. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. 4. Legacy UE is not barred. 5. Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In this contribution, adaptation of the Paging channel was discussed.

Discussion

Adaptation of paging occasions in time domain

Extend the values of N to have increased interval between PFs

The number of PFs in each paging cycle should be decreased as small as possible in NES cell for NES-capable UE. NES cell could have more sleep time for energy saving.

In TR 38.331, PF was determined by nAndPagingFrameOffset in PCCH-Config. The minimum number of PFs for legacy UE is shown as the last row in table 1.  Nmin = 2, 4, 8, 16 for T = 32rf, 64rf, 128rf, 256rf separately.

Table 1: number of PFs determined by nAndPagingFrameOffset for legacy UE

| T(cycle) N(PF) | 32rf | 64rf | 128rf | 256rf |
| --- | --- | --- | --- | --- |
| oneT | 32 | 64 | 128 | 256 |
| halfT | 16 | 32 | 64 | 128 |
| quarterT | 8 | 16 | 32 | 64 |
| oneEighthT | 4 | 8 | 16 | 32 |
| oneSixteenthT | 2 | 4 | 8 | 16 |

Extend the values of N to have increased interval between PFs for Rel-19 NES UE in table 2.

Table 2: number of PFs determined by nAndPagingFrameOffset for Rel-19 NES UE

| T(cycle) N(PF) | 32rf | 64rf | 128rf | 256rf |
| --- | --- | --- | --- | --- |
| oneT | 32 | 64 | 128 | 256 |
| halfT | 16 | 32 | 64 | 128 |
| quarterT | 8 | 16 | 32 | 64 |
| oneEighthT | 4 | 8 | 16 | 32 |
| oneSixteenthT | 2 | 4 | 8 | 16 |
| one32thT | 1 | 2 | 4 | 8 |
| one64thT |  | 1 | 2 | 4 |
| one128thT |  |  | 1 | 2 |
| one256thT |  |  |  | 1 |

Proposal 1: RAN2 should further discuss the cases of one32thT, one64thT, one128thT, one256thT 
   		     for option-b.

When only one PF was configured in the paging cycle for Rel-19 NES UEs, all UE_IDs of NES UE will be allocated to corresponding POs (Ns=8 and FFS for other larger values). This is easily to add new value of ns in TS 38.331 DownlinkConfigCommonSIB for Rel-19 NES UE.

For example, a new value “eight” added in ns.

PCCH-Config ::= SEQUENCE { defaultPagingCycle PagingCycle, nAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15) }, ns ENUMERATED { ,eight ,four, two, one},

Paging Early Indication occasion (PEI-O) for option-b

Paging Early Indication occasion (PEI-O) consists of a set of monitoring occasions (MO), which can include multiple time slots. During these occasions, UE can be to detect a DCI format 2_7. The timing of the PEI-O is determined by a reference point based on the start of the PF (provided by pei-FrameOffset ) and symbol-level offest of the reference point to the start of PEI-O (provided by firstPDCCH-MonitoringOccasionOfPEI-O), as following:

Figure 1: Legacy PEI-O

In TS 38.331, the PEI configuration only allows up to 4 POs in one PF to associate with one PEI-O. For option-b (increased interval between PFs ), if Ns is increased to 8 POs (FFS: 16, 32) in one PF. One legacy PEI-O isn’t enough for option-b, more PEI-Os are needed for more POs in same PF. Example (Two PEI-Os for 8 POs) is as Figure 2.

Figure 2: more PEI-Os in same PF

Observations 1: Multiple symbol-level offsets for different PEIs can indicate different POs in same PF

Another solution is to extend the associated POs in one PF from 4 to 8 or more in PEI-configuration, as Figure 3.

Figure 3: one PEI-O with exended associated POs in same PF

Observations 2: Extend the associated POs in one PF from 4 to 8 or more in PEI-configuration

Compatibility of legacy RRC_IDLE/RRC_INACTIVE UE Legacy and R19 UEs co-existence

In RAN2#125bis agreement, as follows

3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In RAN2#127bis agreement, Legacy UE is not barred, so option 2 will be discussed here.

Legacy UE design should not affect BS energy saving, especially when there is excessive PFs in paging cycle. The implementation of this agreement ensures that paging resources are balanced for legacy UEs and Rel-19 NES UEs. The following is considered for paging resource of NES cell:

Separate PFs for legacy UEs and Rel-19 NES UEs

One solution for separating paging resources is to align the PFs of Rel-19 NES UEs with PFs of legacy UEs and minimize the number of PFs of NES UEs as Figure 4. (Assume paging cycle is 32rf, number of PF is 2 for legacy UE and number of PF is 1 for Rel-19 NES UE).

Figure 4: PF of Rel-19 NES UEs in paging cycle

Separate PMO for legacy UEs and Rel-19 NES UEs in overlapping PF

The other solution is to overlap the PFs of Rel-19 NES UEs with PFs of legacy UEs as Figure 5.

Figure 5: overlap the PFs of Rel-19 NES UEs with PFs of legacy UEs

The PDCCH monitoring occasions (PMO) of Legacy UE and Rel-19 NES UE can be separated in time domain, as following:

Figure 6: PDCCH monitoring occasions (PMO) of Legacy UE and Rel-19 NES UE

Proposal 2: Separated paging resources can be achieved in time domain by:

Solution 1 - Separate PFs for legacy UEs and Rel-19 NES UEs

Solution 2 - Separate PMO for legacy UEs and Rel-19 NES UEs in overlapping PF

Consideration for adaption of paging frame during Cell DTX/DRX non-active period

In RAN2#121bis-e agreement [1], the UE does not transmit SPS/SR/CG during Cell DTX/DRX non-active period.

There is no agreement about common channel behavior during Cell DTX/DRX non-active period, e.g., paging channel for idle mode UE. The compact PF is one feasible method for REL-19 NES UE receiving paging message during cell DTX on-duration.  Further clarifications are needed for this issue:

- In R18 Cell DTX/DRX discussions, only connected mode UEs were addressed, How to extend the cell DTX/DRX mechanism for idle/inactive mode UE.

- How to align the compact/bundle PF with cell DTX on-duration for idle/inactive mode UE.

Proposal 3: Compact/bundle PF should be further studied at least in the scenario of cell DTX/DRX

mechanism for idle/inactive mode UE.

Conclusions

In this contribution, the following observations and proposals are made:

Proposal 1: RAN2 should further discuss the cases of one32thT, one64thT, one128thT, one256thT 
   		     for option-b.

Proposal 2: Separated paging resources can be achieved in time domain by:

Solution 1 - Separate PFs for legacy UEs and Rel-19 NES UEs

Solution 2 - Separate PMO for legacy UEs and Rel-19 NES UEs in overlapping PF

Proposal 3: Compact/bundle PF should be further studied at least in the scenario of cell DTX/DRX

mechanism for idle/inactive mode UE.

Observations 1: Multiple symbol-level offsets for different PEIs can indicate different POs in same PF

Observations 2: Extend the associated POs in one PF from 4 to 8 or more in PEI-configuration

- References

- R2-2304203,” Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”, InterDigital , April 2023.