3GPP TSG-RAN WG2 Meeting #128                                    R2-2410755

Orlando, USA, Nov.18~22, 2024

Agenda item:		8.2.5

Source:		Samsung

Title:			Discussion on A-IoT Topology 2

Document for:	Discussion & Decision

- 1 Introduction

In last RAN2 meeting, the topology 2 were discussed for the first time, and reached the following agreements:

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this contribution, we will discuss the main issues for such topology.

- 2 Discussion

- Resource validity

In our understanding, the dedicated resource configuration already indicates the time/frequency-domain resource allocated to the intermediate node. This information indicates the validity period of the allocated resource. Thus, if the intermediate node is still in connection, no additional design is needed to determine the validity time of the resource. For cases with temporary out of connections (e.g., HO, RLF), some companies propose to define a timer to check the resource validity. However, since the dedicated resource is already allocated, the gNB will not allocate those resource to other UEs. Thus, the intermeidate node can use those resource even in temporary out of connections. In other words, the resource validity determination can be determined by the dedicated resource configuration.

Proposal 1: RAN2 is kindly asked to discuss whether the resource validity can be determined by the dedicated resource configuration.

In case of HO or re-establishment after the RLF, the dedicated resource can be reconfigured by the new cell. To avoid the interference to the new cell, whenever the intermediate node accesses to the new cell, the dedicated resource allocated in the previous cell can be released, and the new resource configured by the new cell can be used.

Proposal 2: RAN2 is kindly asked to agree that the dedicated resource allocated in the source cell can be released once the intermediate node accesses to a new cell after, e.g., HO or reestablishment after RLF.

Meanwhile, in last meeting, companies mentioned the resource validity across cell. This feature is not needed since this requires the coordination among different gNBs, which further complexes the system design.

Proposal 3: RAN2 is kindly asked to exclude the resource validity across mulitple cells.

- Assistant information design for resource allocation

In Topology 2, gNB needs to determine the allocated resource. Thus, some assistant information is required for the gNB either from CN (for RRC based architecture) or from intermediate node (for NAS/UP based architecture). In our view, the resource allocation can be determined by the following aspects:

- Performance requirement of AIoT procedure: each AIoT procedure should have some performance requirement. For example, the inventory procedure needs to be completed within certain period (e.g., completion time); the number of inventoried devices should be larger than a certain number (e.g., percentage of inventoried devices). Those requirements come from the CN. Moreover, those requirements are also applicable to topology 1. Thus, gNB should be aware of the performance requirement of AIoT procedure.

- Device availability: due to energy harvesting, each AIoT device may not be always available. Thus, the intermediate node can only communicate with the available devices. In other words, the resource allocated for AIoT air interface can be aligned with the available time of the devices. Thus, the intermediate node can provide some assistant information to gNB considering the device availability.

- Request from the device: as discussed for topology 1, the device may provide some assistant information to the intermediate node, e.g., energy state report, message “size” indication. Those information can be also used for the resource allocation. Specifically, the intermediate node collect the information from different devices, and then determine the assistant information for the resource allocation which are sent to the gNB.

In topology 1, the above three aspects can be derived by gNB either from CN or from device(s) directly. However, in topology 2, gNB cannot obtain “the device availability” and “request from device”, which are located at intermediate node side; while gNB may not know “performance requirement of AIoT procedure” for NAS/UP-based architecture. Thus, to facilitate the resource allocation, the intermediate node needs to provide some assistant information to the gNB.

Proposal 4: the intermediate node is required to provide some assistant information to the gNB for resource allocation. Details of assistant information can be left to WI stage.

- Dedicated resource configuration

RAN2 already agrees the dedicated resource configuration. However, the unclear part is whether this resource configuration is dynamic or semi-static. Dynamic resource means the gNB can use dynamical signaling to carry resource allocation configuration, e.g., DCI, MAC CE; while the semi-static resource means that the gNB can use RRC signaling for the resource allocation.

Proposal 5: RAN2 is kindly asked to discuss whether the dedicated resource configuration indicates the dynamic resource or semi-static resource.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to discuss whether the resource validity can be determined by the dedicated resource configuration.

Proposal 2: RAN2 is kindly asked to agree that the dedicated resource allocated in the source cell can be released once the intermediate node accesses to a new cell after, e.g., HO or reestablishment after RLF.

Proposal 3: RAN2 is kindly asked to exclude the resource validity across mulitple cells.

Proposal 4: the intermediate node is required to provide some assistant information to the gNB for resource allocation. Details of assistant information can be left to WI stage.

Proposal 5: RAN2 is kindly asked to discuss whether the dedicated resource configuration indicates the dynamic resource or semi-static resource.