3GPP TSG-RAN WG2 Meeting #128	R2-2410762

Orlando, USA, Nov 18-22, 2024

Source:	III

Title:	Discussion on XR DSR enhancements

Agenda item:	8.7.4.2

Document for:	Discussion

- Introduction

During the RAN2#127bis meeting [1], the following agreements were noted:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Topics relative to XR-specific capacity issues was raised as mentioned above. In this contribution, “The issues of multiple reporting thresholds and non-delay critical data in the DSR” are addressed.

- Discussion

In the RAN2#127bis meeting [1], the following agreements were noted:

Agreements on DSR enhancements

- We do not change the definition of delay-critical data

- For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s)

- Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc.

- For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0:

  - Buffer size of data volume in each portion

  - Shortest remaining time among PDCP SDUs buffered in each portion.

- There is no need to include PSI in the enhanced DSR MAC CE.

- A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

    - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

To support enhancement for scheduling, the following issues need to be further considered:

- The issues of multiple reporting thresholds and non-delay critical data in the DSR.

  - The issues of Multiple reporting thresholds and non-delay critical data in the DSR

In the realm of 5G Extended Reality (XR) applications, such as augmented reality (AR) and virtual reality (VR), ensuring optimal performance is critical. This involves managing network delays meticulously to maintain a seamless user experience. One of the tools used to achieve this is the Delay Status Report (DSR). This mechanism allows the UE to report network delay conditions to the network, enabling adaptive adjustments to meet the stringent requirements of XR applications. DSR is crucial for monitoring real-time performance metrics, particularly latency, which can significantly impact the quality of XR experiences. By providing timely and accurate delay information, DSR helps dynamically adjust network parameters to address latency issues and improve overall service quality.

To manage network delays effectively, DSR can be configured with multiple thresholds. Each threshold corresponds to a specific delay level and triggers different network responses based on the severity of the delay. Non-delay critical data may also be included in the delay severity classification.

In XR applications, managing DSR effectively is essential for optimal performance. When the gNB receives a report of non-delay critical data in a DSR, it relies on the remaining time to determine when this data may transition to delay-critical status, without prioritizing immediate resource allocation for that data. When the gNB identifies non-delay critical data, it does not allocate resources to the UE right away. Instead, it monitors the remaining time to assess if and when the data will change status. Only when the gNB determines that the data has become delay-critical does it begin to consider resource allocation for that UE. The key issues related to DSR containing non-delay critical data include:

- Delay in Resource Allocation: The time taken for the gNB to recognize the transition from non-delay critical to delay-critical can lead to significant delays in resource allocation, potentially impacting performance.

- Lack of Feedback on Data Transmission: Since the UE does not inform the gNB about which specific data has been transmitted, the gNB cannot verify whether the data in the DSR has been sent. This lack of feedback creates ambiguity in the system.

- Inconsistent Data Requirements: The delay in resource allocation and the absence of feedback can result in a misalignment between the resource allocation of the gNB and the actual data transmission status from the UE. This inconsistency can lead to incorrect resource configurations, ultimately degrading service quality.

The process of managing non-delay critical data in DSR highlights significant synchronization challenges between the UE and gNB in XR. The inability of the gNB to ascertain the transmission status of data can lead to resource misallocation and performance degradation. Addressing these synchronization issues is critical for optimizing DSR functionality and improving overall network efficiency.

Observation 1: If the DSR includes non-delay critical data, managing the DSR in XR will lead to resource utilization synchronization challenges between the UE and gNB.
Observation 2: For DSRs that include non-delay critical data in XR, a real-time feedback mechanism between the UE and gNB is required to enhance resource allocation efficiency and synchronization.
Proposal 1: When the DSR includes non-delay critical data, the real-time feedback mechanism should be discussed.

- Conclusion

The related proposals and observations from above discussions are below:

Observation 1: If the DSR includes non-delay critical data, managing the DSR in XR will lead to resource utilization synchronization challenges between the UE and gNB.
Observation 2: For DSRs that include non-delay critical data in XR, a real-time feedback mechanism between the UE and gNB is required to enhance resource allocation efficiency and synchronization.
Proposal 1: When the DSR includes non-delay critical data, the real-time feedback mechanism should be discussed.

References

[1] 	Report of 3GPP TSG RAN WG2 #127bis