3GPP TSG-RAN WG2 Meeting #128	R2-2410770

Orlando, USA, Nov18th–22nd, 2024

Source:        ZTE Corporation

Title:           Consideration on the PL Offset MAC CE for R19 MIMO

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

In the last meeting, the PL offset MAC CE was discussed based on the below RAN1’s LS [1] and 2 RAN2’s papers [2][3].

1. Overall Description: Pertaining to the Rel-19 NR MIMO Ph5 WI, RAN1 would like to inform RAN2 about the following: - An initial list of RRC parameters (excel spreadsheet attached) - An agreement requiring MAC CE support: Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In this contribution, we’d like to share our understanding on this topic.

- Discussion

In this chapter, we first analyse TCI State Type for the PL Offset Update MAC CE, then discuss whether the MAC CE shall be defined jointly or separately from the TCI state activate/deactivate MAC CE, at last we would focus on the MAC CE design detail issues.

2.1 TCI State Type for the PL Offset Update MAC CE

On the MAC CE design, the first issue is for which type of the TCI state the PL offset can be updated. According to the submitted papers in the last meeting, there are 2 options:

- Option 1: Update the PL offset for the configured TCI states

- Option 2: Only update the PL offset for the activated TCI states.

Proposal 1: RAN2 to discuss which TCI state type is preferred for the PL Offset Update MAC CE

- Option 1: Update the PL offset for the configured TCI states;

- Option 2: Only update the PL offset for the activated TCI states.

For the option 1, the related TCI state IDs need to be explicitly included in the PL offset MAC CE, otherwise 128 or 64 bits would be used for the TCI sate ID indication. For the option 2, the related TCI state IDs can be implicitly indicated, e.g. with 16bits, each bit indicate whether PL offset updated for the corresponding activated TCI state, which would reduce the PL offset MAC CE size significantly. Thus the key issue is whether there are any scenarios that the option 2 can’t work, based on the companies’ contribution, the main concern on the option 2 is for the below 2 scenarios:

- Scenario 1: The network may intend to update PL offset for the TCI state that are potentially activated later;

- Scenario 2: When the followUnifiedTCI-StateSRS is not configured, the TCI state configured by the RRC (or indicated by the MAC CE) would be used.

For the Scenario 1, it can be solved by including TCI state Activation/Deactivation and PL offset update in one MAC CE at least. If the TCI state activation delay is enough for the UE to process a following MAC CE, another solution is to activate the TCI state first, and then update the corresponding PL offset for the activated MAC CE.

Observation 1: For the case that the network may intend to update PL offset for the TCI state that are potentially activated later, the network can include TCI state Activation/Deactivation and PL offset update in one MAC CE or send a TCI state Activation/Deactivation MAC CE followed by a PL offset update MAC CE .

For the Scenario 2, as summarized in the Table 1, for the PUCCH/PUSCH/SRS case 1, the same mechanism TCI state activation/Indication is used. For the SRS case 2 (i.e. the followUnifiedTCI-StateSRS is not configured for a SRS resource set), a separate set of TCI states may be used for the SRS and thus the option 2 may not work for this case. Thus, first we need to confirm whether the SRS case 2 shall be considered for this topic.

Table 1：The TCI state activation/Indication can be summarized as below:

| Physical Channel Type | TCI state activation/Indication |
| --- | --- |
| PUCCH/PUSCH | MAC CE can activate up to 8 TCI states (or 8 TCI state pairs), DCI would indicate which one is applied. |
| PDCCH-order PRACH | Same to PUCCH/PUSCH, indicated by the DCI, more details are under discussion. |
| SRS | Case 1: followUnifiedTCI-StateSRS is configured, same to the PUCCH/PUSCH, |
|  | Case 2: followUnifiedTCI-StateSRS is not configured for a SRS resource set, according to the 38.214 6.2.1 (as copied in Note) - For the periodic: follow a statically configured TCI state (e.g. srs-TCI-State-r17) for each SRS resource - For AP/SP SRS: can be activated by the MAC CE “SP/AP SRS TCI State Indication MAC CE” or “Serving Cell Set based SRS TCI State Indication MAC CE” |
| Note: 38.214 6.2.1 When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE can assume that SRS resource(s) in any SRS resource set, except SRS resource set for positioning and an SRS resource set configured with followUnifiedTCIStateSRS, can be configured with TCI-State or TCI-UL-State or updated as described in clause 6.1.3.59 or 6.1.3.60 of [10, TS 38.321]. The reference RS in the TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer. |  |

Proposal 2: RAN2 to confirm whether the SRS case 2 (i.e. followUnifiedTCI-StateSRS is not configured for a SRS resource set) should be considered for the PL offset MAC CE design.

According to the RAN1’s conclusion on the framework below, it has a restriction that only 1 or 2 UL TCI state can be applied for the R17/18 respectively, it seems that only SRS case 1 is supported.

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM (inter-cell Beam management) and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

However, if the SRS case 2 should also be considered, we can further confirm whether there are cases the network would configure/indicate the TCI states for the SRS case 2 outside of the unified activated TCI states. If there are no such cases, then the option 2 can still work otherwise the option 1 shall be considered.

Proposal 2a: If the SRS case 2 (i.e. followUnifiedTCI-StateSRS is not configured for a SRS resource set) should be considered, RAN2 to further confirm whether there are use cases that the network would configure/indicate the TCI states for the SRS case 2 outside of the unified activated TCI states. The option 1 should be selected if there are, otherwise the option 2 should be selected.

For the proposal 2 and proposal 2a, if RAN2 can’t achieve consensus, a LS can be sent to RAN1 for confirmation.

Proposal 3: For the proposal 2 and proposal 2a, if RAN2 can’t achieve consensus, a LS can be sent to RAN1 for confirmation.

Based on the above discussion, if the option 1 was selected, RAN2 can further discuss the maximum updated number. If the option 2 was selected, RAN2 can further discuss whether the PL offset would be updated together with the TCI state Activation/Deactivation, in the next chapter, we would focus on this issue.

2.2 Joint or Separate from the TCI State Activation/Deactivation MAC CE

In the last meeting, for the option 2, there are also 2 alternatives:

- Alt 1: Use a separate MAC CE to update the PL offset only;

- Alt 2: Include the PL offset update and the TCI state Activation/Deactivation in one MAC CE.

Proposal 4: If the option 2 (i.e. only update the PL offset for the activated TCI states) was selected, Ran2 to discuss which alternative is preferred:

- Alt 1: Use a separate MAC CE to update the PL offset only;

- Alt 2: Include the PL offset update and the TCI state Activation/Deactivation in one MAC CE.

These 2 alternatives can be used for the different scenarios as below respectively.

- Scenario 1: PL offset update without the activated TCI state change;

- Scenario 2: the activated TCI states and the related PL offsets change simultaneously.

For the scenario 2, considering that there is delay for the active TCI state switching (as defined in the 38.133), if the TCI state activation delay is enough for the UE to process a following MAC CE, the solution 1 can also work, which means for the scenario 2, the UE can send a TCI state Activation/Deactivation MAC CE followed by a PL offset update MAC CE.

Based on the above discussion, the Alt1 needs to be supported for the scenario 1. Whether the Alt 2 is necessary can be further discussed, which is also depends on whether the TCI state activation delay is enough for the UE to process a following MAC CE.

Proposal 4a: If the option 2 (i.e. only update the PL offset for the activated TCI states) was selected, support Alt 1 as baseline, the Alt 2 can be further discussed.

- Alt 1: Use a separate MAC CE to update the PL offset only;

- Alt 2: Include the PL offset update and the TCI state Activation/Deactivation in one MAC CE.

2.3 PL Offset MAC CE Details

In this chapter, we’d like to discuss some details issues for the PL offset update MAC CE as mentioned in the proposal 6 of [2].

Chair: companies are encouraged to check the listed questions in P6 in R2-2409023. We will discuss whether/what to ask to R1. Proposal 6: For the PL offset update MAC CE, discuss the following aspects for the MAC CE design. 1. Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells? 2. How to indicate the PL offset value a. Option 1: indicate an absolute PL offset, with a step size of 4dB, and wait for RAN1 for the value range. b. Option 2: indicate a delta PL offset with respect to the RRC configured value, with a step size of 4dB, and wait for RAN1 for the value range. 3. How many joint/UL TCI states are mapped to a PL offset? a. Option 1: one TCI state (either joint or UL) is indicated per PL offset. b. Option 2: multiple TCI states (either joint or UL) are indicated per PL offset, considering multiple TCI states can have the same updated PL offset value. 4. The TCI states in the MAC CE are at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH. 5. If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

Issue 1: Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells

In the R17/18, the serving cell set mechanism has been supported with simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2. Thus this mechanism can also be kept for the PL offset update MAC CE. However if it requires to introduce new simultaneousSpatial-UpdatedList, it would cause some specification complexity. Furthermore, this issue can be taken as lower priority, after we have clear picture on the MAC CE design for the single CC case.

Proposal 5: on “Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells”, the single serving cell case can be taken as baseline, the multiple serving cells case can be further discussed with lower priority.

Issue 2: How to indicate the PL offset value
    a.	Option 1: indicate an absolute PL offset, with a step size of 4dB, and wait for RAN1 for the value range.
    b.	Option 2: indicate a delta PL offset with respect to the RRC configured value, with a step size of 4dB, and wait for RAN1 for the value range.

The option 2 has been discussed in RAN1 #117, but no consensus achieved in RAN1.

RAN1#117There is no consensus on the following proposal: Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows: - When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW.

Thus the option 1 shall be selected. Furthermore the value ranges has been agreed to BE [-12, 60]dB with step size 4dB, which means 5bits would be enough for the PL offset indication.

RAN1#118 Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB RAN 1#118bis Agreement The lower limit of the value range of PL offset is extended to -12 dB

Proposal 6:  indicate an absolute PL offset with 5 bits and a step size of 4dB.

Issue 3:  How many joint/UL TCI states are mapped to a PL offset

a.	Option 1: one TCI state (either joint or UL) is indicated per PL offset. 
    b.	Option 2: multiple TCI states (either joint or UL) are indicated per PL offset, considering multiple TCI states can have the same updated PL offset value.

On these 2 options, the option 2 can be seen as the optimization of the option 1. A similar optimization was also discussed in RAN1 for the RRC configurations, e.g. whether to define a list of the PL offset, then an index can be used for each TCI state to indicate which PL offset is used. However, this mechanism was not agreed in RAN1, thus there is also no need to support optimization in the option 2. Though there are some differences between the MAC CE and RRC mechanism, the key issue is the same (i.e. whether it’s necessary to introduce any optimization for the case that multiple TCI states have the same updated PL offset value).

Proposal 7: The mechanism “multiple TCI states (either joint or UL) are indicated per PL offset” is not supported.

Issue 4: The TCI states in the MAC CE are at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

This issue is about the applied channel type, as we analyzed in the chapter 2.1, we agree with these channel types.

Proposal 8: The TCI states in the MAC CE are applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Issue 5: If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

The description in the issue 5 is also aligned with many legacy MAC CE definition, which can be taken as the common rule for the joint/separate case.

Proposal 9: If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 to discuss which TCI state type is preferred for the PL Offset Update MAC CE

- Option 1: Update the PL offset for the configured TCI states;

- Option 2: Only update the PL offset for the activated TCI states.

Observation 1: For the case that the network may intend to update PL offset for the TCI state that are potentially activated later, the network can include TCI state Activation/Deactivation  and PL offset update in one MAC CE or send a TCI state Activation/Deactivation  MAC CE followed by a PL offset update MAC CE .

Proposal 2: RAN2 to confirm whether the SRS case 2 (i.e. followUnifiedTCI-StateSRS is not configured for a SRS resource set) should be considered for the PL offset MAC CE design.

Proposal 2a: If the SRS case 2 (i.e. followUnifiedTCI-StateSRS is not configured for a SRS resource set) should be considered, RAN2 to further confirm whether there are use cases that the network would configure/indicate the TCI states for the SRS case 2 outside of the unified activated TCI states. The option 1 should be selected if there are, otherwise the option 2 should be selected.

Proposal 3: For the proposal 2 and proposal 2a, if RAN2 can’t achieve consensus, a LS can be sent to RAN1 for confirmation.

Proposal 4: If the option 2 (i.e. only update the PL offset for the activated TCI states) was selected, Ran2 to discuss which alternative is preferred:

- Alt 1: Use a separate MAC CE to update the PL offset only;

- Alt 2: Include the PL offset update and the TCI state Activation/Deactivation  in one MAC CE.

Proposal 4a: If the option 2 (i.e. only update the PL offset for the activated TCI states) was selected, support Alt 1 as baseline, the Alt 2 can be further discussed.

- Alt 1: Use a separate MAC CE to update the PL offset only;

- Alt 2: Include the PL offset update and the TCI state Activation/Deactivation in one MAC CE.

Proposal 5: on “Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells”, the single serving cell case can be taken as baseline, the multiple serving cells case can be further discussed with lower priority.

Proposal 6:  indicate an absolute PL offset with 5 bits and a step size of 4dB.

Proposal 7: The mechanism “multiple TCI states (either joint or UL) are indicated per PL offset” is not supported.

Proposal 8: The TCI states in the MAC CE are applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Proposal 9: If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

- Reference

- R1-2407285  LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5   RAN1

- R2-2409023	Discussion on RAN2 impacts	Samsung	discussion

- R2-2408092	RAN2 Impacts of Rel-19 NR MIMO	CMCC	discussion