3GPP TSG-RAN WG2 Meeting #128	R2-2410771

Orlando, USA, Nov18th–22nd, 2024

Source:        ZTE Corporation

Title:           Consideration on the UEIBM for R19 MIMO

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

In the last meeting, the UEIBM topic was touched, but the companies’ views are divergent, in this chapter we share our views on this topic based on the latest RAN1 and RAN2 progress.

- Discussion

In this chapter, we first discuss the UEIBM impact on MAC layer, then give our preliminary thinking on the RRC parameters structure, at last we share our understanding on how to handle LTM and UEIBM.

2.1 UEIBM Impact on MAC layer

For the UEIBM, RAN1 has discussed the Events definition (e.g. Event 1, 2 and 7), event trigger condition, UEIBR (UE initiated Beam Report) procedure (e.g. mode A and mode B) and also other related topics. On the event trigger condition, a window would be introduced for the instance statistics, however, whether this mechanism is captured in RAN1 or RAN2 specification is still FFS. In RAN1 discussion, Ran1 has touched some detail issues discussion on this mechanism in [1], e.g. how to calculate the instances, how to define the period for the instances, when to reset the counting and so on. They also have made some agreements on these issues, e.g.

Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

RAN1 may make conclusion based on these detail issues discussion result. Thus, to avoid repeated discussion, RAN2 can wait for Ran1’s further progress.

Proposal 1: On the UEIBR impact on the MAC layer procedure, wait for RAN1’s further progress.

2.2 RRC Impact of the UE Initiated Beam Management (UEIBM)

The RRC impact is mainly from the RRC Parameters, according to the RAN1’s discussion, some UEIBM related RRC parameters have been discussed as below:

| RAN2 Parent IE | Parameter name in the spec | Description | Value range |
| --- | --- | --- | --- |
| [CSI-ReportConfig] | reportTransmissionMode-r19 | This parameter is used to inform one report transmission mode… | {ModeA, ModeB} |
| CSI-ReportConfig | eventThreshold-r19 | This parameter is used to inform a threshold value for trigger event detection regarding Event-2. | FFS |
| CSI-ReportConfig | eventDetectionTimeWindowLength-r19 | This parameter is used to inform the time window for triggering event determination regarding Event-2 | FFS |
| CSI-ReportConfig | eventInstanceCount-r19 | This parameter is used to inform the number of event 2 instances for at least one same new beam within a configured time window that the UE can initiate UEIBM report | FFS |
| CSI-ReportConfig | enabledCurrentBeamReport-r19 | This parameter is to enable or disable whether current beam is always reported. | Up to RAN2 |
| CSI-ReportConfig | nrofReportedRS-UEIBR-r19 | This parameter is used to inform the number of reported RS(s) in a report format for UE-initated/event-driven beam reporting. | FFS |
| CSI-ReportConfig | newBeamResourceSetEvent2-r19 | This parameter is used to confiure the new beam resource set for Event 2 (Opt1) | FFS |
| CSI-ReportConfig | eventType-r19 | This parameter is used to inform a event type for UE-initated/event-driven beam reporting.- Event-1/2/7 | {Event1, Event2, Event7} |
| CSI-ReportConfig | valueOfM | This parameter is used to inform the value of M for Event7 | FFS |
| CSI-ReportConfig | configuredResourceForSecondChannelOfModeB | This parameter is used to inform Type-1 CG PUSCH resource for second channel in mode-B | configuredGrantConfigIndex |

These parameters can be divided into 2 types:

- Type 1: Procedure specific parameters: reportTransmissionMode-r19, configuredResourceForSecondChannelOfModeB

- Type 2: Event specific parameters:

- Event 2: eventThreshold-r19, eventDetectionTimeWindowLength-r19, eventInstanceCount-r19, eventInstanceCount-r19, nrofReportedRS-UEIBR-r19, newBeamResourceSetEvent2-r19.

- Event 7: valueOfM, and may also include the parameters currently defined for the Event 2

For the type 1, it’s quite clear that it shall be configured at the per cell level, but for the type 2 parameters, according to the RAN1’s discussion, the UEIBM are triggered at BWP level (i.e. the active BWP) but the parent IE CSI-ReportConfig is per cell level. Then question is whether and how to associate these event specific parameters with BWP ID.

Observation 1: The UEIBM would be managed on the BWP level, however the current RAN1 parameters are defined as per Cell level.

Take the newBeamResourceSetEvent2-r19 as an example:

| CSI-ReportConfig | newBeamResourceSetEvent2-r19 | This parameter is used to confiure the new beam resource set for Event 2 (Opt1) | FFS |
| --- | --- | --- | --- |

According to the RAN1 agreement, the RS(s) for new beam(s) are explicitly configured. Normally the CSI-RS resource was indicated by the NZP-CSI-RS-Resource.

RAN1#118 Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

In the current the CSI-ResourceConfig framework as shown in the Fig1, the BWP ID is included. There is no restriction that NZP-CSI-RS-ResourceID can only be included by only one CSI-ResourceConfig, thus a NZP-CSI-RS-Resource can be referred by the one or more BWP ID(s).

Fig 1: Framework of the Legacy CSI-ReportConfig

Then the question is how to associate the new beams (identified by the NZP-CSI-RS-ResourceID) configuration for each BWP:

- If the new beams are configured at a cell level, the UE can only determine the related BWP for each new beam based on the corresponding CSI-ResourceConfig->bwp-Id.

For example, if the new beam indexed with NZP-CSI-RS-ResourceID x, and the NZP-CSI-RS-  ResourceID x is referred by both BWP 1 and BWP2, but not referred by the BWP3, then when the BWP1 is activated, the beam indexed with NZP-CSI-RS-ResourceID x would be taken as one of the new beam, but when the UE switches to the BWP3 with MAC CE, the UE would not take this beam as new beam.

- If the new beams are configured at a BWP level, it would be easier for the UE determination, which is also quite flexible from the network configuration aspect.

Proposal 2: Ran2 to discuss whether and how to associate the RAN1 defined event specific parameters (e.g. newBeamResourceSetEvent2-r19) with BWP ID, if no consensus achieved, a LS can be sent to RAN1 for confirmation.

2.2 How to handle LTM and UEIBM

In the last meeting, another issue is on how to handle the LTM and UEIBM. Because of the similarity of these 2 topics, if the same framework (same MAC procedure and/or RRC configuration structure) was used, it can reduce the specification complexity. However, according to the current progress of each topic, at least it’s immature to harmonize these 2 topics with one framework. At this stage, the two topics can be discussed independently, and when both pictures for these 2 topics are clear, it can be further discussed which parts can be unified.

Proposal 3: The LTM and UEIBM should be discussed independently at this stage, it can be further discussed which parts can be unified when the pictures for both topics are clear.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: On the UEIBR impact on the MAC layer procedure, wait for RAN1’s further progress.

Observation 1: The UEIBM would be managed on the BWP level, however the current RAN1 parameters are defined as per Cell level.

Proposal 2: Ran2 to discuss whether and how to associate the RAN1 defined event specific parameters (e.g. newBeamResourceSetEvent2-r19) with BWP ID, if no consensus achieved, a LS can be sent to RAN1 for confirmation.

Proposal 3: The LTM and UEIBM should be discussed independently at this stage, it can be further discussed which parts can be unified when the pictures for both topics are clear.

- Reference

- R1-2407285  LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5   RAN1