3GPP TSG-RAN WG2 Meeting #128	R2-2410772

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda item:	6.1.3.3

Title:	On eDRX operation with Emergency Services

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the issue related to eDRX operation when UE has PDU session associated with emergency services was raised. This contribution provides some considerations on this issue.

Discussion

In TS 23.501, it is stated that when the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception and IDLE eDRX shall not be used.

At the same time, the IDLE eDRX can still be negotiated between AMF and UE in this case. The negotiated IDLE eDRX is intended to be used when the PDU session associated with emergency services is released. The relevant chapter is copied below:

#### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part

And in TS 24.501[3], it is stated that “UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session”.

### 5.3.16 Extended DRX cycle for UEs in 5GMM-IDLE and 5GMM-CONNECTED mode with RRC inactive indication // skipped irrelevant part The UE and the network may negotiate eDRX parameters during a registration procedure when the UE has an emergency PDU session. The network accepts the request to use the eDRX by providing the Negotiated extended DRX parameters IE when accepting the registration procedure. The UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session. // skipped irrelevant part

Observation 1: According to 23.501/24.501, when UE has PDU session associated with emergency services, IDLE eDRX may be negotiated between AMF and UE but shall not be used. Instead, regular DRX is used by UE and AMF.

In current RAN specification, RAN determines whether to configure INACTIVE eDRX according to the IDLE eDRX configuration signaled from AMF. Unfortunately, there is no standard means for RAN to know whether the UE has PDU session associated with emergency services. So, the RAN may configure INACTIVE eDRX for the UE as long as UE’s IDLE eDRX configuration is signaled to the RAN.

Observation 2: In current specification, RAN cannot know whether UE has PDU session associated with emergency services. The RAN may configure INACTIVE eDRX if IDLE eDRX configuration is signaled to the RAN from AMF even if UE has PDU session associated with emergency services.

Current specification does not support to use INACTIVE eDRX but not use IDLE eDRX. The rules to determine whether UE operates in eDRX, and rules to determine PF and PO if UE operates in eDRX are based on assumption that IDLE eDRX is configured and used.

Further, we understand the intention to not use IDLE eDRX is to avoid long paging delay in emergency service callback. This intention also applies to INACTIVE eDRX. So we propose to capture in specification explicitly that UE shall not use INACTIVE eDRX if UE has PDU session associated with emergency services.

Proposal 1: To capture in specification that UE shall not use INACTIVE eDRX if UE has PDU session associated with emergency services.

In Annex A and B, we provide text proposals for TS 38.304 Rel-17 and Rel-18 respectively.

Proposal 2: RAN2 to adopt the text proposals provided in Annex A and B for TS 38.304 Rel-17 and Rel-18 respectively.

As stated above, in current specification, RAN cannot know that UE has PDU session associated with emergency services. Thus even if UE does not apply IDLE eDRX and INACTIVE eDRX, the RAN may still assume the UE operates in eDRX, and transmit RAN paging or CN paging message according to IDLE/INACTIVE eDRX cycle as long as IDLE/IANCTIVE eDRX are configured.

The consequence is the long paging delay caused by the eDRX cycle. The delay issue becomes more severe if long INACTIVE eDRX cycle is configured. The maximum value of INACTIVE eDRX is 1024 hyper frame, i.e. about 2.9 hours.

Observation 3: Even if UE does not use IDLE/INACTIVE eDRX, the RAN will still use IDLE eDRX/INACTIVE eDRX if RAN does not know that the UE has PDU session associated with emergency service.

So, we proposed RAN2 to discuss whether and how to address this issue.

One option is to leave to network implementation to avoid such situation. E.g. the AMF does not signal IDLE eDRX to RAN if UE has PDU session associated with emergency services. Thus the RAN will not configure INACTIVE eDRX. Then the legacy DRX are used for both CN and RAN paging. This option has no impact on specification.

Another option is to indicate in the signaling from AMF to RAN whether UE has PDU session associated with emergency services. In this way, RAN can configure INACTIVE eDRX but not use IDLE/INACTIVE eDRX.  The RAN can use the IDLE/INACTIVE eDRX until the AMF indicates the PDU session associated with emergency services is released. This option has impact on Ng specification.

Proposal 3: RAN2 to decide whether a standard solution is needed to get RAN aware of whether UE has PDU session associated with emergency services, or leave to network implementation to prevent RAN from using eDRX for the UE.

Conclusion

Observation 1: According to 23.501, when UE has PDU session associated with emergency services, IDLE eDRX may be negotiated between AMF and UE but shall not be used. Instead, regular DRX is used by UE and AMF.

Observation 2: In current specification, RAN cannot know whether UE has PDU session associated with emergency services. The RAN may configure INACTIVE eDRX if IDLE eDRX configuration is signaled to the RAN from AMF even if UE has PDU session associated with emergency services.

Observation 3: Even if UE does not use IDLE/INACTIVE eDRX, the RAN will still use IDLE eDRX/INACTIVE eDRX if RAN does not know that the UE has PDU session associated with emergency service.

Proposal 1: To capture in specification that UE shall not use INACTIVE eDRX if UE has PDU session associated with emergency services.

Proposal 2: RAN2 to adopt the text proposals provided in Annex A and B for TS 38.304 Rel-17 and Rel-18 respectively.

Proposal 3: RAN2 to decide whether a standard solution is needed to get RAN aware of whether UE has PDU session associated with emergency services, or leave to network implementation to prevent RAN from using eDRX for the UE.

References

- TS 23.501 Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2

- TS 24.501 Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3

- TS 38.304 17.9.0

Annex A: TP 38.304 Rel-17 V17.9.0

## 7.4	Paging in extended DRX

The UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN. The UE operates in eDRX for CN paging in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1, and UE has no PDU Session associated with emergency services. The UE operates in eDRX for RAN paging in RRC_INACTIVE state if the UE is configured for eDRX by RAN and eDRX-AllowedInactive is signalled in SIB1, and UE has no PDU Session associated with emergency services. If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE operating in eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formula:

The PH for CN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX, CN= (UE_ID_H mod TeDRX, CN), where

-	UE_ID_H: 13 most significant bits of the Hashed ID.

-	TeDRX, CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX, CN = 2, …, 1024 Hyper-frames) configured by upper layers.

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX, CN, where

-	ieDRX, CN = floor(UE_ID_H /TeDRX, CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by upper layers

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of 5G-S-TMSI.

5G-S-TMSI = <b47, b46, …, b0> as defined in TS 23.003 [23].

The 32-bit FCS shall be the ones complement of the sum (modulo 2) of Y1 and Y2, where

-	Y1 is the remainder of xk (x31 + x30 + x29 + x28 + x27 + x26 + x25 + x24 + x23 + x22 + x21 + x20 + x19 + x18 + x17 + x16 + x15 + x14 + x13 + x12 + x11 + x10 + x9 + x8 + x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1) divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where k is 32; and

-	Y2 is the remainder of Y3 divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where Y3 is the product of x32 by "b31, b30…, b0 of S-TMSI or 5G-S-TMSI", i.e., Y3 is the generator polynomial x32 (b31*x31 + b30*x30 + … + b0*1).

NOTE:	The Y1 is 0xC704DD7B for any 5G-S-TMSI value. An example of hashed ID calculation is in Annex A.

Annex B: TP on 38.304 Rel-18 V18.3.0

## 7.4	Paging in extended DRX

The UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN.

For CN paging, the UE operates in eDRX in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1, and UE has no PDU Session associated with emergency services; otherwise, the UE does not operate in eDRX.

For RAN paging, the UE in RRC_INACTIVE state:

-	if the UE is configured for eDRX by ran-ExtendedPagingCycleConfig-r18 and eDRX-AllowedInactive-r18 is signalled in SIB1. and UE has no PDU Session associated with emergency services:

-	operates in eDRX with an eDRX cycle TeDRX, RAN configured by extendedPagingCycle-r18;

-	else if the UE is configured for eDRX by ran-ExtendedPagingCycle-r17 and eDRX-AllowedInactive-r17 is signalled in SIB1, and UE has no PDU Session associated with emergency services:

-	operates in eDRX with an eDRX cycle TeDRX, RAN configured by ran-ExtendedPagingCycle-r17;

-	else:

-	does not operate in eDRX.

If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE operating in eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formula:

The PH for CN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX, CN= (UE_ID_H mod TeDRX, CN), where

-	TeDRX, CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX, CN = 2, …, 1024 Hyper-frames) configured by upper layers.

The PH for RAN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX_RAN= (UE_ID_H mod TeDRX_RAN), where

-	TeDRX_RAN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_RAN = 2, …, 1024 Hyper-frames) configured by RRC.

For CN configured PTW:

PTW_start denotes the first radio frame of the PH for CN that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX, CN, where

-	ieDRX, CN = floor(UE_ID_H /TeDRX, CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by upper layers

For RAN configured PTW:

PTW_start denotes the first radio frame of the PH for RAN that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX_CN, where

-	ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by RRC

UE_ID_H is defined as follows:

UE_ID_H: 13 most significant bits of the Hashed ID.

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of 5G-S-TMSI.

5G-S-TMSI = <b47, b46, …, b0> as defined in TS 23.003 [23].

The 32-bit FCS shall be the ones complement of the sum (modulo 2) of Y1 and Y2, where

-	Y1 is the remainder of xk (x31 + x30 + x29 + x28 + x27 + x26 + x25 + x24 + x23 + x22 + x21 + x20 + x19 + x18 + x17 + x16 + x15 + x14 + x13 + x12 + x11 + x10 + x9 + x8 + x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1) divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where k is 32; and

-	Y2 is the remainder of Y3 divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where Y3 is the product of x32 by "b31, b30…, b0 of S-TMSI or 5G-S-TMSI", i.e., Y3 is the generator polynomial x32 (b31*x31 + b30*x30 + … + b0*1).

NOTE:	The Y1 is 0xC704DD7B for any 5G-S-TMSI value. An example of hashed ID calculation is in Annex A.