3GPP TSG-RAN WG2 Meeting #128	R2-2410791

Orlando, US, 18th Nov.– 22nd Nov., 2024

Agenda Item:	8.11.2

Source:	vivo

Title:	Discussion on random access procedure in SBFD

Document for:	Discussion and Decision

Introduction

According to R2#127bis meeting, the following agreements were made[1]:

Early Identification RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Preamble transmission and re-attempt Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. RACH configuration options The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

In this contribution, we will further discuss the remaining issues on random access procedure in SBFD.

Discussion

RACH triggering events

During R2#127 meeting, RAN2 agreed on the following working assumption [2]:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

The major comments from the companies include:

- No strong concern is observed to exclude any of the events;

- Since the main benefit of SBFD is to reduce latency, we may focus on the latency sensitive events and connected state.

RAN1 agreed during last meeting to extend the previous RAN1 agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode. This implies not only the benefit from SBFD operation applies to both RRC_CONNECTED and RRC_IDLE/INACTIVE, but also we strive for a common design for these modes. Since we do not observe any strong concern to exclude any of the events, and it is anyway up to the network to determine whether to configure SBFD resources for certain purpose, we propose RAN2 to confirm the working assumption. Otherwise, we need to spend much time to case by case discuss which cases are allowed in SBFD while others are not.

Proposal 1: RAN2 to confirm the following working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Discussion on Msg-3 based early identification

During last meeting, RAN2 reached consensus on Msg-1 based early identification for SBFD [1]:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Yet, there was no consensus on Msg-3 based method. From our point of view, when the UE chooses additional RO for preamble transmission, the NW is able to identify SBFD UE and thus can schedule SBFD Msg3 resources so that the SBFD UE may have higher possibility to successfully complete the random access procedure. However, if the UE chooses legacy RO for preamble transmission, this may reflect that the current situation for the UE is not suitable for further using SBFD resources for the coming Msg3 transmission. Besides, the NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed. Therefore, we do not observe the benefit for Msg-3 based early identification if the UE chooses legacy RO with some form of RO selection criteria which RAN2 will further determine as agreed during last meeting [1]:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/ prioritizations, if no additional indication (FFS if there needs to be any) is from network.

Observation 1: If the SBFD-aware UE chooses legacy RO with the RO selection criteria which RAN2 will determine, it reflects the fact that SBFD-based transmission may not be a good choice from scheduling point of view within some upcoming duration. The NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed.

Proposal 2: Do not support Msg3-based early identification for SBFD-aware UE.

SBFD RACH configuration

RAN1 agreed on two options for SBFD RACH configuration [3], i.e. use one single RACH configuration or two separate RACH configurations to indicate ROs within UL sub-band in SBFD symbols. RAN1 also agreed on the following principles from UE point of view:

- Enabling both options at the same time for a UE is not supported;

- UE is not required to support both options;

During last meeting, RAN1 discussed the similar issue from NW point of view and concluded that:

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

Now we can expect that the discussion in RAN1 is closed and RAN2 should start the discussion on it. For RRC_CONNECTED UE, the NW can configure the corresponding SBFD RACH configuration according to the UE Capabilities report. However, it is not clear yet for IDLE/INACTIVE UE whether the NW can configure both SBFD RACH configuration options in the SIB1:

The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

We think before determining whether NW can provide both configurations, RAN2 can first discuss from RAN2’s perspective whether there’s any substantial difference between SBFD RACH configuration option 1 and option 2. During last meeting, RAN1 agreed that even for option 1, separate power control parameters are supported [4]:

Agreement For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - FFS: Details of the separate power control parameters

Therefore, we think both option 1 and option 2 can configure the same legacy ROs and additional ROs with independent power control parameters, and hence it can be easier to agree on the NW to configure only one of the SBFD RACH configuration options in SIB1, since the function of these two options can be the same. If the NW configures both options, it seems to be duplicated and unnecessary, as we may need to further define UE behavior on how and which the RACH configuration is applied.

Proposal 3: The NW configures only one of the SBFD RACH configuration options in SIB1.

Although RAN1 agreed that UE is not required to support both options, we think there is no harm for RAN2 to review this conclusion. Till this day, we think these two options only differ in the form of configuration, and it is really weird to say the SBFD-aware UE only supports one of these two options while not supporting the other option simply due to signaling difference. Therefore, we propose RAN2 to discuss whether a new UE capability is really needed to indicate which SBFD RACH configuration the UE supports.

Proposal 4: RAN2 to discuss whether a new UE capability is needed to indicate which SBFD RACH configuration option the UE supports, or a UE supporting SBFD operation supports both SBFD RACH configuration options (i.e. no new UE capability for indication of supported RACH configuration).

Random access procedure

Carrier selection

A serving cell can be configured with supplementary uplink (SUL) for the purpose of coverage enhancement. In the Rel-18 WI Coverage Enhancement (CovEnh), it was discussed and determined that:

- Msg1 repetition can be configured on SUL;

- It is up to the NW whether to configure both NUL with Msg1/3 repetition and SUL for the purpose of CovEnh;

In SBFD, we propose that similar principle can also be agreeable.

Proposal 5: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement.

The next issue is whether SBFD operation has an impact on carrier selection for random access procedure. In legacy procedure, the UE directly determines the UL carrier if it is explicitly indicated by the NW. Otherwise, the UE compares the DL RSRP with the NW configured SUL threshold to determine it [4].

In SBFD, the NW can enable Msg1 repetition in SBFD resources on TDD-NUL according to RAN1 agreement [2]. The UE compares the DL RSRP with Msg1 repetition threshold to determine the number of repetition. Msg1 repetition on SBFD resources enhance the NUL coverage to some extent, which weakens the motivation for the UE to select SUL. Therefore, we propose RAN2 to discuss whether the NW configuring SBFD on NUL has an impact on NUL/SUL selection. For example, if the UE selects to perform SBFD operation on NUL, the UE can rely on Msg1 repetition with SBFD on NUL instead of choosing SUL.

Proposal 6: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

SSB/CSI-RS selection is a step in random access procedure. If not indicated by the NW, the UE compares the RSRP of the SSB/CSI-RS with the threshold and select an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. The UE then determines RO based on the selected SSB/CSI-RS [5].

In SBFD, legacy ROs and additional ROs can be both mapped to the same SSB/CSI-RS. Considering the NW can evaluate the CLI in SBFD resources based on the UE reports and adjust the SBFD configuration accordingly, the NW may configure different threshold for the UE to select additional ROs or legacy ROs to minimize the CLI in SBFD resources. However, we may just rely on the legacy SSB/CSI-RS selection method. Therefore, we propose RAN2 to confirm that SBFD operation has no impact on SSB/CSI-RS selection for random access procedure.

Proposal 7: Legacy SSB/CSI-RS selection procedure can be reused for random access procedure with SBFD operation.

RO selection

During last meeting, RAN2 agreed to determine certain criteria for RO selection in SBFD [1]:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

RO selection based on absolute prioritization rule is not suggested, e.g. SBFD-aware UE always prioritize additional ROs for preamble transmission, which is very inflexible for NW control. We observe that some companies understand RO selection based on configured RSRP threshold can be considered as a method of relative prioritization rule, and feel reasonable to further discuss on it. There are two main aspects the NW may consider whether to configure/schedule SBFD resources to SBFD-aware UE. One is configuring SBFD for coverage enhancement, and the other is to avoid severe UE-to-UE CLI.

Different configuration strategy determines how an RSRP threshold is used for RO selection in SBFD. For example, if the main concern of NW configuring SBFD is for coverage enhancement, the NW can configure the UE to “prioritize additional ROs” when the RSRP threshold of the selected SSB/CSI-RS is below a configured RSRP threshold. Another example is that if the NW configures additional ROs only for larger RACH capacity and does not mean to have severe UE-to-UE CLI across different SBFD-capable cells, the NW can configure the UE to “prioritize additional ROs” when the RSRP threshold of the selected SSB/CSI-RS is above a configured RSRP threshold. From the given examples, one can tell how the UE prioritizes selection of additional ROs with the configured RSRP threshold depends on the target the NW wants to achieve. Therefore, we propose RAN2 to first determine to introduce a new RSRP threshold for RO selection between additional ROs and legacy ROs, and then discuss how the configured RSRP threshold is used.

Proposal 8: The NW configures RSRP threshold(s) for UE to determine RO selection between additional RO and legacy RO.

Proposal 9: RAN2 to discuss how the configured RSRP threshold is used for RO selection (Note that the thresholds of the following two options may be separate):

- Option 1: e.g. for minimizing CLI across cells/UEs

- If the RSRP of the selected SSB/CSI-RS is above the configured threshold, the UE selects additional RO;

- Else, the UE selected legacy RO.

- Option 2: e.g. for coverage enhancement

- If the RSRP of the selected SSB/CSI-RS is below the configured threshold, the UE selects additional RO;

- Else, the UE selected legacy RO.

If certain RO selection criteria is determined by RAN2, one may wonder whether the criteria can vary when different SBFD RACH configuration option is applied. We suggest RAN2 to strive for a common solution for these two options.

Proposal 10: RAN2 to strive for a common solution for RO selection with either RACH configuration Option 1 or Option 2.

PRACH transmission re-attempt

During last meeting, RAN2 agreed as follows on PRACH transmission re-attempt [1]:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

From technical point of view, the discussion on PRACH transmission re-attempt should be pending on how the RO selection among additional ROs and legacy ROs is determined. However, we think if the UE starts preamble transmission with legacy ROs, it is better to stick to this behavior until the whole RACH procedure is failed, since it is not common that the UE determines that it should first use legacy ROs and then somehow after a few attempts within a RACH procedure it meets the criteria for selecting additional ROs. Therefore, we suggest RAN2 to conclude that if the UE selects legacy ROs first, UE keeps using legacy ROs until RACH procedure failure.

Proposal 11: If the UE selects legacy ROs first in one RACH procedure, UE keeps using legacy ROs until this RACH procedure failure, i.e. the UE is not allowed to switch to additional ROs after several PRACH transmission (re)attempts on legacy ROs in one RACH procedure.

RACH failure determination

Since RAN2 has agreed that “for the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions.” [1], we observe no need to separately monitor RA problems for SBFD RA and legacy RA.

Proposal 12: The MAC layer does not monitor RA problems separately for SBFD RA operation and legacy RA operation.

Conclusion

Based on the discussion, we have the following observation and proposals:

RACH triggering events

Proposal 1: RAN2 to confirm the following working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Discussion on Msg-3 based early identification

Observation 1: If the SBFD-aware UE chooses legacy RO with the RO selection criteria which RAN2 will determine, it reflects the fact that SBFD-based transmission may not be a good choice from scheduling point of view within some upcoming duration. The NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed.

Proposal 2: Do not support Msg3-based early identification for SBFD-aware UE.

SBFD RACH configuration

Proposal 3: The NW configures only one of the SBFD RACH configuration options in SIB1.

Proposal 4: RAN2 to discuss whether a new UE capability is needed to indicate which SBFD RACH configuration option the UE supports, or a UE supporting SBFD operation supports both SBFD RACH configuration options (i.e. no new UE capability for indication of supported RACH configuration).

Random access procedure

Carrier selection

Proposal 5: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement.

Proposal 6: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

Proposal 7: Legacy SSB/CSI-RS selection procedure can be reused for random access procedure with SBFD operation.

RO selection

Proposal 8: The NW configures RSRP threshold(s) for UE to determine RO selection between additional RO and legacy RO.

Proposal 9: RAN2 to discuss how the configured RSRP threshold is used for RO selection (Note that the thresholds of the following two options may be separate):

- Option 1: e.g. for minimizing CLI across cells/UEs

- If the RSRP of the selected SSB/CSI-RS is above the configured threshold, the UE selects additional RO;

- Else, the UE selected legacy RO.

- Option 2: e.g. for coverage enhancement

- If the RSRP of the selected SSB/CSI-RS is below the configured threshold, the UE selects additional RO;

- Else, the UE selected legacy RO.

Proposal 10: RAN2 to strive for a common solution for RO selection with either RACH configuration Option 1 or Option 2.

PRACH transmission re-attempt

Proposal 11: If the UE selects legacy ROs first in one RACH procedure, UE keeps using legacy ROs until this RACH procedure failure, i.e. the UE is not allowed to switch to additional ROs after several PRACH transmission (re)attempts on legacy ROs in one RACH procedure.

RACH failure determination

Proposal 12: The MAC layer does not monitor RA problems separately for SBFD RA operation and legacy RA operation.

References

- R2-2409211	Report from session on V2X/SL, R19 NES and MOB

- R2-2407572	Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R1#117 meeting minutes

- R1#118bis meeting minutes

- TS 38.321, V18.3.0