3GPP TSG-RAN WG2 Meeting #128	R2-2410792

Orlando, US, 18th Nov.– 22nd Nov., 2024

Agenda Item:	8.11.2

Source:	vivo

Title:	Discussion on other aspects in SBFD

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss about L1 based UE-to-UE CLI measurement and reporting.

Discussion

SRS-RSRP measurement and reporting

During last meeting, RAN1 agreed on the following for SRS-RSRP measurement and reporting [1]:

Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused

L3 based CLI measurement and reporting was introduced in Rel-16 [2], which is only for periodical reporting. In SBFD, aperiodic reporting is also supported. New parameters regarding L1 based CLI measurement and reporting will be defined to support both aperiodic reporting and periodical reporting. As agreed, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by RRC. It is likely that the signaling structure for L1 based CLI measurement and reporting is new and separate from the legacy one for L3 based CLI measurement report, and hence the legacy SRS resource set can be reused.

Proposal 1: For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the legacy SRS resource set is reused.

CLI-RSSI measurement and reporting

During last meeting, RAN1 agreed on the following for CLI-RSSI measurement and reporting [1]:

Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Regarding the FFS, if per-DL-subband CLI-RSSI measurements report is enabled for a CLI report configuration, separate measurement reports for two DL subbands are reported from UE, which is similar to subband CSI report. However, this requires the UE to perform more than one measurement report for a CLI report and to report two measurement results, which would increase the reporting overhead. Besides, it requires the UE to have stronger capability to perform two separate measurement reports within a CLI reporting. Two measurement reports for the two DL subband may not be necessary as in some cases the CLI measurement result can be quite similar. In fact, a wideband CLI measurement report across two DL subbands would provide sufficient information of CLI for gNB. If finer granularity of CLI measurement would be needed, NW can configure two separate CLI reporting configurations, each of which is associated with each DL subband, respectively. Therefore, we propose not to persue two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Proposal 2: Not pursue two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the legacy SRS resource set is reused.

Proposal 2: Not pursue two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

References

- R1#118bis meeting minutes

- TS 38.331, V18.3.0