3GPP TSG-RAN WG2 Meeting#128                               	R2-2410793

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.18

Source: 	Google, Samsung, MediaTek Inc.

Title: 	Inclusion of the NB-IoT satellite information in E-UTRAN

Document for:	Discussion and Decision

# Introduction

In the RAN2#127bis meeting [1], an issue was raised regarding the types of the satellite assistance information that can be provided in SIB33. The CR R2-2408648 [2] intends to make it possible for the TN operator to provide the satellite assistance information for the satellites offering NB-IoT services in SIB33, but the discussion was postponed to allow for a more comprehensive solution.

R2-2408648 Enabling SystemInformationBlockType33 for NB-IoT NTN Google CR Rel-18 36.331 18.3.1 5060 - F IoT_NTN_enh-Core - Inmarsat supports this change as this is an important use case. QC would also support this but thinks it’s not sufficient - Novamint agrees it’s an important use case to cover - ZTE thinks this is new feature, not an actual correction. CATT has the same feeling and wonders if this would be a cat F CR. Samsung thinks that if want to support this we should have a clear agreement that this is supported. - CMCC thinks we discussed and excluded this. Xiaomi thinks this was excluded in RAN plenary discussion. Inmarsat doesn’t think this was discussed nor ruled out during a RAN plenary. - ZTE still wonders if this would be an incomplete solution - Google think this is needed for initial search - Samsung agrees there is a use case and supports the CR - Ericsson thinks this would be an optimization: a UE should be able to perform cell search on a IoT-NTN cell from scratch - MTK thinks that from ASN.1 point of view this is already supported and so we could clarify this in the description. - Come back in the next meeting

In this paper, sourcing companies propose a way forward to enable the inclusion of the NB-IoT satellite information in E-UTRAN, with the intention to facilitate the inter-RAT (i.e., E-UTRAN to NB-IoT NTN) cell selection.

# Discussion

As SystemInformationBlockType33 contains the satellite assistance information needed for performing the neighboring NTN cell measurement and SystemInformationBlockType33 is only available for the E-UTRA or eMTC UE (i.e., not available for the NB-IoT UE), it is intuitive that SystemInformationBlockType33 only contains the satellite assistance information for measuring the eMTC NTN cells. Moreover, the linkage between the satellite IDs provided in SystemInformationBlockType33 and the carrier frequencies listed in SIB3 or SIB5 further implies that the satellites described in SystemInformationBlockType33 are only operating in the bands listed in SIB3 and SIB5, which obviously are e-UTRAN/eMTC bands.

The satellite assistance information provided in SystemInformationBlockType33 is not intended for performing measurement on NB-IoT NTN cells.

On the other hand, SystemInformationBlockType33-NB does provide the satellite assistance information for measuring the NB-IoT NTN cells, but it is not available in a regular E-URTAN cell. Given now the NB-IoT NTN has been commercialized and supported by some smartphones capable of LTE, it is beneficial for these UEs capable of both NB-IoT NTN and E-UTRA TN to obtain the satellite assistance information from the E-UTRA TN, so that these UEs can use the satellite assistance information to search an NB-IoT NTN cell during the initial cell selection. To make this happen, SystemInformationBlockType33 should be allowed to carry the satellite assistance information for NB-IoT NTN cells, and hence at least some changes are needed in TS 36.331 to overturn Observation 1.

Since SIB27 is already used to carry the information for inter-RAT (i.e., E-UTRAN to NB-IoT) cell selection, the first change is to create an association between the satellite assistance information in SIB33 and the NB-IoT carrier frequency information in SIB27, so as to explicitly indicate which satellite assistance information in SIB33 is for NB-IoT NTN and which is for eMTC NTN. The TP for the first change can be found in the Annex.

Since now SIB33 can be used to carry the satellite assistance information for different RATs, we think a second change may be needed in the SIB33 description or in the stage 2 description, to clarify that SIB33 (or E-UTRAN) may carry the satellite assistance information for BL UEs, UEs in enhanced coverage, and/or NB-IoT NTN capable UEs.

In view of the imperative market need for the smooth switch from E-UTRA TN to NB-IoT NTN, we respectfully ask RAN2 to enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and to create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27.

Enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27 (TP attached).

# Conclusion

In this paper, we discussed the issue triggered by an imperative market need for the  smooth switch from E-UTRA TN to NB-IoT NTN, and have the following observation.

Observation 1	The satellite assistance information provided in SystemInformationBlockType33 is not intended for performing measurement on NB-IoT NTN cells.

Based on the observation and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposal.

Proposal 1	Enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27 (TP attached).

# Reference

Draft Report of 3GPP TSG RAN WG2 meeting #127bis, ETSI MCC.

R2-2408648, Enabling SystemInformationBlockType33 for NB-IoT NTN, Google.

# Annex (TP for TS 36.331)

TP of the first change

#### –	SystemInformationBlockType27

The IE SystemInformationBlockType27 contains information relevant only for inter-RAT cell selection i.e. assistance information about NB-IoT frequencies for cell selection.

SystemInformationBlockType27 information element

-- ASN1START

SystemInformationBlockType27-r16 ::=	SEQUENCE {

carrierFreqListNBIOT-r16				CarrierFreqListNBIOT-r16		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...

[[	carrierFreqListNBIOT-v19xy			CarrierFreqListNBIOT-v19xy		OPTIONAL	-- Need OR

]]

}

CarrierFreqListNBIOT-r16 ::=				SEQUENCE (SIZE (1.. maxFreqNBIOT-r16)) OF	CarrierFreqNBIOT-r16

CarrierFreqListNBIOT-v19xy ::=				SEQUENCE (SIZE (1.. maxFreqNBIOT-r16)) OF	CarrierFreqNBIOT-v19xy

CarrierFreqNBIOT-r16 ::=		SEQUENCE {

carrierFreq-r16					ARFCN-ValueEUTRA-r9,

carrierFreqOffset-r16			ENUMERATED {v-10, v-9, v-8dot5, v-8, v-7, v-6, v-5, v-4dot5,

v-4,v-3, v-2, v-1, v-0dot5,	v0, v1, v2, v3, v3dot5,

v4, v5, v6, v7, v7dot5, v8, v9}

}

CarrierFreqNBIOT-v19xy ::=		SEQUENCE {

satelliteAssistanceInfoList-r18		SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18

}

-- ASN1STOP

SystemInformationBlockType27 field descriptions

carrierFreqListNBIOT Provides a list of neighbouring NB-IoT carrier frequencies, which may be searched for neighbouring NB-IoT cells.

carrierFreq Provides the ARFCN applicable for the NB-IoT carrier frequency as defined in TS 36.101 [42], Table 5.7.3-1.

carrierFreqOffset Offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [42], clause 5.7.3F. Value v-10 means -10, v-9 means -9, and so on. The values v-8dot5, v-4dot5, v3dot5 and v7dot5 are only applicable for a carrier in a TDD band.

satelliteAssistanceInfoList List of satellite ID(s), used to associate with the satellite assistance information for inter-RAT (i.e., E-UTRAN to NB-IoT NTN) cell selection.

End of TP

TP of the second change (optional)

#### –	SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells. When the SystemInformationBlockType33 is signalled in a TN cell, it may contain satellite assistance information for BL UEs, UEs in enhanced coverage, and/or NB-IoT NTN capable UEs.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

End of TP