3GPP TSG-RAN WG2 Meeting #128	R2-2410795

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.6.4

Source:	Nokia

Title:	On Conditional LTM

WID/SID:	NR_Mob_Ph4-Core- Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN #105, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1], where the objectives relevant to conditional LTM (CLTM) are described as:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM. In this contribution, we discuss the baseline procedure for CLTM support and identify the related key issues.

# 2	Discussion

We identify the following key aspects to be discussed in RAN2.

Phase # 1: Conditional cell switch condition configuration

Phase # 2: Conditional cell switch condition evaluation

Phase # 3: Early synchronization

Phase # 4: Conditional cell switch execution

## Conditional cell switch condition configuration

For CLTM, the NW-triggered procedures outlined in Rel-18 LTM would need to be revisited to enable the support of UE-autonomous cell switch triggering. It is anticipated that the basic principles of CHO will be reused. This implies that the UE will be configured with one or more conditions for conditional execution for one or more candidate cells or beams. When these conditions are met, the UE is expected to autonomously execute a CLTM cell switch to the candidate cell where the condition is satisfied.

To achieve this objective, it is necessary to study both - the types of conditions and how these conditions will be configured to the UE. In LTM, RS/beam-level measurements are used for early DL and UL synchronization procedures, as well as for the selection of a TCI state provided in the cell switch command. Following similar principles, for event-triggered L1 measurement reporting, beam/RS level events have already been agreed upon.

In RAN2#127bis, the following agreements related to the configuration of conditional execution conditions were made:

RAN2#127bis agreements: - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

To specify details of configuration of L1 measurement-based conditional execution conditions, it is important to understand how it was done in CHO. Conditional mobility is a paradigm introduced in Rel. 16 to enhance mobility robustness. This has been done through enhancement of the RRC measurement framework. For this a short introduction of the RRC measurement framework is needed.

Figure 1: Measurement object, measurement ID and the report configuration and the relation to conditional configuration.

In RRC measurement framework there are three key elements.

- Measurement ID

- Measurement Object

- Reporting configuration.

The measurement ID links the measurement object to reporting configuration.

The measurement object indicates a frequency band to be measured. The measurement object can also include list of cells UE is allowed to measure and the list of cells UE is not allowed to measure. The measurement object is designed in a way that it also allows UE to detect cells that it was not configured to measure with specific SSB locations. UE can scan the band configured in measurement object to detect the SSBs of neighbouring cells.

The reporting configuration indicates how the UE can report the measurements it conducted using the measurement object. The reporting configuration may indicate the reporting type, where it can be periodic, aperiodic or event based. In case of event based, the reporting configuration also indicates the event type, e.g., A3 and the event parameters such as time to trigger and offset.

In Rel. 16, the reporting configuration was enhanced to include the conditional execution condition. The conditional execution condition is indicated as a reporting type. Under this reporting configuration also the execution condition and related parameters are indicated. So instead of triggering a report UE would trigger the execution of a conditional configuration. This is achieved through linking the measurement ID to a conditional configuration ID.

Once a cell that is measured through the measurement object, fulfils the execution condition, UE is expected to decode the conditional configuration that is linked to the measurement. The UE compares if the PCI of the cell that fulfils the execution condition is the same as the cell that fulfils the execution condition. If it is the same, then UE executes handover using the conditional configuration. More details on this can be read in Section 5.3.5.13.4 and 5, TS 38.331. After the execution condition is fulfilled from the UE side, the execution is the same as a non-conditional handover.

CLTM can follow a similar structure in the sense that

- The LTM-CSI-reporting configuration can be enhanced with an indicator to differentiate between reporting and conditional execution condition.

  - Following the CHO and Rel. 18 LTM paradigm, this can be determined by the source DU for the initial conditional cell switch and by the candidate DUs for the subsequent conditional cell switches.

- LTM candidate configuration can be linked to a ReportConfigID that is configured with event-based reporting

  - The linking of report configuration with candidate configuration shall be done by source CU.

Observation 1: In Rel. 16 CHO, the reporting configuration was enhanced to include the conditional execution condition, where the conditional execution condition is indicated as a reporting type, and the execution condition and related parameters are indicated.

Proposal 1: For L1 execution conditions, LTM Candidate configuration is linked to Report-config-ID that is configured with conditional events.

Proposal 2: For L1 execution conditions, the DUs configure the conditional events in LTM reporting configuration.

Proposal 3: For L1 execution conditions, the CU can link the reporting configuration that indicates the execution condition to LTM candidate configuration.

In Rel-18, for FR1, L3 measurements can be used to trigger LTM. This is considered for the cases where UE does not support L1 measurements for some candidate frequencies. L3 measurement condition can also be considered for CLTM. Here, the L3 measurement-events are mapped to LTM configurations instead of CHO configurations. Additionally, NW can configure early L3 measurement events in this case to allow NW to do early sync procedures before the L3 condition for CLTM

Proposal 4: Support for L3 based conditions for CLTM.

### 2.1.1 Configuration for subsequent CLTM

The work item scope agreed to support subsequent CLTM.  For subsequent CLTM, the conditional events and their mapping need to be configured for each of the LTM candidate cells.  The condition and its mapping to candidate cells can be included in the LTM configuration structure, like in SCPAC. However, in SCPAC, the conditions are mapped to L3 measurement ID that does not have candidate cells included in the measurement object. In LTM, a report configuration is mapped to a resource configuration that consists of candidate ID and measurement RS information. One simple option to support subsequent CLTM is introducing the CLTM related parameters such as event definition and list of candidates /RSs within the given resource set in the report configuration within each candidate RRC configuration. This may require change of report configuration whenever candidate cell is modified in the LTM configuration. In the SCPAC approach only candidate ID can be mapped and require further changes to include beam specific information.

Proposal 5: RAN2 to down-select between SCPAC like configuration OR extension of report configuration to include CLTM parameters for subsequent LTM.

## Conditional cell switch condition evaluation

In Rel-18 network energy saving (NES) feature, the conditional evaluation is linked to NES mode of the serving cell. In other words, the conditional evaluation for the events will be triggered only when serving cell is in NES mode. Similar triggering can be considered for CLTM, where criteria such as availability of timing advance and early downlink synchronization may be used. This way, conditional LTM can benefit from the increased robustness of conditional cell switch execution without compromising on interruption time during cell switch.

Observation 2: NW triggering of condition evaluation for CLTM allows for increased robustness of the conditional cell switch execution without compromising on interruption time during cell switch.

Proposal 6: Network triggering of condition evaluation is supported.

If the network wants to allow conditional LTM to be triggered towards the candidate cells for which early TA acquisition and TCI state activation is completed, the event definition need not include candidate resource set information. It can be implicitly formed on completion of the above steps.

Proposal 7: For CLTM, implicit triggering of condition evaluation for a candidate cell based on Early-TA acquisition and Early-TCI state activation is considered.

To allow the network to start the early TA and early TCI state activation at appropriate time prior to CLTM, the network may configure separate LTM events with event parameters which may trigger events slightly earlier than the events defined as condition for CLTM. Alternatively additional offset for early reporting for the condition can be used to indicate that DU to start the early sync related actions for CLTM.

Proposal 8: CLTM event with additional offset can be used to trigger L1 measurement report prior to condition evaluation.

As discussed above, the support of L3 measurements in CLTM is FFS. On the other hand, Rel. 18 LTM already supports co-existence with CHO feature (implicitly with L3 measurements). Thus, it is expected that the co-existence of L1-based CLTM and L3-based CLTM will be beneficial and should focus on the determination of which conditional execution to select if conditions for multiple candidate cells are fulfilled at the same time.

The decision on whether MAC or RRC will determine the LTM candidate cell for execution have an impact for the co-existence. Firstly, the determination of the LTM candidate cell for CLTM execution and evaluation of CLTM condition should be distinguished. It is clear that, the same entity for which the measurements are configured for, should be responsible for the evaluation.

Proposal 9: For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

## Early synchronization

### RACH-based early TA acquisition

A UE configured with CLTM may perform a cell switch without receiving a cell switch command, when any of the evaluated conditional execution conditions for a candidate cell is met. Therefore, the existing mechanism of RACH-based TA information transfer to the UE in the cell switch command is not applicable to CLTM and a different approach is required, if RACH-less conditional LTM cell switches are to be enabled. The transmission of a random-access response (RAR) to provide TA information to the UE was considered both in Rel-18 and Rel-19 LTM discussions, but was ruled out for non-conditional intra- and inter-CU LTM and left as FFS for conditional LTM:

RAN2#126 agreement: RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

In RAN2#127bis, it was agreed to support PDCCH-ordered early TA acquisition for conditional LTM:

RAN2#127bis agreement: PDCCH ordered early TA acquisition is supported for conditional LTM.

Given that, as explained above, the exact timing of a CLTM cell switch is not controlled by the network, to ensure that a TA value is available when the conditional execution condition is met, the estimated TA value must be provided to the UE soon after the RA preamble transmission via a TA information message/RAR. Nevertheless, a CLTM condition for a cell switch to a candidate cell may be configured but may not be applicable for a conditional cell switch from the current serving cell, e.g., may be applicable for CLTM from the target cell. A clarification is required on whether TA information/RAR should be provided for TAs associated with such candidate cells.

Observation 3: A clarification is needed on whether TA information/RAR should be provided for TAs associated with candidate cells that are not applicable for a conditional cell switch from the current serving cell.

To reduce unnecessary signaling and TA maintenance effort on the UE side, it is preferred that TA information/RAR is provided to the UE only if it is possible that the upcoming cell switch is a CLTM cell switch to the candidate cell associated with the TA, i.e., that a CLTM condition for a cell switch from the current serving cell to the candidate cell with which the TA is associated has been configured and condition evaluation has started/may start following a triggering event. For all other candidate cells, the TA is not provided to the UE before the cell switch command, so that compatibility with Rel. 18 LTM is ensured and the relevant agreement made for Rel. 19 inter-CU LTM is followed.

Proposal 10: Support transmission of RAR to the UE for the TA of a candidate cell to which the UE may perform a CLTM cell switch from the current serving cell. For all other candidate cells, the TA is not provided to the UE before the cell switch command.

During Rel-18 LTM discussions, the topic of RAR transmission was extensively studied, with RAR transmission from either the source or the candidate cell being considered. Although RAR from the candidate cell has a shorter delay compared to RAR from source, where the estimated TA value must propagate from the candidate to the source cell via the F1 interface, its main drawback is that, depending on UE capability and cell frequency, it may lead to increased interruption time at the source cell, since the UE must monitor the PDCCH of the candidate cell and may not be able to receive data from the source while monitoring for RAR.

Observation 4: Transmission of RAR from source cell to the UE results in lower interruption time for early TA acquisition compared to RAR from target cell.

In addition, the RAR from source option is fully compatible with the Rel-18 early UL synchronization procedure and minimal changes are required to support the augmentation with a RAR, in contrast to the RAR from target option, where more modifications to the behavior of the UE, the source cell and the candidate cell are required.

Observation 5: Transmission of RAR from source cell to the UE results in less specification effort compared to RAR from target cell.

Proposal 11: For a UE configured with CLTM for a candidate cell, support transmission of RAR from the source cell.

During the RAN2 last meeting, it was discussed that, for PDCCH ordered early TA in conditional LTM, a new MAC CE can be introduced to deliver the TA value and the corresponding candidate configuration ID to UE before the triggering of conditional LTM execution.Regarding this aspect of RAR container, before concluding that no RAR would be supported in Rel-18 LTM, multiple options were considered ,:

-	Alt1: MAC PDU (like RAR MAC PDU in 4 step RACH)

-	Alt2: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

Alt1 has the smallest specification impact, but Alt2 has the advantage that it does not require the UE to monitor PDCCH in type 1 common search space (CSS). It was also proposed to re-use existing LTM Cell Switch Command MAC CE is FFS. However, re-using LTM CSC MAC CE creates the situation when MAC CE semantics depends on the context which is not a desired design choice. RAN2 should down-select one of the abovementioned alternatives Alt1 and Alt2.

Proposal 12: Down-select one of the following two alternatives for the RAR container and determine the RAR content accordingly:

-    Alt1: MAC PDU (like RAR MAC PDU in 4 step RACH)

-    Alt2: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

In Rel. 18 LTM, the entity triggering the early TA acquisition procedure is the source DU, based on measurement reports from the UE. For coexistence of conditional and non-conditional LTM, the support of the legacy Rel. 18 mechanism for triggering early TA acquisition is needed. In CLTM, depending on the measurement reporting configuration, it may be useful to also enable triggering of the early TA acquisition procedure by the UE.

Observation 6: For coexistence of conditional and non-conditional LTM, the support of the legacy Rel. 18 mechanism for triggering early TA acquisition is needed.

Observation 7: In CLTM, depending on the measurement reporting configuration, it may be useful to also enable triggering of the early TA acquisition procedure by the UE.

To this end, an event can be configured to the UE and, when triggered, the UE may either perform RA preamble transmission or request a PDCCH order from the source DU. The latter option is preferred, as it allows for control of the timing of the use of CFRA resources by the network and, consequently, for CFRA preamble sharing among UEs. During last Rel. 19 meeting, it has been proposed to down-select between UE trigger and L1/L3based NW trigger for RACH-based TA acquisition. From our perspective, both options: legacy and UE-triggered related, shall be allowed. Details of co-existence are FFS.

Proposal 13a: The legacy L1/L3 measurement-based NW trigger for RACH-based TA acquisition is supported.

Proposal 13b: The UE may trigger the RACH-based early TA acquisition procedure by sending a request to the source DU based on an event configured by the network.

After a TA value has been obtained, it must be ensured that at the time of the conditional cell switch, the TA value is valid. The maintenance of the TA can be performed either at the source DU or at the UE. Maintenance at the source DU has the advantage of minimal specification impact. Nevertheless, in some cases, depending on the measurement reporting configuration, e.g., if the UE is not configured to report measurements to the source DU, it may not be possible for the source DU to detect significant change in the UE-candidate cell link which should trigger a refresh of the TA. Moreover, considering that the point of CLTM is for UE to autonomously trigger cell switch, it suffers the drawback that UE would not be aware at switching time whether the TA is valid or not.

Observation 8: If the maintenance of the TA is performed on the network side, when a CLTM cell switch is triggered, the UE will not be able to determine the validity of the available TA.

If the UE triggers cell switch with an expired TA, this would inadvertently lead to failure of the LTM execution procedure.  It would then make sense perform the maintenance of the TA at the UE.

During last Rel. 19 meeting, it has been proposed to down-select how UE shall maintain TA value. It has been proposed either to base on time alignment timer (TAT)-like approach or TA validity verification based on L1 measurements. From our perspective, TAT-like approach shall be chosen as a base to follow legacy method, but it is FFS whether to add L1 measurements-based TA validity verification. Proposed options are not mutually exclusive from our perspective. In fact, an approach similar to CG-SDT can be followed for more reliable TA validation, where the UE is configured to perform both TAT- and measurement-based TA validation.

Proposal 14: When CLTM is configured, the maintenance of the TA value should be performed at the UE and a maintenance method similar to CG-SDT can be followed.

### UE-based TA estimation

An alternative way for the UE to obtain the TA required for RACH-less cell switch is UE-based TA estimation. In Rel-18 LTM, the UE may obtain the UE-based TA estimate after configuration with UE-based TA estimation and no further signaling is required. It is up to the network to provide enough time to the UE to obtain the UE-based TA estimate before issuing a cell switch command. In RAN2#127bis, it was agreed to support UE-based TA measurement also for CLTM:

RAN2#127bis agreement: UE based TA measurement mechanism is supported for conditional intra-CU LTM.

In CLTM, a conditional cell switch may be triggered before the UE has acquired a valid TA estimate, leading to a potentially unnecessary RACH-based cell switch with increased interruption time. Therefore, we should consider whether/how to ensure that the UE has a UE-based TA estimate when a conditional cell switch is triggered.

Observation 9: Cell switch conditions can be met before UE acquires valid TA estimate what can force UE to perform RACH-based cell switch.

Proposal 15: When UE-based TA estimation and CLTM is configured, consider whether/how to ensure that a UE-based TA estimate is available when a conditional cell switch is triggered.

Furthermore, the issue of the first UL transmission of the UE in a RACH-less CLTM switch is even more pronounced, as the network cannot even exploit the RACH-based early UL synchronization procedure to anticipate the arrival of the UE at the candidate cell and the beam the UE is going to use. Hence, the network may be forced to monitor any provided configured grant (CG) on all beams. To mitigate the issue, the UE may inform the network about the acquisition of a UE-based TA estimate.

Observation 10:  Having the network be unaware about cell switch and the beam that UE is going to use after moving to target cell leads to wasting of resources at target cell.

Proposal 16: When UE-based TA estimation and CLTM are configured, consider whether/how the UE informs the network about the status of UE-based TA estimation.

### TCI state activation

In RAN2#127bis, it was agreed that early TCI state activation/deactivation will be supported for CLTM:RAN2#127bis agreement: Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

Regarding the activation/deactivation mechanism, the following options were discussed, with the intention to down-select one of them:

- Option 1: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

- Option 2: UE determines the TCI state to activate based on L1 measurement results without NW involvement. The detailed procedure is up to UE implementation.

In our view, Option 1 should be supported by default, as coexistence of conditional and non-conditional LTM should be inherently supported.

Observation 11: Support of Option1 enables the co-existence between conditional and non-conditional LTM.

Proposal 17: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused

On the other hand, we do not see the need to down-select only one of the options, as we also see merit in UE-autonomous TCI state activation. Since cell switch triggering is left to the UE based on a network-configured event, it may be reasonable to enable UE-based TCI state activation to further reduce the need for measurement reporting. This can be accomplished by configuring an autonomous TCI state activation event to the UE. A potential issue of such a configuration is that the network will not be aware of the autonomously activated TCI states which may be needed for the RACH-less cell switch as explained below. To resolve this issue, the UE may inform the source cell about the autonomously activated TCI states.

Proposal 18: Consider whether/how autonomous TCI state activation is supported for UEs configured with CLTM.

For RACH-less conditional cell switch, the source cell, having knowledge about the network-based and/or autonomously activated TCI states, can inform the candidate cell about the activated TCI states, so that the candidate can activate and monitor the corresponding CGs. If such information is not provided to the candidate cell, it must activate and monitor the CGs immediately after providing them in the RRC configuration.

Proposal 19: For a UE configured with CLTM, the source cell may inform the candidate cells about the activated TCI states.

## Conditional cell switch execution

As discussed above, allowing the UE to trigger a cell switch execution has significant impact on the actions that need to be taken before and after a conditional cell switch. An important consequence is the network’s uncertainty about the timing of the cell switch, which makes the preparation of the network for the UE’s access to the target cell more complicated. A possible way to overcome this challenge and simplify the procedures related to UE’s first access to the target is the transmission of a message from the UE to the source cell, informing the source cell that a conditional cell switch condition has been met. Upon receiving such a notification, the source cell can trigger procedures for UE’s access to the target cell as in Rel. 18 LTM, e.g., send a cell switch notification, or it can even decide whether a cell switch is allowed, in case more network control on the cell switch is desired, and inform the UE about it.

Observation 12: Autonomous UE’s decision on cell switch causes uncertainty at NW side with regards to timing of the cell switch and consequent preparation for UE admission

Nevertheless, such a message may never reach the source cell as the UE-source cell link quality may already be too low soon after the conditional cell switch condition is met. In addition, while the UE will be waiting for a response to the message, the link may further deteriorate potentially resulting in radio link failure. Therefore, we should further study if and how this message should be transmitted. Alternatively, the UE could directly inform the target about its access, e.g. by sending a scheduling request to the target.

Proposal 20: The UE can inform the NW about triggering cell switch. RAN2 should discuss how to enable this.

For RACH-less access to the target cell, apart from the TA, the UE must be provided with an UL grant. In Rel. 18 LTM, both CG and DG were supported as they have their own benefits. In RAN2#127bis, support of CG was agreed:

RAN2#127bis agreement: For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

Providing a CG may incur a large resource reservation overhead, especially when multiple CGs for multiple UEs are provided by the target cell, with the resources being reserved from RRC configuration to cell switch execution. Nevertheless, for non-conditional LTM, thanks to the network triggering of the cell switch and the transmission of cell switch notification, optimizations of the resource management on the network side are possible, as well as monitoring of the CG only after a cell switch has been decided. The lack of network awareness of when the cell switch in CLTM is triggered requires for alternative solutions to avoid monitoring of the CGs starting from RRC configuration and to enable more flexible resource management on the network side. To this end, as the RAR transmission may constitute the last signaling exchange between the UE and the source cell before a potential conditional cell switch, the RAR transmission and its preceding steps (RA preamble transmission, TA information transfer) may be used for the activation of a CG. Alternatively, a message from the UE to the NW can be used for the activation of a CG. Such a message would also facilitate the support of DG. Utilizing a DG mitigates issues related to increased resource reservation which were discussed earlier for CG.

Observation 13: Lack of any cell switch indication to the NW leads to inflexibilities in resource management at NW side.

Proposal 21: RAN2 to discuss CG activation based on RA-driven early TA acquisition procedure and/or message from the UE to the NW informing about cell switch triggering.

Proposal 22: DG-based admission is supported. RAN2 to discuss DG triggers.

A further aspect that must be considered regarding UE-triggered cell switch execution is that the conditional execution condition may be met while the early TA acquisition procedure is in progress, e.g., while the UE is waiting to receive the RAR. In such a case, it may be beneficial to allow for completion of the early TA acquisition before the cell switch is executed, so that a RACH-less cell switch can be performed and an additional unnecessary random access procedure is avoided. On the other hand, like the above considerations for the notification message from the UE to the source, while the UE is waiting for the RAR, the UE-source cell link may deteriorate, the RAR may never be received by the UE and the UE may experience a radio link failure, leading eventually to a larger interruption time compared to that of a RACH-based cell switch. Therefore, the details about if and how long the UE should wait for a RAR require further consideration.

Observation 14: Allowing for slight delay in cell switch while waiting for RAR is beneficial as it can increase the chances for RACH-less admission to target cell.

Proposal 23: If the conditional cell switch execution condition is met while early TA acquisition is in progress, allow for completion of the early TA acquisition procedure before triggering the cell switch.

Furthermore, it is likely that conditional cell switch execution conditions for multiple cells are met. The actions that the UE has performed to prepare a cell switch to a candidate cell prior to the triggering of the condition may be different for each of the triggered candidates, e.g., the UE may have performed DL and UL synchronization with one of the triggered candidates, but only DL synchronization with another. To reduce cell switch interruption time, it is reasonable that the UE prioritizes execution to the former.

Observation 15: Conditional cell switch conditions for multiple cells can be met simultaneously

Proposal 24: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which it has performed cell switch preparation actions resulting in reduced interruption time.

Looking from the perspective of L1-based and L3-based CLTM co-existence, a single entity responsible for determination can handle two conditions being triggered at the same time: for L1-based CLTM and L3-based CLTM. To re-use the Rel. 18 procedure as much as possible, the MAC can be the deciding entity for the determination of conditional LTM execution.

Proposal 25: For the entity responsible for the determination of conditional LTM execution down-select between the below options

- Option 1: MAC

- Option 2: RRC

- Option 3: MAC for L1-based conditional LTM and RRC for L3-based conditional LTM.

# 3	Conclusion

This document has made the following observations:

Observation 1: In Rel. 16 CHO, the reporting configuration was enhanced to include the conditional execution condition, where the conditional execution condition is indicated as a reporting type, and the execution condition and related parameters are indicated.

Observation 2: NW triggering of condition evaluation for CLTM allows for increased robustness of the conditional cell switch execution without compromising on interruption time during cell switch.

Observation 3: A clarification is needed on whether TA information/RAR should be provided for TAs associated with candidate cells that are not applicable for a conditional cell switch from the current serving cell.

Observation 4: Transmission of RAR from source cell to the UE results in lower interruption time for early TA acquisition compared to RAR from target cell.

Observation 5: Transmission of RAR from source cell to the UE results in less specification effort compared to RAR from target cell.

Observation 6: For coexistence of conditional and non-conditional LTM, the support of the legacy Rel. 18 mechanism for triggering early TA acquisition is needed.

Observation 7: In CLTM, depending on the measurement reporting configuration, it may be useful to also enable triggering of the early TA acquisition procedure by the UE.

Observation 8: If the maintenance of the TA is performed on the network side, when a CLTM cell switch is triggered, the UE will not be able to determine the validity of the available TA.

Observation 9: Cell switch conditions can be met before UE acquires valid TA estimate what can force UE to perform RACH-based cell switch.

Observation 10:  Having the network be unaware about cell switch and the beam that UE is going to use after moving to target cell leads to wasting of resources at target cell.

Observation 11: Support of Option1 enables the co-existence between conditional and non-conditional LTM.

Observation 12: Autonomous UE’s decision on cell switch causes uncertainty at NW side with regards to timing of the cell switch and consequent preparation for UE admission

Observation 13: Lack of any cell switch indication to the NW leads to inflexibilities in resource management at NW side.

Observation 14: Allowing for slight delay in cell switch while waiting for RAR is beneficial as it can increase the chances for RACH-less admission to target cell.

Observation 15: Conditional cell switch conditions for multiple cells can be met simultaneously

And proposed the following:

Proposal 1: For L1 execution conditions, LTM Candidate configuration is linked to Report-config-ID that is configured with conditional events.

Proposal 2: For L1 execution conditions, the DUs configure the conditional events in LTM reporting configuration.

Proposal 3: For L1 execution conditions, the CU can link the reporting configuration that indicates the execution condition to LTM candidate configuration.

Proposal 4: Support for L3 based conditions for CLTM.

Proposal 5: RAN2 to down-select between SCPAC like configuration OR extension of report configuration to include CLTM parameters for subsequent LTM.

Proposal 6: Network triggering of condition evaluation is supported.

Proposal 7: For CLTM, implicit triggering of condition evaluation for a candidate cell based on Early-TA acquisition and Early-TCI state activation is considered.

Proposal 8: CLTM event with additional offset can be used to trigger L1 measurement report prior to condition evaluation.

Proposal 9: For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Proposal 10: Support transmission of RAR to the UE for the TA of a candidate cell to which the UE may perform a CLTM cell switch from the current serving cell. For all other candidate cells, the TA is not provided to the UE before the cell switch command.

Proposal 11: For a UE configured with CLTM for a candidate cell, support transmission of RAR from the source cell.

Proposal 12: Down-select one of the following two alternatives for the RAR container and determine the RAR content accordingly:

-    Alt1: MAC PDU (like RAR MAC PDU in 4 step RACH)

-    Alt2: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

Proposal 13a: The legacy L1/L3 measurement-based NW trigger for RACH-based TA acquisition is supported.

Proposal 13b: The UE may trigger the RACH-based early TA acquisition procedure by sending a request to the source DU based on an event configured by the network.

Proposal 14: When CLTM is configured, the maintenance of the TA value should be performed at the UE and a maintenance method similar to CG-SDT can be followed.

Proposal 15: When UE-based TA estimation and CLTM is configured, consider whether/how to ensure that a UE-based TA estimate is available when a conditional cell switch is triggered.

Proposal 16: When UE-based TA estimation and CLTM are configured, consider whether/how the UE informs the network about the status of UE-based TA estimation.

Proposal 17: R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused

Proposal 18: Consider whether/how autonomous TCI state activation is supported for UEs configured with CLTM.

Proposal 19: For a UE configured with CLTM, the source cell may inform the candidate cells about the activated TCI states.

Proposal 20: The UE can inform the NW about triggering cell switch. RAN2 should discuss how to enable this.

Proposal 21: RAN2 to discuss CG activation based on RA-driven early TA acquisition procedure and/or message from the UE to the NW informing about cell switch triggering.

Proposal 22: DG-based admission is supported. RAN2 to discuss DG triggers.

Proposal 23: If the conditional cell switch execution condition is met while early TA acquisition is in progress, allow for completion of the early TA acquisition procedure before triggering the cell switch.

Proposal 24: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which it has performed cell switch preparation actions resulting in reduced interruption time.

Proposal 25: For the entity responsible for the determination of conditional LTM execution down-select between the below options

- Option 1: MAC

- Option 2: RRC

- Option 3: MAC for L1-based conditional LTM and RRC for L3-based conditional LTM.