3GPP TSG-RAN2 Meeting #128	R2-2410799

Orlando, USA, Nov.18th –22nd, 2024

Source: 			ZTE Corporation

Title: 	Evaluation on RRM measurement prediction

Agenda item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

In the RAN2 126 meeting, the simulation scenario and the corresponding prioritization for RRM measurement prediction are defined [1], as follows:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this document, we’d like to provide our simulation results and the corresponding analysis for the case with high priority.

- Discussion

## 2.1 General

In the last meeting, we have agreed that companies are free to consider non-AI and simple AI. In this meeting, we provide our simulation results with non-AI approach for temporal domain prediction to study how much the gain that AI can provide. The general idea of non-AI approach is ‘sample and hold’, but details (e.g. which result to be held) are different with different RRM sub use cases. Taking Figure 1 as an example:

Figure 1: An example for temporal domain prediction

Regarding non-AI approach for RRM sub use case 1 (L1 beam result -> L1 beam result), the L1 measurement results within prediction window reuses the L1 measurement result from last actual measurement, and then L3 filtering is performed as legacy. For Figure 1, the predicted result with non-AI approach at T3 is calculated by the formula: (1-L3 filtering parameter) * L3 filtered results at T2 + L3 filtering parameter * L1 filtered results at T2.

Regarding non-AI approach for RRM sub use case 2 (L3 filtered cell result  L3 filtered cell result), the L3 filtered measurement results within prediction window reuses the L3 filtered measurement results from last actual measurement. For Figure 1, the predicted results with non-AI approach at T3 = L3 filtered results at T2 = (1-filtering parameter) *L3 filtered result at T1 + L3 filtering parameter * L1 filtered results at T2. So, in our simulation, the simulation results with non-AI approach are different with different RRM sub use cases in the same condition.

## 2.2 FR1 to FR1 intra-freq temporal domain case B

FR1 to FR1 intra frequency temporal domain case B is done by predicting sub set measurement instances in temporal domain of the same cell. In the RAN2 #127 meeting, companies reached a consensus that for temporal domain prediction case B, example 2 are recommended to be baseline. In the example 2, the measurement results from actual measurement within observation window is not continuous, as shown in Figure 2:

Figure 2: Temporal domain case B example 2

As mentioned in our previous contribution (R2-2409207), for temporal domain prediction case B, the following filtering scheme can be considered:

- Filtering option 1: L3 filtering is based on the L1 filtered result and the last L3 filtered cell result;

- Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction;

- Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

Compared with filtering option 1, filtering option 2 and filtering option 3 are not impacted by prediction errors in the previous predictions, since all results are from actual measurements when performing L3 filtering. Besides, with filtering option 2 and 3, the prediction does not need to be performed in order, which reduces required computation resources and simulation time. So, in our simulation, option 2 and option 3 are adopted. Specifically, in the procedure of modelling training, the cell results used as model input takes filtering option 2 and option 3 above, while the cell result used as model output uses the ground true result (i.e. no measurement is reduced); and in the procedure of modelling inference, the cell result used as model input also uses filtering option 2 and filtering option 3, and the predicted result is compared with ground true value to calculate average RSRP difference.

The tables below show some important simulation assumptions in our simulation and the corresponding simulation results. The details please see our simulation results table in the FTP.

Table 2.2.1: Simulation temple table for FR1 to FR1 temporal domain measurement prediction case B

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | Whether LOSsoft is modeled or not | Not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.2.2 |

Table 2.2.2: Simulation results for temporal domain prediction case B

| MRRT | [observation window, prediction window] and detailed skipping pattern | RRM sub use case |  | ARSRP difference with UE speed = 30km/h | ARSRP difference with UE speed = 90km/h |
| --- | --- | --- | --- | --- | --- |
| 50% | [600ms, 200ms] skipping pattern: 1:1 | RRM sub use case 1 | AI | 0.502 | 0.693 |
|  |  |  | Non-AI | 0.538 | 0.722 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.522 | 0.673 |
|  |  |  | Non-AI | 0.636 | 0.775 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.521 | 0.701 |
|  |  |  | Non-AI | 0.542 | 0.904 |
|  | [1000ms,200ms] skipping pattern: 1:1 | RRM sub use case 1 | AI | 0.498 | 0.679 |
|  |  |  | Non-AI | 0.538 | 0.722 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.508 | 0.668 |
|  |  |  | Non-AI | 0.636 | 0.775 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.508 | 0.669 |
|  |  |  | Non-AI | 0.542 | 0.904 |
|  | [400ms,400ms] skipping pattern: 2:2 | RRM sub use case 1 | AI | 0.452, 0.592 | 0.618, 0.930 |
|  |  |  | Non-AI | 0.513, 0.667 | 0.668, 1.015 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.469, 0.611 | 0.607, 0.912 |
|  |  |  | Non-AI | 0.474, 0.615 | 0.630, 0.959 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.482, 0.644 | 0.658, 0.974 |
|  |  |  | Non-AI | 0.486, 0.648 | 0.776, 1.151 |
|  | [1200ms,400ms] skipping pattern: 2:2 | RRM sub use case 1 | AI | 0.449, 0.579 | 0.595, 0.895 |
|  |  |  | Non-AI | 0.513, 0.667 | 0.668 1.015 |
|  |  | RRM sub use case 2(filtering option 2) | AI | 0.453, 0.609 | 0.599, 0.891 |
|  |  |  | Non-AI | 0.474, 0.615 | 0.630, 0.959 |
|  |  | RRM sub use case 2(filtering option 3) | AI | 0.452, 0.598 | 0.598, 0.877 |
|  |  |  | Non-AI | 0.486, 0.648 | 0.776, 1.151 |
| 60% | [400ms,600ms] skipping pattern: 2:3 | RRM sub use case1 | AI | 0.465, 0.612, 0.722 | 0.625, 0.917, 1.203 |
|  |  |  | Non-AI | 0.523, 0.682, 0.801 | 0.681, 1.015, 1.354 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.474, 0.623 0.742 | 0.601, 0.896, 1.195 |
|  |  |  | Non-AI | 0.480, 0.627, 0.747 | 0.626, 0.945, 1.274 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.499, 0.663, 0.788 | 0.666, 0.960, 1.234 |
|  |  |  | Non-AI | 0.511, 0.675, 0.808 | 0.839, 1.192, 1.543 |
|  | [1400ms,600ms] Skipping pattern: 2:3 | RRM sub use case 1 | AI | 0.463, 0.589, 0.695 | 0.598, 0.844, 1.075 |
|  |  |  | Non-AI | 0.523, 0.682, 0.801 | 0.681, 1.015, 1.354 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.461, 0.613, 0.739 | 0.592, 0.879, 1.141 |
|  |  |  | Non-AI | 0.480, 0.627, 0.747 | 0.626, 0.945, 1.274 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.458, 0.603 0.712 | 0.590, 0.887, 1.125 |
|  |  |  | Non-AI | 0.511, 0.675, 0.808 | 0.839, 1.192, 1.543 |
| 66.7% | [800ms, 400ms] Skipping pattern: 1:2 | RRM sub use case 1 | AI | 0.544, 0.674 | 0.779, 1.050 |
|  |  |  | Non-AI | 0.557, 0.711 | 0.801, 1.141 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.544, 0.692 | 0.700, 0.987 |
|  |  |  | Non-AI | 0.627, 0.753 | 0.767, 1.008 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.553, 0.709 | 0.725, 0.994 |
|  |  |  | Non-AI | 0.627, 0.780 | 1.131, 1.478 |
|  | [400ms, 800ms] skipping pattern: 2:4 | RRM sub use case1 | AI | 0.464, 0.599, 0.708, 0.812 | 0.650, 0.953 1.231, 1.517 |
|  |  |  | Non-AI | 0.510, 0.657, 0.775, 0.894 | 0.710, 1.046, 1.366, 1.693 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.470, 0.613 0.727, 0.840 | 0.598, 0.908 1.208, 1.493 |
|  |  |  | Non-AI | 0.476, 0.622, 0.739, 0.848 | 0.618, 0.948, 1.269, 1.600 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.497, 0.648 0.771, 0.890 | 0.691, 0.983 1.256, 1.516 |
|  |  |  | Non-AI | 0.514, 0.671, 0.795, 0.921 | 0.894, 1.246, 1.576, 1.914 |
|  | [1600ms,800ms] skipping pattern: 2:4 | RRM sub use case 1 | AI | 0.458, 0.583, 0.677, 0.772 | 0.616, 0.871, 1.105, 1.356 |
|  |  |  | Non-AI | 0.511, 0.658, 0.776, 0.895 | 0.711, 1.047, 1.368, 1.694 |
|  |  | RRM sub use case 2 (filtering option 2) | AI | 0.471, 0.619, 0.733, 0.835 | 0.593, 0.887, 1.156, 1.407 |
|  |  |  | Non-AI | 0.476, 0.622, 0.739, 0.848 | 0.618, 0.948, 1.269, 1.600 |
|  |  | RRM sub use case 2 (filtering option 3) | AI | 0.459, 0.604, 0.719, 0.826 | 0.592, 0.878, 1.130, 1.377 |
|  |  |  | Non-AI | 0.516, 0.673, 0.797, 0.923 | 0.898, 1.251, 1.580, 1.918 |

- The impact of skipping pattern on the prediction accuracy

RAN2 has agreed that companies need to report MRRT, observation window length and prediction window length when providing their simulation results, and detailed skipping pattern are optional to report. In this contribution, we study the impact of the skipping pattern on the prediction accuracy with the same MRRT. Based on the simulation results, we find that with even with the same MRRT, different skipping pattern results in different prediction accuracy. For instance, we simulate 3 skipping patterns with 66.7% MRRT, including [800ms, 400ms], [400ms, 800ms], and [1600ms, 800ms] cases. And their average RSRP differences with RRM sub use case 1 and 90km/h UE speed are 1.050 dB, 1.517dB and 1.356dB, respectively. One possible reason is that different skipping pattern have different prediction window length and observation window length, which impacts prediction accuracy. So, we suggest companies to report the adopted skipping pattern (the measurement in which time instance is skipped, and the measurement in which time instance is actual measured) when providing their simulation results.

Observation 1: Regarding temporal domain prediction case B, with the same MRRT, different skipping pattern can provide different prediction performance.

Proposal 1: For temporal domain prediction case B, suggest companies to report the adopted skipping pattern when providing simulation results

- The impact of filtering

As mentioned above, filtering option 2 and option 3 are adopted in our simulation. And the simulation results show that the prediction with two filtering options provide similar prediction accuracy, the maximum RSRP difference between the average prediction error with two filtering options is less than 0.062dB in the same condition.

Observation 2: Regarding temporal domain prediction case B, filtering option 2 and filtering option 3 provide similar prediction performance, where

- Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction;

- Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

- AI based approach vs Non-AI based approach

Our simulation results show that in the case that prediction window is short, there is almost no prediction accuracy gain compared with non-AI approach. For example, if the prediction window is 200ms (only one predicted value within prediction window), the best gain that AI can provide is only 0.203dB. And the prediction performance gain is increased as prediction window length increases. If the prediction window is 800ms, the best gain that AI can provide is 0.514dB. This is because with long prediction window, the change of UE location and channel condition are more obvious, so sample and hold cannot work well.

Observation 3: Regarding temporal domain prediction case B, with pretty short prediction window, there is almost no prediction accuracy gain compared with non-AI approach. While as prediction window length increases, the gain that AI can provide increases.

In addition, we also observe that in high speed scenario, the AI approach can provide more gain compared with non-AI. For example, with UE speed = 30km/h, the best gain that AI can provide is 0.123dB; while with UE speed = 90km/h, the best gain that AI can provide is0.514dB. The reason is: the channel condition in high speed scenario changes more rapidly, so the sample and hold cannot work well.

Observation 4: Regarding temporal domain prediction case B, the gain that AI can provide is more obvious in high speed scenario.

Proposal 2: Capture the observation 1~4 for temporal domain prediction case B in the TR.

## 2.3 FR2 to FR2 intra-freq temporal domain case A

For intra-frequency temporal domain case A, continuous historical measurement results are used to predict continuous future measurement results, and no measurement is reduced. The table below show some important simulation assumption in our simulation and the corresponding simulation results. The details please see our simulation results table in the FTP.

Table 2.3.1: Simulation temple table for FR2 to FR2 temporal domain measurement prediction case A

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | Whether LOSsoft is modeled or not | Not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.3.2 |

Table 2.3.2: simulation results for temporal domain case A

| RRM sub use case | Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |  |
| --- | --- | --- | --- | --- |
| RRM sub use case 1 | 60 | [800ms, 1200ms] | AI | 0.795, 1.361, 2.042 |
|  |  |  | Non-AI | 0.799, 1.869, 2.959 |
|  |  | [800ms, 800ms] | AI | 0.810, 1.391 |
|  |  |  | Non-AI | 0.829, 1.955 |
|  |  | [800ms, 400ms] | AI | 0.758 |
|  |  |  | Non-AI | 0.831 |
|  |  | [1200ms,400ms] | AI | 0.739 |
|  |  |  | Non-AI | 0.831 |
|  |  | [1600ms,400ms] | AI | 0.746 |
|  |  |  | Non-AI | 0.831 |
|  | 120 | [800ms, 1200ms] | AI | 1.109, 1.987, 2.920 |
|  |  |  | Non-AI | 1.213, 2.909, 4.546 |
|  |  | [800ms, 800ms] | AI | 1.087, 1.945 |
|  |  |  | Non-AI | 1.203, 2.932 |
|  |  | [800ms, 400ms] | AI | 0.992 |
|  |  |  | Non-AI | 1.206 |
|  |  | [1200ms,400ms] | AI | 0.961 |
|  |  |  | Non-AI | 1.206 |
|  |  | [1600ms,400ms] | AI | 0.963 |
|  |  |  | Non-AI | 1.206 |
| RRM sub use case 2 | 60 | [800ms, 1200ms] | AI | 0.665, 1.394, 2.178 |
|  |  |  | Non-AI | 1.138, 2.213, 3.265 |
|  |  | [800ms, 800ms] | AI | 0.687, 1.400 |
|  |  |  | Non-AI | 1.208, 2.340 |
|  |  | [800ms, 400ms] | AI | 0.680 |
|  |  |  | Non-AI | 1.248 |
|  |  | [1200ms,400ms] | AI | 0.672 |
|  |  |  | Non-AI | 1.248 |
|  |  | [1600ms,400ms] | AI | 0.666 |
|  |  |  | Non-AI | 1.248 |
|  | 120 | [800ms, 1200ms] | AI | 0.936, 2.059, 3.193 |
|  |  |  | Non-AI | 1.773, 3.453, 5.072 |
|  |  | [800ms, 800ms] | AI | 0.941, 2.015 |
|  |  |  | Non-AI | 1.844, 3.574 |
|  |  | [800ms, 400ms] | AI | 0.946 |
|  |  |  | Non-AI | 1.886 |
|  |  | [1200ms,400ms] | AI | 0.916 |
|  |  |  | Non-AI | 1.886 |
|  |  | [1600ms,400ms] | AI | 0.914 |
|  |  |  | Non-AI | 1.886 |

- AI based approach vs Non-AI based approach

Based on our simulation results, we can observe that in the UE speed = 60km/h, the best gain that AI can provide is 1.087dB; while in the UE speed = 120km/h, the best gain is 1.879dB. So, similar as in temporal domain case B, the AI approach can provide more gain in high speed scenario. Besides, the gain that AI model can provide increases with the increase of prediction window length. For example, if prediction window length is 400ms, the best gain that AI can provide is only 0.972dB, while if the prediction window is 1200ms, the best gain can achieve 1.879dB.

Observation 5: Regarding temporal domain prediction case A, the gain that AI can provide is more significant in high speed scenario.

Observation 6: Regarding the temporal domain prediction case A, the gain that AI can provide increases with the increase of prediction window length.

Proposal 3: Capture the observation 5~6 for temporal domain prediction case A in the TR.

## 2.4 FR1 to FR1 inter-frequency (frequency domain)

Frequency domain measurement prediction is to predict the measurement results of one frequency based on the measurement results of another frequency. The tables below show some important simulation assumption in our simulation and the corresponding simulation results. The details please see our simulation results table in the FTP. Regarding to the metric for prediction accuracy, besides the agreed average L3 cell level RSRP difference, we also provide the CDF curve of RSRP difference, which is more intuitive and can provide more information, e.g. the worst prediction performance.

Table 2.4.1: Simulation template table for frequency domain measurement prediction

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in [4]) | Option 2 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes |
|  | Whether LOSsoft is modeled or not | Not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.4.2 |

Table 2.4.2: Simulation results for frequency domain prediction

|  | Inter-frequency shadow fading correction | ARSRP difference with UE speed = 30km/h | ARSRP difference with UE speed = 90km/h |
| --- | --- | --- | --- |
| 2G4G | full | 0.99 | 0.99 |
|  | no | 1.38 | 1.4 |
| 4G2G | full | 1 | 1 |
|  | no | 1.39 | 1.43 |

| - CDF for RSRP difference in the case of 30km/h and 2G4G prediction | - CDF for RSRP difference in the case of 90km/h and 2G4G prediction |
| --- | --- |
| (c) CDF for RSRP difference in the case of 30km/h and 4G2G prediction | (d) CDF for RSRP difference in the case of 90km/h and 4G2G prediction |

Figure 3: CDF curve for RSRP difference in the frequency domain prediction

Comparing the simulation results with full correlation and no correlation inter-freq shadowing fading, we find that no matter average RSRP difference value or RSRP difference CDF curve, the prediction accuracy of frequency domain prediction with full correlation inter-freq shadowing fading is better than that with no correlation model. This is because the uncertainty of shadowing fading brings additional difficulty for prediction. But even with no correlation inter-freq shadowing model, the maximum average RSRP difference is only 1.43dB, which is acceptable.

Observation 7: Regarding frequency domain prediction, the prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation shadowing fading model.

Observation 8: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.43dB, which is acceptable.

Proposal 4: Capture the observation 7 and 8 for frequency domain prediction in the TR.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

- FR1 to FR1 intra-freq temporal domain case B

Observation 1: Regarding temporal domain prediction case B, with the same MRRT, different skipping pattern can provide different prediction performance.

Observation 2: Regarding temporal domain prediction case B, filtering option 2 and filtering option 3 provide similar prediction performance, where

- Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction;

- Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

Observation 3: Regarding temporal domain prediction case B, with pretty short prediction window, there is almost no prediction accuracy gain compared with non-AI approach. While as prediction window length increases, the gain that AI can provide also increases.

Observation 4: Regarding temporal domain prediction case B, the gain that AI provides is more obvious in high speed scenario.

- FR2 to FR2 intra-freq temporal domain case A

Observation 5: Regarding temporal domain prediction case A, the gain that AI can provide is more significant in high speed scenario.

Observation 6: Regarding the temporal domain prediction case A, the gain that AI can provide increases with the increase of prediction window length.

- FR1 to FR1 inter-frequency (frequency domain)

Observation 7: Regarding frequency domain prediction, the prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation shadowing fading model.

Observation 8: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.43dB, which is acceptable.

Proposal 1: For temporal domain prediction case B, suggest companies to report the adopted skipping pattern when providing simulation results.

Proposal 2: Capture the observation 1~4 for temporal domain prediction case B in the TR.

Proposal 3: Capture the observation 5~6 for temporal domain prediction case A in the TR.

Proposal 4: Capture the observation 7 and 8 for frequency domain prediction in the TR.

- Reference

- Report of RAN2 126 meeting.