3GPP TSG-RAN WG2 Meeting #128	R2-2410804

Orlando, USA, Nov 18th – 22nd, 2024		Revision of R2-2408970

Agenda item:	8.8.2

Source: 	Sequans Communications

Title: 	Downlink coverage enhancement SMTC impacts

Document for:	Discussion and Decision

# Introduction

During the previous meetings, the following proposal were discussed:

R2-2407532	Downlink coverage enhancement SMTC impacts	Sequans Communications	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: SSBs needs to be time multiplexed across the satellite footprint over up to 80ms or 320ms range

Observation 2: Existing SMTCs framework is limited to up to 4 SMTCs per frequency layer

Proposal 1: RAN2 to study SMTC impact of SSB time multiplexing across the satellite footprint

RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

R2-2408970	Downlink coverage enhancement SMTC impacts	Sequans Communications	discussion	Rel-19	NR_NTN_Ph3-Core	R2-2407532

[…]

Proposal 2: Consider extending the NTN SMTC list / defining a new list to signal at least all neighbor SMTCs

In this contribution, we discuss further this topic.

# Discussion

## Increased number of configured SMTCs

So far, discussion did not progress as most companies are waiting for RAN1 inputs. We believe this is also the case for other topics (e.g. barring), so it should not prevent RAN2 to make progress.

A reasonable assumption could be that due to beam-hopping / larger SSB periodicity, it is expected that the number of configured SMTCs would increase. This is just because in addition to existing reasons to have several SMTCs configured (e.g. different satellites), the beam hopping will require to use different SSB offsets where legacy operation would just have used one. More details are included in the Annex, but the exact figures are not important and could indeed wait RAN1/4 conclusions.

Proposal 1: RAN2 assumes that the number of configured SMTCs may increase due to beam-hopping / larger SSB periodicity (depending on RAN1/4 solution)

We assume the SMTC framework would be used by simplicity/reuse, however similar effect could be achieved by other signalling ways (e.g. a DTX patterns configured for neighbour cells).

Based on this assumption, RAN2 could work on possible impacts.

## Power consumption impact / RRM impact

Increasing the number of configured SMTCs does not come for free. It would have an important impact on IDLE UE power consumption. The NR baseline is to have 1 SMTC per frequency layer so that the UE can minimize wakeups, while the NW ensure cells synchronicity on each layer. This is a key baseline feature which accommodates for the spacing of SS compared to LTE.

Hence, related power consumption impact should be considered carefully.

Proposal 2: RAN2 investigates related additional power consumption impact (including legacy UEs)

We believe RAN2 may consider separately legacy UEs/UEs not supporting DL CE, and DL CE UEs.

Legacy UEs / UE not supporting DL CE

This case is considered in our companion contribution. Basically, we consider that adding DL CE SMTCs within legacy SMTC signaling would have a detrimental effect on legacy UEs and should be avoided. This can be addressed by using R19 signaling instead – we copy paste the corresponding proposal:

Proposal 3: RAN2 considers signaling DL CE cells SMTCs separately (e.g. new SMTC5 list)

DL CE UEs

Rel-19 DL CE UEs will have to operate in whole new scenarios compared to legacy UEs. We believe power consumption will be impacted by the increased number of SMTCs required for operating those new scenarios. However, there is room for improvement. Our main proposals regarding DL CE UEs are detailed below.

## Enhanced SMTCs selection

Whether 4 SMTCs are enough, or 6 SMTCs are required, the beam hopping scenario will introduce additional complexity to the UE.

In legacy scenarios without beam hopping, only 1 SMTC is required in general. The 3 additional SMTCs are only required to accommodate different satellites broadcasting cells not synchronized with the serving cell (e.g., from different gNBs). In a scenario with one gNB broadcasting cells (even through different satellites), one could expect those cells to be synchronized and 1 SMTC is enough. But with beam hopping scenarios, multiple SMTCs will become the norm.

In our view, some additional changes would be useful to help the UE to select the SMTCs on which it has to perform measurements (differently said, to choose which SMTC to consider for measurements). Typically, the UE should only consider SMTCs corresponding to the N closest neighbor cells, depending on its capabilities (N=1 in general, exceptionally 2).

Observation 1: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Idle/Inactive mode

So far, the reference location (or moving reference location in case of EMC) of the serving cell is broadcasted. This allows for instance location-based measurement triggering.

In addition, the reference location (or moving reference location in case of EMC) of neighbor cells could be broadcasted. This would enable the UE to know which SMTC(s) to select/measure. The SMTCs/ Reference location would be associated by e.g. the PCI of the neigbor cell.

In case SSB beams are used, an alternative would be to use the serving SSB beam as a rough indication of the UE location within the cell. The SMTC signaling could then be enhanced to associate neighbor SMTCs with specific SSB index(es). In the example below, where each cell is realized with 4 SSB beams, the UE would need to select only 2 SMTCs instead of 6. While less precise, this alternative avoids some overhead as well as location acquisition in IDLE.

Figure 3 - Location-based UE SMTC selection using SSB beams

Proposal 4: Consider enhancements to allow location-based UE SMTC selection (broadcast of neighbor cells reference location / SSB indexes associated with SMTCs)

Connected mode

In connected, it is up to the NW to configure the SMTCs for measurements by the UE.

For QEF cells, without considering UE mobility, no specific impact is expected (this is not a scenario with mobility to neighbor cells).

For EMC, or QEF/EF with UE mobility to neighbor cells, ideally the NW would need the UE location to configure only the needed SMTCs. In general, the NW may not have a precise UE location available. Coarse location can be requested but at the expense of signaling overhead. Moreover, a moving UE would also need to update its location, which would then require an update of the SMTCs configured by the NW.

Observation 2: In connected, NW may not have UE location information needed to configure SMTCs to UE

One solution would be to use R18 location-based CHO, without associated measurement. In such case, no SMTCs need to be configured for measurements before HO execution. The SMTC associated to each HO candidate can be configured as usual as part of each target cell candidate handover command.

Using location-based CHO without associated measurement may still have some drawbacks. It would increase the handover interruption time. Moreover, it is an optional feature, and it comes with additional complexity.

Observation 3: R18 location-based CHO without associated measurement avoids SMTCs configuration, but adds complexity and may increase handover interruption time

In our view, it could also be beneficial to allow the location-based UE SMTC selection in connected. With this scheme, the NW would configure SMTCs without a priori knowledge of UE position (or with limited knowledge), while the UE would only select/measure the needed SMTCs based on its position and neighbor cells (moving) reference (similarly as in IDLE/INACTIVE).

At least for intra-satellite intra-frequency mobility (such as the mobility inside the satellite footprint), there should be no drawbacks with such solution.

Proposal 5: Consider location-based UE SMTC selection also in connected

# Conclusion

In this contribution, we make the following observations and proposals:

Proposal 1: RAN2 assumes that the number of configured SMTCs may increase due to beam-hopping / larger SSB periodicity (depending on RAN1/4 solution)

Proposal 2: RAN2 investigates related additional power consumption impact (including legacy UEs)

Proposal 3: RAN2 considers signaling DL CE cells SMTCs separately (e.g. new SMTC5 list)

Observation 1: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Proposal 4: Consider enhancements to allow location-based UE SMTC selection (broadcast of neighbor cells reference location / SSB indexes associated with SMTCs)

Observation 2: In connected, NW may not have UE location information needed to configure SMTCs to UE

Observation 3: R18 location-based CHO without associated measurement avoids SMTCs configuration, but adds complexity and may increase handover interruption time

Proposal 5: Consider location-based UE SMTC selection also in connected

# References

- “System-Level Evaluation of Beam Hopping in NR-Based LEO Satellite Communication System”, Huawei

# Annex – Required number of SMTCs

The possibility of SSB periodicity extension was added to the WID and is currently discussed by RAN1. Previous relevant RAN1 observations are as follows:

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

The periodicity increase is required to multiplex in time the transmissions of SSBs across the satellite footprint. Taking Set 1-2 as an example, the periodicity 320ms enables 64 different SSB bursts / half-frames to be illuminated successively, assuming beam hopping every 5ms (dwelling time of 5ms). Those 64 hops yield 64*16=1024 illuminated beam footprints and the coverage ratio of 1024/1058=96.8%.

An extension to 640ms is also discussed, that would allow reaching 100% coverage ratio and ~10ms dwelling time, while the number of required SSB burst offsets could still be the same as the number of beam hops.

During last meeting, some companies argued that the existing maximum number of SMTCs (4) was enough. In our understanding, this depends on the scenario and on the beam hopping pattern.

We continue with the above case as an example (set 1-2). It is expected that 4 NR beams per cell would be used to leverage SSB beams and reduce L3 mobility, which means that the 16 beam footprints would be realised with 4 cells * 4 SSB beams. In the figures below, colours are associated to SSB burst / SMTC offsets (with a 5ms granularity). Cells with same colour are illuminated simultaneously. “S” indicates the serving cell considered.

Aggregated beams

Minimizing the number of SMTCs can be done by aggregating the beam footprints illuminated at each hop, i.e. illuminating groups of 4 adjacent cells simultaneously as follows:

Figure 1 - Aggregated beam hopping pattern

In this example, 4 neighbour SMTCs are required. We already reach the maximum that is possible to signal, without any margin to accommodate other scenarios.

Observation 1: Existing limitation of SMTCs forces to use an aggregated beam pattern

Scattered beams

However, one advantage of satellite beam hopping is also to reduce/remove inter-beam interferences by not illuminating adjacent beams simultaneously (considering frequency reuse factor of 1). This was analysed for NR for instance in ‎[1]: “significant performance gains can be achieved by the proposed beam hopping scheme, especially under the distance limit constraint that avoids scheduling adjacent beams simultaneously”.

Observation 2: Scattered beam hopping pattern is beneficial to avoid inter-beam interferences

If we try to (minimally) scatter the 4 cells group from above, we obtain the following pattern:

Figure 2 - Scattered beam hopping pattern

In this example, 5 neighbour SMTCs are required. However, 6 SMTCs would be required if we want to scatter the cells even more.

Observation 3: With scattered beam hopping pattern, up to 6 neighbor SMTCs are required