3GPP TSG-RAN WG2 Meeting #128	R2-2410806

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.8.2

Source: 	Sequans Communications

Title: 	Downlink coverage enhancement access control

Document for:	Discussion and Decision

# Introduction

During the last meeting, the following was discussed and agreed:

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this contribution, we discuss further this topic.

# Discussion

## Barring

Possible barring was discussed in the context of SSB periodicity extension. Currently the SSB periodicity is included in SIB1 (ssb-PeriodicityServingCell in ServingCellConfigCommonSIB) and may be configured up to 160ms. A UE only assumes 20ms periodicity for initial cell selection (cell search).

For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

Hence, up to 160ms, it seems not required to bar a cell in SIB1 for that reason, since it is also in SIB1 that the UE knows the real SSB periodicity – and can then operate perfectly.

Observation 1: Up to 160 ms periodicity, barring doesn’t seem required

If the SSB periodicity is extended to higher value than 160ms, it seems more likely that barring is needed.

Proposal 1: RAN2 waits for RAN1 before deciding whether barring is needed

Moreover, above RAN2 agreements do not define what “DL-CE” means, which makes progress difficult.

## Down-prioritizing / preventing re-selection

This was put in the agenda item based on observations from ‎[1]:

“Let’s consider a serving cell without DL CE, where both UE capable/incapable of DC CE could exit in the serving cell, and 6 neighboring cells, where cell 1/5 are cells with DL CE while the rest of cells are not operating with DL CE. In this case, there is no need for a UE incapable of DL CE (including legacy UEs) to perform neighboring cell measurement or re-select to cell 1/5 since the UE may not be able to camp or normally operate in such cells. However, cell 1/5 can still be considered for UEs capable of DL CE during cell reselection.”

Preventing re-selection if UE cannot camp/normally operate

If a UE is not able to camp or normally operate in such DL CE cells, the cell would be barred. A UE already cannot reselect on a barred cell, as it will not be considered as a candidate for cell reselection.

Observation 2: Preventing re-selection may already be realized by legacy way (cell barring)

However, it is correct that this might come with additional power consumption from UE side, which we believe should be addressed.

Proposal 2: RAN2 investigates power consumption issues (and possible other drawbacks) on legacy UEs

Using “excluded cell list”

In ‎[1], it is proposed to reuse the “excluded cell list”, which also enables the UE to exclude the DL CE cells from candidate for cell reselection. However, the only gain doing so is to prevent SIB1 reading (which has the barring indication). This would hence be a UE power optimisation, enabling to prevent useless SIB1 acquisition.

Observation 3: Reusing “excluded cell list” only prevents SIB1 acquisition

Moreover, one could wonder why this improvement would be required for DL CE while it is not required for other cause of barring (e.g. NES).

On the other hand, above proposal does not help in preventing useless cell search. To be recognized as a cell to be excluded, the UE would have first needed to acquire SSB of the DL CE cell. If the UE does not manage to acquire the DL CE SSB at all, the above proposal is useless. If the UE has no problem acquiring the DL CE SSB, it is not clear why the cell would need to be barred in the first place, and if it is, why the legacy solution would not be enough.

As far as we understood, the main issue with DL CE in the context of SSB periodicity extension is related to initial SSB acquisition (e.g. when the UE assumes 20ms while periodicity is 160ms).

Observation 4: The main issue is related to DL CE cell SSB acquisition, not SIB1 acquisition

So, we believe that in the considered scenario, the most relevant situation is that a UE “not supporting DL CE” (whatever that means) would have issues already acquiring the DL CE SSB (i.e. it would take time / power consumption). This is what should be prevented in the first place, to help UE power consumption.

Observation 5: Legacy UEs should not search for DL CE cells in the first place

Using new SMTC5 list

As detailed in our companion contribution, we believe DL CE (due to beam hopping) will require signalling additional SMTC configurations. E.g., we could be in following scenario where blue cells does not support DL CE, and share the same SMTC; while other neighbour cells support DL CE, and have different SMTC configuration (on a given frequency layer, as SMTCs are per frequency layer).

In general, the issue will be that the UE spends time and power searching DL CE cells on those additional SMTC configurations. For this reason, it is crucial that the NW does not signal DL CE SMTC configurations within legacy SMTC signalling.

Another issue is related to handling of RRMs. E.g., currently the cell reselection requirements are conditional to the number of configured SMTCs. Configuring more SMTCs than supported by legacy UEs would result in making the existing RRM requirements no longer applicable (see Annex). This is more a RAN4 topic, but we believe it would also make sense to not mix up those DL CE cells with legacy cells.

Observation 6: Reusing legacy SMTC signaling for DL-CE cells may lead both to legacy UE power consumption issue, as well as vanishing of legacy RRM requirements

Proposal 3: Additional DL-CE SMTC configurations should NOT be signaled within legacy SMTC signaling

Hence, an alternative could be to consider signalling the DL CE cells SMTC configurations in a new list (e.g. SMTC5). That way, the NW (if it wishes so) can isolate those DL CE cells so that only UEs “supporting DL CE” would try to search for those cells.

Proposal 4: RAN2 considers signaling DL CE cells SMTCs separately (e.g. SMTC5 list)

The NW could either ensure complete non-overlap of SSBs from legacy/DL CE cells, or only partial, as it wishes. In case of partial overlap, if a legacy UE happens to decode a DL CE SSB (by chance), this is not an issue, as it can be handled the legacy way (i.e. barring the DL CE cell, if required).

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Up to 160 ms periodicity, barring doesn’t seem required

Proposal 1: RAN2 waits for RAN1 before deciding whether barring is needed

Observation 2: Preventing re-selection may already be realized by legacy way (cell barring)

Proposal 2: RAN2 investigates power consumption issues (and possible other drawbacks) on legacy UEs

Observation 3: Reusing “excluded cell list” only prevents SIB1 acquisition

Observation 4: The main issue is related to DL CE cell SSB acquisition, not SIB1 acquisition

Observation 5: Legacy UEs should not search for DL CE cells in the first place

Observation 6: Reusing legacy SMTC signaling for DL-CE cells may lead both to legacy UE power consumption issue, as well as vanishing of legacy RRM requirements

Proposal 3: Additional DL-CE SMTC configurations should NOT be signaled within legacy SMTC signaling

Proposal 4: RAN2 considers signaling DL CE cells SMTCs separately (e.g. SMTC5 list)

# References

- R2-2408655, “Consideration on downlink coverage enhancements”,  ZTE Corporation, Sanechips

# Annex – 38.133

#### 4.2C.2.3 Measurements of intra-frequency NR cells […] The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

#### 4.2C.2.4 Measurements of inter-frequency NR cells [….] The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the [UE capability], otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in Table 4.2C.2.4-1 and Table 4.2C.2.4-2 is expected.