3GPP TSG-RAN2#128	R2-2410814

Orlando, USA, November 18th-22nd, 2024

Agenda item:		8.4.4

Source:		Lenovo

Title:		 LP-WUS in RRC Connected Mode

Document for:		Discussion and Decision

- Introduction

Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [1]. The following agreements were made at the previous meetings:

Agreements at RAN1 #116bis: - Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used. - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g., whether above is applicable to one or more serving cells) Agreements at RAN1 #118: - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. • LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. • UE is configured with legacy C-DRX configurations as Rel-18 • UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) o FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration • LP-WUS triggers the start of a timer during which UE monitors PDCCH o FFS the timer is existing timer and/or new timer • UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected o drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL • No impact on RRM/RLM/BFD measurement requirements is assumed • For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) o Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up o Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not • UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration • Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. • Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. • FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE - Note: Above can be revisited considering RAN2 decisions. Agreements at RAN2 #127: - For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. Agreements at RAN2 #127bis: - For option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LP-WUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LP-WUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

This contribution looks at the possible RRC Connected Mode procedures for an LP-WUR enabled UE including activation/ deactivation and the use of LP-WUS in Connected Mode DRX.

- Discussion

  - Activation/ Deactivation of LP-WUS Monitoring

RAN1 has already agreed in previous meetings that gNB RRC singaling will be used to enable/ disable PDCCH monitoring triggered by LP-WUS. And for further activation/ deactivation of LP-WUS monitoring after it has been enabled, RAN1 has provided four options as mentioned above.

- RAN1 has agreed that gNB RRC signalling is used to enable/disable PDCCH monitoring triggered by LP-WUS.

In our view, the activation/deactivation of LP-WUS is related to the traffic arrival and channel conditions, we think at least some implicit activation/ deactivation methods need to be discussed. For example, LP-WUS monitoring may be deactivated when some UL data becomes available for transmission in the HARQ buffers. We also think it would be beneficial to discuss other activation/ deactivation methods so that network may have more control of when the UE monitors for LP-WUS.

Furthermore, RAN1 has agreed that the gNB is aware of LP-WUS monitoring in the UE. We think that some explicit indication may be used for the purpose of network awareness, i.e., the UE indicates to the network when it is actively monitoring for LP-WUS or when it stops monitoring for the LP-WUS (e.g., due to fulfilling the LR exit conditions). For example, some dedicated RRC signalling may be used for this purpose.

In the case that LP-WUS monitoring is enabled, RAN2 to consider additional conditions to activate/deactivate the LP-WUS monitoring.

UEs in RRC_CONNECTED state should report its LP-WUS monitoring status to gNB, e.g., by dedicated RRC signalling.

  - LP-WUS for Connected-Mode DRX

As per the R16 power saving item, in RRC Connected Mode, the MR may be configured with a DCI 2_6 (Wake-Up Signal). This DCI format 2_6 is monitored outside of the DRX active time to decide whether the MR needs to be woken up in the next drx-onDurationTimer. RAN2 should discuss how the UE may use the LP-WUS to wake up the MR such that the power saving gain achieved by using the LR must be higher than the power saving gain achieved by using only the DCI format 2_6, at the least.

- In RAN2#127bis meeting, it has been agreed that for Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. And it is FFS whether the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. During the RAN1#118 meeting, it has been agreed that for Option 1-2, UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. It was also further agreed in RAN1 that the Rel-16 mechanism will be reused for periodic CSI and periodic L1/RSRSP reporting.

- It was unclear during the last meeting if the duration of drx-onDurationTimer is considered as active time. In our view, even if the timer is running, it will no longer be considered as active time when Option 1-2 is configured since drx-onDurationTimer will no longer trigger PDCCH monitoring. Instead, the running of the new timer (i.e., PDCCH monitoring triggered by LP-WUS reception) will be considered as active time when Option 1-2 is configured. Hence, the UE should be able to receive the LP-WUS signal even during the drx-onDurationTimer duration (depending on the configuration/ cycle of LOs).

- There are two schools of thought regarding whether drx-onDurationTimer must be started – whether it is always started but PDCCH monitoring is not done during the running of this timer, or that drx-onDurationTimer is not started at all. In our view, neither of these options work well for CSI reporting. As per legacy mechanism, semi-persistent CSI reporting is only done when drx-onDurationTimer is started. Hence, not starting the drx-onDurationTimer will impact at least semi-persistent CSI reporting. Whereas, always starting the drx-onDurationTimer will degrade the power saving gain offered by using LP-WUS as the UE would need to wake-up at least once every DRX cycle for such reporting when configured. Additionally, periodic CSI/L1-RSRP reporting will also always be reported if drx-onDurationTimer is either always started or never started (based on network configuration as agreed in RAN1) which can increase the power consumption of the UE when the UE may not have any DL data to receive. Hence, we think the drx-onDurationTimer can be started on the basis of whether the UE is already actively monitoring PDCCH (e.g., PDCCH monitoring triggered by LP-WUS or running of drx-InactivityTimer etc.) when the drx-onDurationTimer is to be started. This way, the UE only reports such CSI when necessary. The exact details may be left for FFS.

- Proposal 3 The UE should be able to receive LP-WUS within the duration of legacy drx-onDurationTimer (if configured) for Option 1-2.

- Proposal 4 For Option 1-2, the legacy drx-onDurationTimer is started only if the UE is already monitoring PDCCH when the drx-onDurationTimer is to be started. The exact details may be left to FFS.

- For both Option 1-1 and Option 1-2, it’s possible that there’s no data transmission expected before the timer (i.e., drx-onDurationTimer for option 1-1, and newly defined timer for option 1-2) expires. In this case, network may indicate UE to stop PDCCH monitoring and activate LP-WUS monitoring directly for power efficiency consideration. As the baseline solution, legacy (Long) DRX Command MAC CE can be re-used. When UE receives such signalling as legacy, UE will stop the PDCCH monitoring and start the LP-WUS monitoring. Therefore, it is suggested that RAN2 to discuss using legacy network signalling (e.g., DRX Command MAC CE, etc.) to stop PDCCH monitoring and activate LP-WUS monitoring (if possible) for both option1-1 and option1-2.

- Proposal 5 RAN2 to discuss using legacy network signalling (e.g., DRX Command MAC CE etc.) to stop PDCCH monitoring and activate LP-WUS monitoring (if possible), for both option1-1 and option1-2.

  - MR wake-up due to UL data

During the Study Item Phase, it had been agreed in RAN2 that MR needs to wake-up for PDCCH monitoring when the MR performs some UL data transmission [3].

- Agreements: UL transmission by MR also triggers PDCCH monitoring by MR.

This also means that when new UL data becomes available for transmission, it can be considered as an implicit indication to wake-up the MR, if the MR had previously been asleep. This implicit and/ or explicit indication to wake-up may result in a frequent wake-up of MR whenever new data becomes available for transmission after LP-WUS activation leading to a reduction in the power saving gain offered by configuring the LP-WUR.

- Frequent wake-up of MR caused by UL transmission can lead to a reduction in the power saving gain offered by LP-WUR configuration.

Hence, some enhancements may be considered in the wake-up behavior of the MR to optimize the power saving gain even when some UL transmission needs to take place. For example, one way to avoid reduction in the power saving gain is by delaying the MR wake-up (where wake-up refers to transmission of UL data and the consequent monitoring of PDCCH) when new UL data becomes available for transmission (especially if the data that arrived is not urgent), i.e., the wake-up may be postponed until absolutely necessary.

Hence, RAN 2 should discuss how MR sleep time may be prolonged when UL data becomes available for transmission.

- Proposal 6 RAN2 to discuss means of improving MR sleep time, e.g., by delaying (non-urgent) UL transmissions.

  - Others

- SPS and CG impacts

- For UEs in RRC_Connected state, the resources for SPS and CG are configured by the network to UEs for potential downlink and uplink transmission. The UE may perform DL reception and UL transmission on the configured SPS and CG resources. LP-WUS impacts on the configured SPS/CG needs to be identified when UE performs LP-WUS monitoring.  It is suggested that RAN2 discuss potential UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state. For example, in the case of UE performs LP-WUS monitoring, i.e., UE MR enters sleep state, whether the configured SPS/CG resource remains valid, and whether UE MR wakes up on the configured SPS/CG resource needs to be clarified.

- Proposal 7 RAN2 to discuss UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state.

- Dual DRX Groups

- Serving cells of a MAC entity may be configured by RRC with two DRX groups, each DRX group has separate DRX parameters to be applied for FR1 and FR2. Then RAN2 needs to identify that for UEs configured with LP-WUS configuration, whether common LP-WUS configuration or per DRX group LP-WUS configuration are provided to UE. In legacy, DRX configuration is provided in the MAC cell group configuration, while DCP (if configured) is provided in PCG configuration. Considering that the solutions introduced for DCP mechanism is taken as baseline for LP-WUS design in Option 1, it is proposed that LP-WUS configuration is configured per-DRX group for UEs configured with dual DRX groups.

- Proposal 8 LP-WUS configuration is configured per-DRX group for UEs configured with dual DRX groups.

- Conclusion

This document discusses some procedures for a LR capable UE in the RRC Connected Mode. Following are the proposals and observations made in this document:

- Observation 1: RAN1 has agreed that gNB RRC signalling is used to enable/disable PDCCH monitoring triggered by LP-WUS.

- Proposal 1: In the case that LP-WUS monitoring is enabled, RAN2 to consider additional conditions to activate/deactivate the LP-WUS monitoring.

- Proposal 2: UEs in RRC_CONNECTED state should report its LP-WUS monitoring status to gNB, e.g., by dedicated RRC signalling.

- Proposal 3: The UE should be able to receive LP-WUS within the duration of legacy drx-onDurationTimer (if configured) for Option 1-2.

- Proposal 4: For Option 1-2, the legacy drx-onDurationTimer is started only if the UE is already monitoring PDCCH when the drx-onDurationTimer is to be started. The exact details may be left to FFS.

- Proposal 5: RAN2 to discuss using legacy network signalling (e.g., DRX Command MAC CE etc.) to stop PDCCH monitoring and activates LP-WUS monitoring (if possible) for both option1-1 and option1-2.

- Observation 2: Frequent wake-up of MR caused by UL transmission can lead to a reduction in the power saving gain offered by LP-WUR configuration.

- Proposal 6: RAN2 to discuss means of improving MR sleep time, e.g., by delaying (non-urgent) UL transmissions.

- Proposal 7: RAN2 to discuss UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state.

- Proposal 8: LP-WUS configuration is configured per-DRX group for UEs configured with dual DRX groups.

- References

[1] 3GPP TR 38.869 v0.2.0: Study on low-power wake up signal and receiver for NR (Release 18)

[2] 3GPP TS 38.321, Medium Access Control (MAC) protocol specification.

[3] R2 123bis Session Notes: Report from session on Mobility Enh, Mobile IAB and LP-WUS.

[4] 3GPP TSG RAN WG1#118, Chair’s Motes