3GPP TSG-RAN WG2 Meeting #128	R2-2410815

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	vivo

Title:	MRO for Rel-18 mobility features

Agenda Item:	8.10.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#127bis meeting, RAN2 reached the following agreements regarding the MRO for Rel-18 mobility features:

LTM Agreements - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. Agreements - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR. CHO with candidate SCGs Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

In this contribution, we will provide further analysis of the remaining issues.

- Discussion

  - LTM

At the RAN2#127bis, it was agreed to include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. The agreement was made due to the SHR may be triggered by the threshold rather than T304, e.g., triggered by T310/T312. For these cases, the RACH type (RA-based or RACH-less) can not be implicitly indicated based on the RA info in SHR since the RA info can only be included for the T304-triggered cases.

However, if the SHR is not triggered by T304 threshold, i.e., triggered by T310/T312 threshold, the RACH type is useless as the near failure is not due to RACH attempt to target but due to poor radio at source and/or too-late HO command.

To address the issue, two options can be considered:

- Option 1: Revert the previous agreement, i.e., no explicit indicator in SHR on whether the successful LTM execution was RACH-less or RACH-based.

- Option 2: For RACH-less LTM, the UE logs and reports whether and how the UE got the TA value used for a near-failure LTM switch (gNB indicated or UE determined) in the SHR. The present of TA value source means RACH-less LTM.

Observation 1: If the SHR is not triggered by T304 threshold, i.e., triggered by T310/T312 threshold, the RACH type is useless as the near failure is not due to RACH attempt to target, but is due to poor radio at the source and/or too-late HO command.

Proposal 1: For the RACH type in SHR for LTM, the following two options can be considered:

- Option 1: Revert the previous agreement, i.e., no explicit indicator in SHR on whether the successful LTM execution was RACH-less or RACH-based. The RACH type can be explicitly indicated by the presence of RA info in SHR.

- Option 2: For RACH-less LTM, the UE logs and reports whether and how the UE got the TA value used for a near-failure LTM switch (gNB indicated or UE determined) in the SHR. The presence of TA value source can indicate RACH-less LTM.

  - CHO with candidate SCG

At the RAN2#125bis meeting, it was agreed to study failure and near-failure scenarios for CHO with candidate SCGs.

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

However, the current progress is mainly on the failure case. And it was agreed that UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO)

From the NW perspective, the NW expects the conditions for CHO and CPAC is as close as possible. So if PCell changes with PSCell addition/change is successfully executed but if the time difference between T1 and T2 is large, the NW needs to know that situation, UE should report some information to assist the NW in optimizing the configuration.

For the above near-failure scenario, the SHR can be taken as the baseline for enhancement. The most straightforward way is introducing another triggering event for the SHR report, i.e., the time gap between the time when conditions for CHO and CPAC are met exceeds a configured threshold. In addition, the same info should be included in the SHR.

Proposal 2: Introduce another triggering event for the SHR report, i.e., the time gap between the time when conditions for CHO and CPAC are met exceeds a configured threshold.

Proposal 3: For the near-failure scenario of CHO with candidate SCGs, introduce the following information in the SHR report:

- Time difference time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO).

- The first fulfilled event, CHO or CPAC.

- Conclusion

Based on the above analysis, we make the following proposals:

LTM

Observation 1: If the SHR is not triggered by T304 threshold, i.e., triggered by T310/T312 threshold, the RACH type is useless as the near failure is not due to RACH attempt to target, but is due to poor radio at the source and/or too-late HO command.

Proposal 1: For the RACH type in SHR for LTM, the following two options can be considered:

- Option 1: Revert the previous agreement, i.e., no explicit indicator in SHR on whether the successful LTM execution was RACH-less or RACH-based. The RACH type can be explicitly indicated by the presence of RA info in SHR.

- Option 2: For RACH-less LTM, the UE logs and reports whether and how the UE got the TA value used for a near-failure LTM switch (gNB indicated or UE determined) in the SHR. The presence of TA value source can indicate RACH-less LTM.

CHO with candidate SCG

Proposal 2: Introduce another triggering event for the SHR report, i.e., the time gap between the time when conditions for CHO and CPAC are met exceeds a configured threshold.

Proposal 3: For the near-failure scenario of CHO with candidate SCGs, introduce the following information in the SHR report:

- Time difference time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO).

- The first fulfilled event, CHO or CPAC.

- Reference