3GPP TSG-RAN WG2 Meeting #128	R2-2410816

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	vivo

Title:	Discussion on RACH optimization for SDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

- Introduction

At the RAN2#127 meeting, the following agreements were made regarding RACH optimization for SDT:

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it. UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

At the RAN2#127bis meeting, further agreements were reached:

Agreements - For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log. - RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

In this contribution, we will provide our views on the open issues.

- Discussion

For the failure cause, the SDT procedure will be unsuccessfully completed for the following cases:

Once initiated, the SDT procedure is either: - successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or to continue in RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or - unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or integrity check failure while SDT procedure is ongoing, or expiry of SDT-specific timing alignment timer or configuredGrantTimer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

From our understanding, within the causes, cell re-selection is not an abnormal case, and the rest causes are quite similar to RLF causes as follows:

rlf-Cause-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, beamFailureRecoveryFailure, lbtFailure-r16, bh-rlfRecoveryFailure, t312-expiry-r17, spare1},

Therefore, cell re-selection does not need to be indicated in the failure cause of SDT and the other cause values can be considered: T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, integrityCheckFailure, TATimer-Expiry.

Proposal 1: For the SDT failure cause in RA-Report, the candidate values include: T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, integrityCheckFailure, TATimer-Expiry.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Proposal 1: For the SDT failure cause in RA-Report, the candidate values include: T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, integrityCheckFailure, TATimer-Expiry.

- Reference