3GPP TSG-RAN2 Meeting #128	R2- 2410818

# Orlando, United States, Nov 18th – 22nd, 2024

Agenda item:	6.1.1.1

Source:	LG Electronics Inc.

Title: 	Discussion on RAN1 LS on waveform determination

Document for:	Discussion and Decision

1.	Introduction

In RAN2#128 meeting, RAN2 has received an LS from RAN1 on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH (R2-2409515) [1]. This document shows our views on answers of the questions from RAN1.

2.	Discussion

According to the RAN1 LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH [1], there is ambiguity on RAN1 which msg3-transformPrecoder is used, for fallbackRAR case and for the case when MsgA-TransformPrecoder is not configured. During the RAN1 discussion, following two alternatives were discussed for each case, without any conclusion:

For this case, the following two alternatives were discussed on which msg3-transformPrecoder should be used without conclusion in RAN1. - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - Alt 2: - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.

Based on RAN1 discussion, following questions are asked to RAN2:

Q1: From RAN2 point of view, which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and for MsgA PUSCH when msgA-TransformPrecoder is not configured respectively? Q2: Is the related UE behavior already defined in RAN2 spec?

First, in our understanding, RAN2 had not extensively discussed about this issue before and the details of waveform determination is currently defined in RAN1. Therefore, it should be note that RAN2 only gives a preference based on existing RACH partitioning framework, and the final decision should be made in RAN1.

For Q1, regarding MsgA PUSCH when msgA-TransformPrecoder is not configured, during the RACH partitioning discussion in RAN2#119 meeting, based on the submitted CR in [2], it is agreed in [3] to use the RA parameters which are related to the RO/preamble configuration using the RACH-ConfigCommon in the same AdditionalRACH-Config, in order to simplify the UE implementation. Following is an exemplary description in current RRC specification to clarify that RA parameter in RACH-ConfigCommon in the same AdditionalRACH-Config is used, if the configuration is not provided in RACH-ConfigCommonTwoStepRA.

msgA-RestrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets for 2-step random access type preamble. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE applies the value in field restrictedSetConfig in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the value of restrictedSetConfig in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access.

Similarly, for msgA-TransformPrecoder, it would be slightly preferred to use the corresponding parameter (i.e., msg3-TransformPrecoder) in RACH-ConfigCommon in the same AdditionalRACH-Config as well, in order to simplify the UE implementation to determine which parameter should be used for RA procedure (i.e., Alt 2).

For fallbackRAR case, same principle can be applied in order to have a common procedure to determine RA-related parameter for all cases. Therefore, from RAN2 perspective, it is preferred to use the corresponding parameter (i.e., msg3-TransformPrecoder) in RACH-ConfigCommon in the same AdditionalRACH-Config (i.e., Alt 2).

Therefore, for both cases, RAN2 should answer that Alt 2 is preferred based on existing RACH partitioning framework defined in RAN2.

For Q2, as mentioned above, there is no related UE behaviour to determine msg3-transformPrecoder for both cases.

Proposal 1. For Q1, RAN2 answer as follows: “From RAN2 perspective, it is preferred to use msg3-transformPrecoder as follows for both cases:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.”

Proposal 2. For Q2, RAN2 answer as: “In current RAN2 specification, there is no related UE behaviour to determine msg3-transformPrecoder for both cases”

3.	Conclusion

In this contribution, it is discussed on an LS from RAN1. This document includes following proposals:

Proposal 1. For Q1, RAN2 answer as: “From RAN2 perspective, it is preferred to use msg3-transformPrecoder as follows for both cases:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.”

Proposal 2. For Q2, RAN2 answer as: “In current RAN2 specification, there is no related UE behaviour to determine msg3-transformPrecoder for both cases”

4.	Reference

[1] R2-2409515 LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH

[2] R2-2207989 RRC corrections to common RACH framework, Huawei, HiSilicon

[3] R2-2208996 Correction for features applicable for common signalling for RACH Partitioning, Ericsson