3GPP TSG-RAN2 Meeting #128	R2- 2410819

# Orlando, United States, Nov 18th – 22nd, 2024

Agenda item:	7.0.2.9 (NR_cov_enh2-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on RAN1 LS on the time period for R18 preamble repetition

Document for:	Discussion and Decision

1.	Introduction

In RAN2#128 meeting, RAN2 has received an LS from RAN1 on R18 CovEnh WI (R2-2409513) [1]. This document shows our views on answers of the questions from RAN1.

2.	Discussion

According to the RAN1 LS on the time period for R18 preamble repetition [1], there are several questions to RAN2 in order to determine time period for Msg1 repetition based on “configured” repetition numbers, as follows:

Q0: Has the case#1 above been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification. Q1: Has the case#2 above been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification. Q2: Has the following case#1-rev (a combination of the case#1 and case#2) been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification. - Case#1-rev: the same as the case#1 except that the feature combination msg1-repetition-r18 in the second RACH-ConfigCommon is replaced with msg1-repetition-r18+redCap-r17. Q3: If the answer to Q1 is “not precluded”, according to the following specification excerpt of TS 38.331, a UE capable of msg1-repetition but incapable of RedCap ignores the RACH resource indicated by the third FeatureCombinationPreambles in the case#2. In addition to the ignored RACH resource, does it mean that the corresponding parameter repetition number n8 is also ignored, i.e. considered as “not configured” to the UE? featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.. Q4: If the answer to Q1 is “not precluded”, according to the following RAN2 agreement, a UE capable of RedCap must select the RACH resource indicated by the third FeatureCombinationPreambles in the case#2. For the RedCap UE, does it mean that the corresponding parameter repetition number n2 and n4 indicated by the first and second FeatureCombinationPreambles in the case#2 are considered as “not configured” to the UE? RAN2#116bis Proposal 1’: [Easy] In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW. - Agreed. Q5: If the answer to Q1 is “not precluded”, in a case where a UE selects a set of PRACH resources according to featurePriorities and S5.1.1d of TS 38.321, whether or not only parameter repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”?

For Q0, the Case#1 is described as follows:

Case#1: Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon. All these repetition numbers are associated to one feature combination(i.e. msg1-repetition-r18). Figure 1

On the other hand, according to the current RACH partitioning framework, only one FeatureCombinationPreambles IE with the same feature combination can be defined for each RA type for each Msg1 repetition number. In other words, more than one FeatureCombinationPreambles IEs are not allowed in RAN2 only if those FeatureCombinationPreambles IEs are associated with same feature combination and same Msg1 repetition.

– FeatureCombinationPreambles The IE FeatureCombinationPreambles associates a set of preambles with a feature combination. For parameters which can be provided in this IE, the UE applies this field value when performing Random Access using a preamble in this featureCombinationPreambles, otherwise the UE applies the corresponding value as determined by applicable Need Code, e.g. Need S. On a specific BWP, there can be at most one set of preambles associated with a given feature combination per RA Type (i.e. 4-step RACH or 2-step RACH) per MSG1 repetition number.

Therefore, since each the FeatureCombinationPreambles IE includes different Msg1 repetition number in Case#1, Case#1 is possible configuration in RAN2 perspective.

Proposal 1. For Q0, RAN2 answers as: “Case#1 is not precluded by the existing RAN2 signaling and procedures.”

For Q1, the Case#2 is described as follows:

Case#2: Only single RACH-ConfigCommon is configured with multiple feature combinations. As shown in Figure 2, repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17). Figure 2

In other words, in Case#2, there are three FeatureCombinationPreambles IEs in RACH-ConfigCommon included directly within BWP configuration:

- First FeatureCombinationPreambles IE configures set of RA resource associated with [Msg1 repetition, Msg1 repetition number = n2];

- Second FeatureCombinationPreambles IE configures set of RA resource associated with [Msg1 repetition, Msg1 repetition number = n4]; and

- Third FeatureCombinationPreambles IE configures set of RA resource associated with [Msg1 repetition + RedCap, Msg1 repetition number = n8].

This case is allowed and not precluded in RAN2 signalling perspective, since each FeatureCombinationPreambles IE has different Msg1 repetition number or different feature combination.

Proposal 2. For Q1, RAN2 answers as: “Case#2 is not precluded by the existing RAN2 signaling and procedures.”

For Q2, case#1-rev can be described as below:

Case#1-rev: the same as the case#1 except that the feature combination msg1-repetition-r18 in the second RACH-ConfigCommon is replaced with msg1-repetition-r18+redCap-r17. - BWP-UplinkCommon - RACH-ConfigCommon - FeatureCombinationPreamble (1): Msg1 repetition, Repetition number = n2 - FeatureCombinationPreamble (2): Msg1 repetition, Repetition number = n4 - AdditionalRACH-Config - RACH-ConfigCommon - FeatureCombinationPreamble (3): Msg1 repetition+RedCap, Repetition number = n8

As mentioned above, RACH configuration is allowed unless there are more than one FeatureCombinationPreambles IEs configured with same feature combination and same Msg1 repetition number for a RA type. Therefore, case#1-rev is also allowed in RAN2 signaling, since each FeatureCombinationPreambles IE in case#1-rev has different feature repetition number or different feature combination.

Proposal 3. For Q2, RAN2 answers as: “Case#1-rev is not precluded by the existing RAN2 signaling and procedures.”

For Q3, the highlighted text below is added in order to disallow the usage of RACH partition associated with any feature defined in the future releases (e.g., Rel-17 UEs shall not use RACH partition associated with Msg1 repetition defined in Rel-18), based on the discussion point 8 of [2].

featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.

In general, if a RRC configuration is ignored, the corresponding configuration is not visible in the MAC layer. In this sense, if the UE is incapable of Msg1 early indication for RedCap (e.g., for non-(e)RedCap UE), the corresponding RACH resource is ignored, and the MAC layer considers that the corresponding FeatureCombinationPreambles IE is not configured. Therefore, from RAN2 perspective, for Case#2, the UE considers that all configuration of third FeatureCombinationPreambles IE (i.e., set of RA resource associated with [RedCap + Msg1 repetition, Msg1 repetition number = n8]) is not configured, if the UE is incapable of RedCap.

Proposal 4. For Q3, RAN2 answers as: “For Case#2, if the UE is incapable of RedCap, the UE considers that all parameters within third FeatureCombinationPreamble is not configured from MAC perspective.”

For Q4, it should be first noted that the following agreement should not be interpreted that RedCap UE must select RA resource for RedCap early indication (i.e., RedCap RACH partition), regardless of other feature combinations. Rather, it should be interpreted that RedCap UE prioritized RedCap RACH partition over common RA resource partition (i.e., RACH partition without any feature indication). Therefore, it is possible that other RACH partition (with other feature indication) is selected even though RedCap RACH partition is configured.

RAN2#116bis Proposal 1’: [Easy] In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW. - Agreed

In detail, according to current MAC procedure, RedCap UE (i.e., UE capable of RedCap) selects RACH resource based on the channel quality as follows for Case#2:

- If RSRP of pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum8-r18, applicable Msg1 repetition number includes 2, 4, and 8. Then, all of three FeatureCombinationPreamble are available and the RACH resources configured by third FeatureCombinationPreamble (i.e., [RedCap + Msg1 repetition, Msg1 repetition number = n8]) is selected.

- Else if RSRP of pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum4-r18, applicable Msg1 repetition number includes 2, and 4. In this case, only first and second FeatureCombinationPreamble are available, since Msg1 repetition number 8 is not applicable. Then, the RACH resources configured by second FeatureCombinationPreamble (i.e., [Msg1 repetition, Msg1 repetition number = n4]) is selected.

- Else if RSRP of pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum2-r18 or if rsrp-ThresholdMsg1-RepetitionNum2-r18 is not configured for the case of no legacy partition, applicable Msg1 repetition number includes 2. Then, only first FeatureCombinationPreamble are available and the RACH resources configured by first FeatureCombinationPreamble (i.e., [Msg1 repetition, Msg1 repetition number = n2]) is selected.

- Otherwise, RACH resource which is not associated with any feature is selected.

In addition, since the selection of RACH partition is performed in MAC layer based on the configuration (unless it is ignored in RRC layer), the parameters included in corresponding FeatureCombinationPreambles IEs should be considered as “configured,” even though the RACH partition is not selected for preamble transmission.

Therefore, if the UE is capable of RedCap, the UE should consider that repetition number n2, n4, n8 indicated by the first, second, and third FeatureCombinationPreambles in the case#2 are “configured.”

Proposal 5. For Q4, RAN2 answers as follows:

- Even though the UE is capable of RedCap UE, there are some cases that UE uses RACH resources configured by first or second FeatureCombinationPreambles, based on RSRP of downlink pathloss reference for Case#2.

- Even though RACH resources in some FeatureCombinationPreambles IEs are determined as not available or not selected in MAC layer, the UE should consider that the corresponding FeatureCombinationPreambles IE is “configured”, unless it is ignored in RRC layer.

- Therefore, the corresponding parameter repetition number n2 and n4 indicated by the first and second FeatureCombinationPreambles in the case#2 are also considered as “configured”

For Q5, as mentioned in Q4, even though the MAC selects one RACH partition (with one Msg1 repetition number), the UE should consider that the other RACH partitions are also configured. In this sense, all the parameter repetition numbers provided by the RRC layer should be considered as “configured,” including the Msg1 repetition number associated with the selected set of PRACH resources and Msg1 repetition numbers which associated with other set of PRACH resources.

Note that it is possible to use the Msg1 repetition number even though it is not selected for initial transmission, since the fallback from low repetition number to high repetition number is supported within the same feature combination. Therefore, even though the UE selects set of PRACH resources associated with low repetition number (e.g., first FeatureCombinationPreambles IE in Case#2), UE is allowed to switch to the set of PRACH resources associated with high repetition number with the same feature combination (e.g., second FeatureCombinationPreambles IE in Case#2), if it is configured.

Proposal 6. For Q5, RAN2 answers as follows:

- All the parameter repetition numbers provided by the RRC layer should be considered as “configured,” including the Msg1 repetition number associated with the selected set of PRACH resources and Msg1 repetition numbers which associated with other set of PRACH resources.

- According to MAC procedure, it possible to use another Msg1 repetition number which is not selected for initial transmission of RA preamble, since it is allowed to fallback from low the Msg1 repetition number to high Msg1 repetition number within the same feature combination.

3.	Conclusion

In this contribution, it is discussed on LSs from RAN1. This document includes following proposals:

Proposal 1. For Q0, RAN2 answers as: “Case#1 is not precluded by the existing RAN2 signaling and procedures.”

Proposal 2. For Q1, RAN2 answers as: “Case#2 is not precluded by the existing RAN2 signaling and procedures.”

Proposal 3. For Q2, RAN2 answers as: “Case#1-rev is not precluded by the existing RAN2 signaling and procedures.”

Proposal 4. For Q3, RAN2 answers as: “For Case#2, if the UE is incapable of RedCap, the UE considers that all parameters within third FeatureCombinationPreamble is not configured from MAC perspective.”

Proposal 5. For Q4, RAN2 answers as follows:

- Even though the UE is capable of RedCap UE, there are some cases that UE uses RACH resources configured by first or second FeatureCombinationPreambles, based on RSRP of downlink pathloss reference for Case#2.

- Even though RACH resources in some FeatureCombinationPreambles IEs are determined as not available or not selected in MAC layer, the UE should consider that the corresponding FeatureCombinationPreambles IE is “configured”, unless it is ignored in RRC layer.

- Therefore, the corresponding parameter repetition number n2 and n4 indicated by the first and second FeatureCombinationPreambles in the case#2 are also considered as “configured”

Proposal 6. For Q5, RAN2 answers as follows:

- All the parameter repetition numbers provided by the RRC layer should be considered as “configured,” including the Msg1 repetition number associated with the selected set of PRACH resources and Msg1 repetition numbers which associated with other set of PRACH resources.

- According to MAC procedure, it possible to use another Msg1 repetition number which is not selected for initial transmission of RA preamble, since it is allowed to fallback from low the Msg1 repetition number to high Msg1 repetition number within the same feature combination.

4.	Reference

[1] R2-2409513 LS on the time period for R18 preamble repetition

[2] R2-2206708 Phase2, CR review - Report from [AT118-e][507][RA Part] CP open issues and CR 38.331 (Ericsson)