3GPP TSG RAN2 Meeting #128	R2-2410842

Orlando, USA, November 18th - 22nd, 2024

Agenda item:	8.5.3

Source:	Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS

Title:	Considerations on OD-SIB1 feature

Document for:		Discussion and Decision

# Introduction

In the previous RAN Plenary #105 meeting in Melbourne, during the discussion on NES checkpoint, a proposal was made by Deutsche Telekom’s delegate & RAN VC to remove restriction that Cell A and NES cell must be different (at different times). The proposal was endorsed, according to RAN Plenary’s MoMs shown below for reference, but the WID had not been updated respectively.

RP-242141	On demand SIB 1 and the need for case 1 for multi-vendor RAN	Vodafone, Deutsche Telekom,

Orange, Lenovo, Google,

Fraunhofer IIS and

Fraunhofer HHI, ETRI,

CEWiT

Replaces

RAN WG1 and RAN WG2 discussed 3 scenarios for the support of on demand SIB1:

- Case 1: Standalone, where configuration to request SIB1, SIB1 request and SIB1 are delivered within the SIB1-less NES cell

- Case 2: Anchor: configuration to request SIB1 is delivered by an Anchor cell, but SIB1 request and SIB1 are delivered within the SIB1-less NES cell

- Case 3: Anchor: configuration to request SIB1, SIB1 request and SIB1 (for the SIB1-less NES cell) are delivered by the Anchor cell

Proposal 1: The co-signing companies recommend supporting Case 1 in Rel 19 in addition to case 2.

DT: some clarification is needed before we could go forward with case 1, anchor cell can be same as NES cell? a cell could work as normal cell during the day and as NES cell in the night, if these are 2 different cells, then we cannot agree to case 1

RAN chair: focussing on case 2 is acceptable?

Qualcomm: if WUS configuration for neighbours is still possible, then we would be ok

RAN chair: asks RAN1 chair to draft a way forward in RP-242353

The document was noted.

Replaced by

RP-242353	Way forward on NES checkpoint	RAN1 chair (Samsung)

The document was revised.

Replaced by RP-242384

RP-242384	Way forward on NES checkpoint	RAN1 chair (Samsung)

Replaces

RP-242353

DT: would like to remove restriction that Cell A and NES cell must be different (at different times)

Panasonic: things that this is possible

RAN chair: as you talk about different times, this is possible

conclusion: proposal is endorsed

In RAN2#127-bis meeting, the following relevant agreements were made. However, during the online discussion, it was clear that some companies were not taking into consideration this sub-case of Case 2 as it was not clearly included at the WID.

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, our intention is to clarify this issue so that RAN2 WG has a common understanding and takes into consideration for the next RAN2 meetings’ steps/agreements the fact that the provision of WUS Config for a cell itself is covered by the work task from RAN Plenary to RAN2.

# Discussion

According to the updated WID for NES that was approved in RAN Plenary #105, Case 2 was agreed to be supported for normative work. The WID objective referring to Case 2 is shown below.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

Unfortunately, the above-mentioned definition of Case 2 was not updated properly during the previous RAN Plenary meeting to reflect the proposal of allowing NES cell and cell A to be the same cell at different times, a proposal that was finally endorsed as highlighted at Introduction paragraph. From our perspective, we consider a possible scenario that a gNB would like to become a SIB1-less cell after operating as Cell A (i.e. cell A could work as normal cell during the day and as NES cell in the night).

To avoid any possible misunderstanding among RAN2 delegates, R2-2408454 was submitted at the previous RAN2 meeting, RAN2#127bis, aiming to clarify this point with the following proposal, “Proposal 1: It is proposed to clarify that Cell A can also transmit the WUS configuration for itself in the case it intends to become a SIB1-less cell.”. During the online discussion, this specific proposal was not treated, but it became obvious that not all RAN2 delegates were willing to agree on cell A transmitting the WUS configuration for itself, as the WID did not include this point clearly. However, based on the highlighted agreements of RAN2#127-bis meeting (shown at Introduction section), it seems that NES cell can provide UL-WUS configuration for itself.

Observation 1: Following strictly the updated WID, and specifically the note referring to Cell A (Cell A: A cell that is periodically transmitting at least its own SIB1), leads to confusion as it gives the wrong impression that a Cell A is not allowed to become NES cell in different time.

It is obvious that an agreement is required focused on clarifying that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell so that RAN2 WG works along this agreement.

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

Proposal 2: If the above-mentioned proposal cannot be agreed during this RAN2 meeting, a proper revision of WID in the upcoming RAN Plenary meeting should be pursued to reflect the already endorsed proposal.

# Summary and Conclusions

In this document, we highlighted a misalignment observed in RAN2 WG stemming from the fact that the updated WID gives the wrong impression that a Cell A is not allowed to become NES cell in different time and thus we propose the following:

Observation 1: Following strictly the updated WID, and specifically the note referring to Cell A (Cell A: A cell that is periodically transmitting at least its own SIB1), leads to confusion as it gives the wrong impression that a Cell A is not allowed to become NES cell in different time.

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

Proposal 2: If the above-mentioned proposal cannot be agreed during this RAN2 meeting, a proper revision of WID in the upcoming RAN Plenary meeting should be pursued to reflect the already endorsed proposal.

# References

[1]		“MeetingReport_RAN_105_240912_eom”

[2] 		R2-2408454, “Barring and SIB1-less case 2”

[3]		“Draft_RAN2_127b_Meeting_Report_v3”