3GPP TSG-RAN WG2 Meeting #128                                    	R2-2410875

Orlando, US, Nov. 18th – 22nd, 2024

Agenda item:	8.9.3

Source:	Deutsches Zentrum für Luft- und Raumfahrt (DLR) e.V., ESA, Inmarsat, Viasat

Title:	Implicit pointer for locating DSA replicas for EDT of CB-Msg3

Document for:	Discussion and Decision

- Introduction

RAN WID [1] has the following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signalling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

In RAN2#127bis, it was agreed that

RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if
configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

In this document, we discuss procedures that allow a UE to decide on the placement of the replicas of a CB-msg3 within the available resources, and the use of an implicit pointer that – without cost in terms of overhead – allows the network to infer the positions of the replicas upon decoding one of them in order to avoid unnecessary usage of downlink resources.

- Discussion

As noted in recent contributions, e.g., [3-6], the performance of Msg3-EDT transmission in NB-IoT can be substantially improved at low to medium channel loads if multiple message copies are sent. In particular, the use of solutions inspired by diversity slotted ALOHA (DSA) [7] requires minor modifications to the specifications with no relevant impact on RAN1, and has been shown to provide substantial throughput gains for BLER values of interest [8].

Considering these aspects, it was agreed during RAN2#127bis to introduce DSA support for EDT in NB-IoT. Accordingly, when specified by the network configuration, a UE has the possibility to send multiple replicas of its CB-msg3 over a certain number of allocated EDT occasions.

To do so, the UE has to determine which occasions to use. In this respect, it is important that the selection procedure is specified, and follows a randomization criterion, reducing the risk of repeated collision among multiple terminals selecting the same subset of resources [7].

Observation 1: When DSA is supported for CB-msg3 transmissions, UEs shall pick the EDT occasions over which to send their replicas randomly.

Moreover, when DSA is used, it can be beneficial for the receiver to be aware of the positions of all the replicas sent by a UE as soon as one of them is decoded.

In the first place, this would allow the gNB to properly handle acknowledgements, e.g, providing feedback immediately after the first replica is decoded, and avoiding transmission of multiple acknowledgements for the other replicas of the same UE. This will ensure that downlink resources are not unnecessary wasted and will avoid unexpected behavior in the UE when receiving multiple Msg4 acknowledgements.

Helping the network determine locations of other DSA replicas may also have other positive effects. Consider for instance the case in which the gNB has decoded one DSA message from UE1 and one from UE2, and is able to understand that both of them have also picked the same EDT opportunity Oc to send another replica. By knowing this, the gNB can already expect that a collision among at least UE1 and UE2 takes place over that resource. Thus, it could prioritize decoding over other EDT occasions, or smartly handle a retransmission request by correctly interpreting the decoding failure over Oc as due to collision of at least two already retrieved users and not to, e.g., poor channel conditions.

Observation 2: Allowing the gNB to determine the positions of the DSA replicas sent by a UE upon decoding one of them can lead to resource savings in the downlink (e.g., avoid multiple unnecessary Msg4 acknowledgements).

An effective approach to make the gNB aware of the EDT resources selected by a UE to send its DSA replicas as soon as one of them is decoded consists in the use of an implicit pointer. The solution does not require any additional signaling from the network side to the UE, and does not trigger any modification or overhead cost in the CB-msg3 sent by the UEs.

The idea consists in using information that is unique to the UE about to transmit a CB-msg3 via DSA as the seed of a pseudo-random number generator responsible to output the positions of the replicas. At the receiver side, the decoding of one of the replicas allows to infer the seed and thus to obtain the other EDT occasions employed by the UE by running the same algorithm.

Observation 3: The use of an implicit pointer can make the gNB aware of the positions of the DSA replicas sent by the UE as soon as one of them is decoded. The solution requires no modification to the CB-msg3 and entails no cost in terms of overhead.

A basic example of how the algorithm could work is as follows, assuming that num_replicas copies have to be sent over a set of num_resources EDT occasions.

- The UE applies a hash function (e.g., CRC) to (a portion of) the payload of the CB-msg3 it wants to send vis DSA, obtaining an unsigned integer

- The value is used as seed for a pseudo-random number generator (RNG), e.g., a simple linear congruential generator

- The RNG outputs num_replicas pseudo-random values, each of which is mapped to one of the num_resources available EDT occasions, e.g., taking the modulo num_resources operator.

It shall be noted that the algorithm may provide multiple times the same location, which in turn cannot be used for different replicas. In this case, the function can be simply be called iteratively until a different location for the replica is obtained.

Assume now that the gNB decodes one DSA replica of a UE in an EDT occasion. Using the retrieved message payload, the gNB can run the same procedure, thus obtaining the positions used by the UE to send all its other replicas.

The reported solution provides the required randomization in replica placement (see observation 1), and can be used to obtain an implicit pointer by the gNB. Variations of the approach are possible, for instance considering a concatenation of the UE id or other parameters such as the frame ID instead of a hash function of portion of the CB-msg3 payload.

Proposal 1: RAN2 to consider the use of an implicit pointer to allow the network infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions.

- Conclusion

Following observations and proposals are made:

Observation 1: When DSA is supported for CB-msg3 transmissions, UEs shall pick the EDT occasions over which to send their replicas randomly.

Observation 2: Allowing the gNB to determine the positions of the DSA replicas sent by a UE upon decoding one of them can lead to resource savings in the downlink (e.g., avoid multiple unnecessary Msg4 acknowledgements).

Observation 3: The use of an implicit pointer can make the gNB aware of the positions of the DSA replicas sent by the UE as soon as one of them is decoded. The solution requires no modification to the CB-msg3 and entails no cost in terms of overhead.

Proposal 1: RAN2 to consider the use of an implicit pointer to allow the network infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions.

- References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

[2] R2-2405202, “Discussion on EDT enhancements”, ESA, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan, May 2024.

[3] R2-2406593, “Discussion on DSA and CRDSA Performance,” 3GPP TSG-RAN WG2 Meeting #127, Maastricht, the Netherlands, Aug. 2024.

[4] R2-2406593, “Performance of Advanced Random Access Protocols,” 3GPP TSG-RAN WG2 Meeting #127, Maastricht, the Netherlands, Aug. 2024.

[5] R2-2408547, “Repetitions and Delay Considerations about SA and DSA,” 3GPP TSG-RAN WG2 Meeting#127-bis, Hefei, China, Oct. 2024.

[6] R2-2408863, “Discussion on throughput and delay performance of slotted ALOHA and diversity slotted ALOHA”, 3GPP TSG-RAN WG2 Meeting#127-bis, Hefei, China, Oct. 2024.

[7] G. Choudhury abd S. Rappaport, “Diversity ALOHA—A Random Access Scheme for Satellite Communications,” in IEEE Transactions on Communications, Vol. COM-31, No. 3, pp. 450-457, Mar. 1983.

[8] R2-2409231,	“Report of [AT127bis][301][R19 IoT NTN] Working point for CB-msg3,” 3GPP TSG-RAN WG2 Meeting#127-bis, Hefei, China, Oct. 2024.