3GPP TSG-RAN WG2 #128 	R2-2410882

Orlando, USA, 18 - 22 November 2024

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was updated [1], however, the UL capacity objective and justification is the same as before.

The main objective related to support of capacity enhancements for uplink are:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

And the justification for this objective is:

Need for Uplink capacity enhancement: NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified. Therefore, in order to unlock the additional UL capacity potential, there is a need to identify methods to de-couple the UL from the DL as much as possible. Apart from enhancements above, reducing the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction could be beneficial, therefore should be studied and if beneficial, specified.

In this contribution we look further into this objective.

# 2	Discussion

## 2.1	Main characteristics and Terminology

The main characteristics of UL capacity enhancements will likely be:

- UEs in IDLE with a fresh GNSS measurement may use this new functionality, for example, based on information received in broadcasting, UE implementation, and UE situation at the moment (for example UE buffer size)

- A first message UE->eNB is sent on a contention-based resource, this message is likely to be similar to msg3 in EDT

- A reply message eNB-> UE is sent to one UE on the regular (N)PDSCH (possibly replies to many UEs can be combined for saving PDCCH resources), this message is likely to be similar to msgB for 2-step RA in NR

- UEs that do not get a reply message within a reasonable time will (after random backoff?) need to retransmit

- UEs that fail too many times most likely shall stop using the contention based channel

- There must be a mechanism to control the load on the contention-based channel, for example some kind of broadcasted back off

During RAN2#127bis, there was a statement captured in the chair notes:

We use the term CB-msg3 EDT for now

However, we believe this term is confusing as it gives the impression that this is EDT – but it is likely to be a separate functionality that has similarities with EDT but also similarities with 2-step Random Access in NR.

Therefore, we propose to give it a name that do not contain the word EDT nor PUR to avoid future misunderstandings – this will make it easier to refer to the functionality.

The first message is similar to msgA in 2-step RA in NR (without the preamble part) but especially the second message will be similar to msgB in 2-step RA in NR. We suggest using similar names but with a prefix:

The new contention-based EDT like functionality is called “contention-based data transmission” or CBDT

The first message from UE to eNB is called “contention-based message A” or CB-msgA

The reply to CB-msgA from eNB to UE is called “contention-based message B” or CB-msgB

## 2.2	Further analysis of UL capacity enhancements and DSA

This is the second sub-objective of the UL capacity enhancements.

We consider some new issues.

### 2.2.1	Standardized 5QIs for NTNs

In 23.501 section 5.7 the standardized 5QIs are defined, among them we have one for NTN:

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Non-GBR (NOTE 1) | 90 | 1100ms (NOTE 10,NOTE 13, NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 10: In RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms,~21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |  |

In NOTE 1 above, it is clear delayed packets shall not be included in the PER, further for PDB we have:

Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER.

In the table we see that, for 5QI 10, the PDB is 1100 ms where of 1080 ms is for RAN and 20 ms for CN. Further, an acceptable PER is 10^-6.

This means from an QoS point of view, it is acceptable if 2% or less of the packets experience a delay above 1080 ms in RAN.

In GEO, a delay of 1080 ms is one first transmission and two retransmissions if the UE has UL resources every TTI for a first transmission and there is no scheduling delay, or one first transmission and one retransmission if the UE need to send SR and wait for a grant before the first transmission.

What ever service an operator target, there must be an acceptable PDB and acceptable number of packet arriving late, and an acceptable level of packets lost.

For example, if we assume that the QoS requirements allow for two attempts for each packet, then we must have a fail rate for each transmission at 10^-3 or better to achieve a PER of 10^-6 or better.

(Actually, much higher fail rate is acceptable if it happens for less than 2% of the packet attempts as the PDB definition allows 2% of packets to have longer delay.)

Each service shall have an acceptable packet delay budget and an acceptable packet error rate, for example the standardized 5GI 10, has an RAN PDB of 1080 ms and a PER of 10^-6.

### 2.2.2	Refined analysis of DSA performance

In the models described in R2-2405202 [3] it is assumed that if there is no packet collision, then the packet is always correctly decoded. Here we extend that model by assuming that a non-colliding packet has a BLER.

The BLER will depend on the available resources (number of repetitions, frequency, time and transmit power) and the propagation channel and the interference. The link adaptation can have a target BLER, in terrestrial network for example 10% is common in evaluations – but for the reasons discussed last meeting, to not increase delay and to not waste transmit power on one packet, in NTNs we may need a much lower target BLER.

We assume same models as in [3], that is a slotted system with Poisson arrivals, colliding packets always fails, but we add that a non-colliding packet fail with probability BLER. In this case we define success as a packet that is non-colliding and do not have a block error, the throughput is the same but instead of packet loss rate we use the total failure rate (the probability that no packet gets through correct, that is either all packets collide or the non-colliding packets has a block error).

The results are plotted on the next two pages.

For the low collision rates we are interested, DSA schemes may have more than one replica that do not collide – and if each of the non-colliding replicas has a BLER – incorrect packet is only if all non-colliding replicas have a block error. Thus the diversity of Diversity Slotted Aloha also can help against the block errors!

Obviously for throughput, the SA scheme can be scaled by the probability of 1 – BLER, while for DSA the effect of BLER on throughput will be less.

For SA, the BLER sets a floor for the total failure rate (total failure cannot be lower than the floor), while for 2 replicas DAS the BLER^2 is the floor of total failure rate and for 3 replicas the floor is BLER^3.

For SA, DSA with 2 replicas (2r) and DSA with 3 replicas (3r) with and without BLER we have

And the same data plotted for a certain BLER:

The throughput is not greatly affected by adding the block errors, but total failure rate is.

The inclusion of BLER in the analysis of DSA do not greatly change the throughput but increases the probability of packet failure.

## 2.4	Configuration of uplink resources

Last meeting RAN2 agreed:

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

For the configuration, we can likely reuse how (N)PRACH occasions are configured,

For example, for NB-IoT FDD we have indicated the parameters that can likely be used in a CBDT-Config:

NPRACH-ConfigSIB-NB-r13 ::=			SEQUENCE {

nprach-CP-Length-r13				ENUMERATED {us66dot7, us266dot7},

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsNPRACH-InfoList-NB-r13	OPTIONAL,	-- Need OR

nprach-ParametersList-r13		NPRACH-ParametersList-NB-r13

}

NPRACH-Parameters-NB-r14 ::=			SEQUENCE {

nprach-Parameters-r14					SEQUENCE {

nprach-Periodicity-r14					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560}

OPTIONAL,	-- NEED OP

nprach-StartTime-r14					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r14				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r14				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r14	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r14			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r14				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r14					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r14		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r14					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

RAN2 to consider the existing (N)PRACH configuration as a starting point for the CB-msgA configuration.

## 2.5	Mechanisms to implement DSA

In this section we discuss the required mechanisms to properly implement DSA in IoT-NTN.

### 2.5.1	CB-msgA (msg3) PUSCH resource configuration

There is an FFS from RAN2#127bis on whether we have a CE level specific configuration for DSA. In our view, repetition is also needed for DSA as the number of repetitions for (N)PUSCH can be up to 128 while the number of replicas for DSA is typically no more than 3, which is far from enough to provide sufficient reliability in challenging coverage conditions. On the other hand, there is no need to configure separate CB-msgA (N)PUSCH resources for DSA. In L1, each DSA replica is treated independently, and it is transparent to the NW whether a DSA replica is for the same or different packet. A UE applying DSA can select the CE level for CB-msgA (N)PUSCH resources based on both the RSRP selection threshold and the number of replicas, once the CE level is determined, the UE transmits CB-msgA in (N)PUSCH resources with the corresponding CE level and the NW does not need to know whether or not DSA has been applied.

There is no need to configure separate CB-msgA (N)PUSCH resources for DSA.

A UE applying DSA can select the CE level for CB-msgA (N)PUSCH resources based on both the RSRP selection threshold and the number of replicas.

The parameters needed for DSA are 1) a window length (for example in terms of CB-msgA occasions) 2) the number of replicas (for example 1, 2, or 3)

### 2.5.2	UE identity for scheduling CB-msgB (Msg4) transmission

It was agreed in RAN2#127bis that the RNTI used at least to schedule CB-msgB transmission is derived “based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission” and FFS how this is impacted by DSA. In our view, the RNTI should be derived the same way regardless of whether DSA is applied or not as each DSA replica is treated independently in L1. A UE transmitting multiple replicas may receive multiple CB-msgB and each is scheduled with a different RNTI derived based on resource associated to the PUSCH occasion used to transmit the corresponding replica.

the RNTI used to schedule CB-msgB (Msg4) transmission should be derived the same way regardless of whether DSA is applied or not.

Further to, decrease the need for replying with multiple CB-msgB, users that select different preambles (subcarriers) shall monitor for the ame RNTI, that is the RNTI shall not be dependent on the subcarrier used.

the RNTI used to schedule CB-msgB (Msg4) transmission should not depend on the subcarrier used for sending CB-msgA

If a UE has transmitted multiple replicas (for the same packet) before receiving CB-msgB, the UE can skip monitoring CB-msgB once it receives a CB-msgB which contains a matching Contention Resolution Identity.

The UE may skip monitoring CB-msgB (Msg4) once it receives a CB-msgB containing a matching Contention Resolution Identity.

Besides, if the UE receives a CB-msgB containing a matching Contention Resolution Identity and the number of already transmitted replicas is less than the configured maximum number, the UE need not to transmit the remaining replicas.

A UE may stop transmitting the remaining replicas if it has received a CB-msgB (msg4) containing a matching Contention Resolution Identity.

## 2.6	Design of CB-msgB for decreased DL signalling

The second WI sub-objective of this topic is the efficient delivery (reduced overhead) of Msg4 (CB-msgB ). In RAN2#126 meeting, [2] showed how the overhead reduction cannot come from optimizations in the RRCEarlyDataComplete ASN.1 structure. Thus, RAN2 should focus on different alternatives such as the transmission of a shared CB-msgB .

If multiple UEs can be acknowledged in one CB-msgB, we will decrease delay and we will use much less (N)PDCCH resources.

In NR 2-step random access, MsgB can consist of a Backoff Indicator and one or multiple successRAR and/or fallbackRAR sub-PDUs. Each successRAR consists of UE Contention Resolution Identity, and some more fields. Hence, a MsgB can be used to respond to multiple UEs which have transmitted MsgA, i.e., a shared MsgB.

NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

In NR 2-step RA we have (from NR MAC spec):

Figure 6.1.5a-4: Example of a MSGB MAC PDU with MAC SDU(s)

Figure 6.1.5a-5: Example of a MSGB MAC PDU without MAC SDU(s)

In NR 2-step RA, the NR successRAR looks like this (from NR MAC)

Figure 6.2.3a-2: successRAR

A similar design can be applied to CB-msgB, i.e., CB-msgB can consist of a Backoff Indicator and one or multiple contention resolution identities. The other fields in the successRAR are not needed, except maybe if we implement HARQ feedback for CB-msgB then “HARQ feedback Timing Indicator”, “TPC” and “PUCCH Resource Indicator” can be of interest from successRAR. Possibly, we may also support one (or more?)  MAC subheader plus MAC SDU for CCCH, DCCH or DTCH in order to support mobile-terminated EDT.

The contention resolution Identity can reuse the “UE Contention Resolution Identity MAC Control Element” that is derived from the CCCH (that is from RRCConnectionResumeRequest or RRCEarlyDataRequest depending on if UP-EDT or CP-EDT is configured for CBDT or a new RRC message if that is decied).

This avoids the transmission of separate CB-msgB for each individual UE whose CB-msgA transmission is decoded successfully.

Introduce multiplexing of several UE Contention Resolution Identities in CB-msgB using NR msgB as baseline.

Introduce a Backoff Indicator in CB-msgB.

Discuss whether CBDT needs to support mobile terminated EDT.

In legacy CP-EDT, msg3 will have a UE identity in the RRC message:

RRCEarlyDataRequest-r15-IEs ::=     SEQUENCE {

s-TMSI-r15                                S-TMSI,

…

For UP-EDT

RRCConnectionResumeRequest-r13-IEs ::=          SEQUENCE {

resumeIdentity-r13                                                CHOICE {

resumeID-r13                                                      ResumeIdentity-r13,

Then in CB-msg4, the UE can repeat part of the RRC message in a UE Contention Resolution Identity MAC Control Element used for the contention resolution (from LTE MAC):

#### 6.1.3.4         UE Contention Resolution Identity MAC Control Element

The UE Contention Resolution Identity MAC control element is identified by MAC PDU subheader with LCID as specified in table 6.2.1-1. This control element has a fixed 48-bit size and consists of a single field defined as follows (figure 6.1.3.4-1)

-    UE Contention Resolution Identity: If this MAC control element is included in response to an uplink CCCH transmission, then this field contains the uplink CCCH SDU if the uplink CCCH SDU is 48 bits long. If the CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the uplink CCCH SDU. If this MAC control element is included in response to an uplink DCCH transmission (i.e. the MAC entity is configured with rach-Skip or rach-SkipSCG), then the MAC entity shall ignore the contents of this field.

Figure 6.1.3.4-1: UE Contention Resolution Identity MAC control element

The CB-msgB contention resolution uses legacy UE Contention Resolution Identity MAC Control Element for the contention resolution.

## 2.7	Contention mechanism: collision detection and backoff

CB-msgA transmission may fail due to collision or bad reception quality at the network side. In NR 2-step random access, when the transmission of MsgA fails, the UE may have another MsgA transmission attempt with a random backoff and an increased MsgA transmission power (i.e., power ramping). The UE would switch to 4-step random access if the number of MsgA transmissions has reached msgA-TransMax, provided that preambleTransMax is greater than msgA-TransMax. A similar mechanism can be introduced for contention based Msg3 EDT transmission to handle Msg3 transmission failures.

After a failure in CB-msgA transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power. FFS parameters and details of the mechanism.

After a number of failed CB-msgA transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

## 2.8	Contention control

When CBDT is applied, the UE shall start another round of attempts for CB-msgA transmission (after waiting for a random back-off) only if all the replicas in the current round have been transmitted and none of them is acknowledged by the NW.

For CBDT the UE shall only start another round of attempts for CB-msgA transmission when all the replicas in the current round have been transmitted and no acknowledgement have been received.

NR 2-step random access introduces fallbackRAR where the NW could issue a UL grant to retransmit MsgA MAC PDU in Msg3. Similar mechanism can be introduced for CB-msgA retransmission, i.e., CB-msgB could indicate a set of (N)PUSCH resources for CB-msgA retransmission and the resources can be separate from the CB-msgA (N)PUSCH resources which is up to NW implementation, a UE receiving the CB-msgB which does not contain the matching Contention Resolution Identity randomly select one resource from the set of (N)PUSCH resources to transmit Msg3 again.

Moreover, RAN2 agreed to continue the work on CB-msgA assuming that OCC2 might also apply to CB-msgA transmission. In this case CB-msgB may also indicate for which OCC the (N)PUSCH resources can be used for CB-msgA retransmission, and the UE should only select the (N)PUSCH resources to retransmit CB-msgA if it has transmitted CB-msgA with the indicated OCC.

CB-msgB could indicate a set of (N)PUSCH resources for CB-msgA retransmission which may be separate from the CB-msgA (N)PUSCH resources.

When OCC is applied CB-msgB may indicate for which OCC the (N)PUSCH resources can be used for CB-msgA retransmission.

These proposals allows for a dynamic handling of the UL resources such that when load increases on the CBDT, more resources can easily be added to decrease the collisions, without the need for SIB updates or waiting until all users have read the updated system information.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Each service shall have an acceptable packet delay budget and an acceptable packet error rate, for example the standardized 5GI 10, has an RAN PDB of 1080 ms and a PER of 10^-6.

Observation 2	The inclusion of BLER in the analysis of DSA do not greatly change the throughput but increases the probability of packet failure.

Observation 3	NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The new contention-based EDT like functionality is called “contention-based data transmission” or CBDT

Proposal 2	The first message from UE to eNB is called “contention-based message A” or CB-msgA

Proposal 3	The reply to CB-msgA from eNB to UE is called “contention-based message B” or CB-msgB

Proposal 4	RAN2 to consider the existing (N)PRACH configuration as a starting point for the CB-msgA configuration.

Proposal 5	There is no need to configure separate CB-msgA (N)PUSCH resources for DSA.

Proposal 6	A UE applying DSA can select the CE level for CB-msgA (N)PUSCH resources based on both the RSRP selection threshold and the number of replicas.

Proposal 7	The parameters needed for DSA are 1) a window length (for example in terms of CB-msgA occasions) 2) the number of replicas (for example 1, 2, or 3)

Proposal 8	the RNTI used to schedule CB-msgB (Msg4) transmission should be derived the same way regardless of whether DSA is applied or not.

Proposal 9	the RNTI used to schedule CB-msgB (Msg4) transmission should not depend on the subcarrier used for sending CB-msgA

Proposal 10	The UE may skip monitoring CB-msgB (Msg4) once it receives a CB-msgB containing a matching Contention Resolution Identity.

Proposal 11	A UE may stop transmitting the remaining replicas if it has received a CB-msgB (msg4) containing a matching Contention Resolution Identity.

Proposal 12	Introduce multiplexing of several UE Contention Resolution Identities in CB-msgB using NR msgB as baseline.

Proposal 13	Introduce a Backoff Indicator in CB-msgB.

Proposal 14	Discuss whether CBDT needs to support mobile terminated EDT.

Proposal 15	The CB-msgB contention resolution uses legacy UE Contention Resolution Identity MAC Control Element for the contention resolution.

Proposal 16	After a failure in CB-msgA transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power. FFS parameters and details of the mechanism.

Proposal 17	After a number of failed CB-msgA transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

Proposal 18	For CBDT the UE shall only start another round of attempts for CB-msgA transmission when all the replicas in the current round have been transmitted and no acknowledgement have been received.

Proposal 19	CB-msgB could indicate a set of (N)PUSCH resources for CB-msgA retransmission which may be separate from the CB-msgA (N)PUSCH resources.

Proposal 20	When OCC is applied CB-msgB may indicate for which OCC the (N)PUSCH resources can be used for CB-msgA retransmission.

# 4	References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc., RAN#105, Sept. 2024

R2-2405201, “Efficient delivery of RRCEarlyDataComplete message”, TOYOTA Info Technology Center, RAN2#126, May, 2024.

R2-2405202, “Discussion on EDT enhancements”, ESA, RAN2#126 Fukuoka, May 2024.

R1- 2407548, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, From: RAN1 To: RAN2; RAN4, RAN1#118 Aug. 2024.

R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, From: RAN4, To: RAN2, Cc: RAN1, RAN4#112, Aug. 2024.

# Appendix A	LS from RAN1 and RAN4

RAN2 had a reply LS from RAN1 [4]:

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the response into account. To RAN4: ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

And this reply from RAN4 [5]:

# 1. Overall Description: RAN4 would like to thank RAN2 for the LS R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN2 asked for RAN4 following questions: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP. # 2. Actions: RAN4 respectfully asks RAN2 to take the above information into account. # 3. Date of Next TSG-WG4 Meetings: TSG-WG4#112bis from 2024-10-14 to 2024-10-18 Hefei, China TSG-WG4#113 from 2024-11-18 to 2024-11-22 Orlando, US # A. Annex A: UE transmit timing for Category M1 for Satellite Access (TS36.133) 7.20A.2 Requirements The UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. Note 1: TS is the basic timing unit defined in TS 36.211. # A. Annex B: UE transmit timing for NB-IoT for Satellite Access (TS36.133) 7.24A.2 Requirements The UE initial transmission timing error shall be less than or equal to ±Te_NTN_M1 where the timing error limit value Te_NTN_M1 is specified in Table 7.24A.2-1. This requirement applies when it is the first transmission in a DRX cycle, eDRX_CONN cycle, or the first transmission in a repetition period (R>1) for PUCCH, PUSCH, and SRS, or the first transmission after an uplink transmission gap in a repetition period (R>1) for PUCCH or PUSCH, or it is the PRACH transmission, or it is the transmission on PUR, or the first transmission in each segment of PUSCH/PUCCH for eMTC in a repetition period. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving cell minus. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving cell. NTA_Ref for PRACH is defined as 0. (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA_Ref for other channels is not changed until the next timing advance is received. Table 7.24A.2-1: Te_NTN_M1 Timing Error Limit CE Mode; Te_NTN_M1_. A; 41*TS. B; 65*Ts. Note 1: TS is the basic timing unit defined in TS 36.211. Note 2: This requirement applies regardless of the downlink carrier bandwidth..

# Appendix B	RAN2 agreements

## B.1	RAN2#125bis

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

## B.2	RAN2#126

1 RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

## B.3	RAN2#127

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

## B.4	RAN2#127bis

In evaluation of possible methods to improve UL capacity for CB-msg3 ETD-like transmission we focus on a working point where the PLR is lower than 10% (this does not mean that we are mandating any network to work at any working point)

We use the term CB-msg3 EDT for now

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.