3GPP TSG-RAN WG2 Meeting #128                                                          R2-2410890

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:   8.2.4

Source:             Wiliot Ltd.

Title:                 A-IoT Random Access - failure/success feedback indication for following D2R data

Document for: Discussion and Decision

## Introduction

In RAN2#127bis, the following initial agreements were made on Random Access [1]

RAN #127bis Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

This document details an analysis of the 2step vs 3step Random Access failure/success feedback indication for following D2R data.  For each case the impact of implementing an Ack based protocol, a Nack based protocol, or a no response protocol is presented.  The analysis assumed impairment and interference.

## Analysis

### 2.1 Analysis Introduction

We present an analysis for inventory completion time for type 2b RF-based EH devices, at InH-Office scenario.

For the inventory time analysis, we assume that a single Reader and single Energy Source are active at a given point in time.

The following deployment is analyzed:

| Deployment | Reader coverage radius | Device distribution | Number of devices in coverage range | Energizer | Comment |
| --- | --- | --- | --- | --- | --- |
| Deployment | 14.14m | Uniformly distribution of 1.5 devices per m2 | 943 | 30dBm, 100% duty-cycle, 900MHz | Required coverage when BS positioned on square lattice with D=20m in RAN1#116bis |

Table 1: Deployment

Using the following paging protocol:

- Paging Command: A downlink command issued from the Reader to the Device to trigger and configure an inventory session

- Round Command: A downlink command issued from the Reader to the Device to signal the start and configure a round.  Rounds consist of occasions where upon receiving a round command, devices that are energized and are permitted to participate in the given round randomly select an occasion to transmit the device’s uplink payload to the Reader.  The number of occasions in the round are indicated in the Round command.  The Round command is issued following the Paging command, where several rounds may be issued to facilitate the inventory session

- Occasion Indication: A downlink indication issued from the Reader to the Device to signal the start of an occasion.  The Occasions indication includes the occasion’s index, enabling the Device to sync to the select occasion selected upon receiving the Round command

the following acknowledgement-based protocol schemes for the Device to Reader communication in the select occasion are analyzed:

| Idx | Ack-based protocol | Description |
| --- | --- | --- |
| 1 | 2-step, Ack based | MSG1: Device transmits to Reader a packet with the RN16 and device ID MSG2: Reader transmits to Device an ACK if packet successful received at Reader |
| 2 | 2-step, Nack based | MSG1: Device transmits to Reader a packet with the RN16 and device ID MSG2: Reader transmits to Device a NACK if packet unsuccessfully received at Reader |
| 3 | 2-step, NoAck | MSG1: Device transmits to Reader a packet with the RN16 and device ID |
| 4 | 3-step, Ack based | MSG1: Device transmits to Reader a packet with the RN16 MSG2: Reader transmits to Device an ACK if packet successfully received at Reader MSG3: Device transmits to Reader a packet with the device ID MSG4: Reader transmits to Device an ACK if packet successful received at Reader |
| 5 | 3-step, Nack based | MSG1: Device transmits to Reader a packet with the RN16 MSG2: Reader transmits to Device an ACK if packet successfully received at Reader MSG3: Device transmits to Reader a packet with the device ID MSG4: Reader transmits to Device a NACK if packet unsuccessfully received at Reader |
| 6 | 3-step, NoAck | MSG1: Device transmits to Reader a packet with the RN16 MSG2: Reader transmits to Device an ACK if packet successfully received at Reader MSG3: Device transmits to Reader a packet with the device ID |

Table 2: Acknowledgement-based protocol schemes

### 2.2 Random Access

The round messaging between the AIoT Reader and AIoT Device based on the 3-step signaling procedure is as follows.  The abovementioned Acknowledgement-based protocol schemes (Table 2) are a subset thereof:

We assess the timing using the following packet structure for Reader to Device (R2D) and Device to Reader (D2R) packets:

#### 2.2.1 Occasion Duration

To analyze the time for execution of the RA protocol we assume for this analysis the following durations.

R2D messages:

| Duration Label | Message | StartInd [us] | SyncField [bits] | Payload [bits] | Duration [us] (252[kbps]) |
| --- | --- | --- | --- | --- | --- |
| T_PagingCmd | PagingCmd | 28 | 16 | 40 | 250.22 |
| T_RndCmd | RoundCmd | 28 | 16 | 22 | 178.79 |
| T_OccInd | OccasionInd | 28 | 16 | 18 | 162.92 |
| T_Ack | Ack | 28 | 16 | 18 | 162.92 |

Table 3: R2D message durations

D2R messages:

| Duration Label | Message | StartInd [us] | SyncField [bits] | Payload [bits] | Duration [us] (960[kbps]) |
| --- | --- | --- | --- | --- | --- |
| T_RN16 | RN16 | 0 | 8 | 16 | 25 |
| T_DeviceID | DeviceID | 0 | 8 | 96 | 108.33 |
| T_RN16AndDeviceID | RN16AndDeviceID | 0 | 8 | 112 | 125 |

Table 4: D2R message durations

Gap durations:

| Duration Label | Description | [us] |
| --- | --- | --- |
| T_ReaderTxRx | The time for Reader to move from Tx to Rx and for the Device to move from Rx to Tx | 25 |
| T_ReaderRxTx | The time for Reader to move from Rx to Tx and for the Device to move from Tx to Rx | 25 |

Table 5: Gap durations

We now assess the occasion durations for different cases:

Cases for no acknowledgement: 2-step

| Occasion Duration case | Message | Summation of Duration Labels | Duration [us] |
| --- | --- | --- | --- |
| Empty Occasion | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Collision Occasion | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Payload Failure | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Payload Success | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |

Table 6: 2-step NoAck based Occasion Duration per case

Cases for no acknowledgement: 3-step

| Occasion Duration case | Message | Summation of Duration Labels | Duration [us] |
| --- | --- | --- | --- |
| Empty Occasion | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Collision Occasion | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Random Access Failure | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Random Access Ack Failure | RN16+ Ack | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_ReaderRxTx | 263 |
| Payload Failure | RN16+ Ack+ DeviceID | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_DeviceID+ T_ReaderRxTx | 371 |
| Payload Success | RN16+ Ack+ DeviceID+ Ack | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_DeviceID+ T_ReaderRxTx | 371 |

Table 7: 3-step NoAck based Occasion Duration per case

Cases for Ack-based: 2-step

| Occasion Duration case | Message | Summation of Duration Labels | Duration [us] |
| --- | --- | --- | --- |
| Empty Occasion | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Collision Occasion | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Payload Failure | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Payload Success | RN16AndDeviceID+ Ack | T_RN16AndDeviceID+ T_ReaderRxTx+ T_Ack | 313 |

Table 8: 2-step Ack based Occasion Duration per case

Cases for Ack-based: 3-step

| Occasion Duration case | Messages | Summation of Duration Labels | Duration [us] |
| --- | --- | --- | --- |
| Empty Occasion | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Collision Occasion | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Random Access Failure | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Random Access Ack Failure | RN16+ Ack | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_ReaderRxTx | 263 |
| Payload Failure | RN16+ Ack+ DeviceID | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_DeviceID+ T_ReaderRxTx | 371 |
| Payload Success | RN16+ Ack+ DeviceID+ Ack | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_DeviceID+ T_ReaderRxTx+ T_Ack | 534 |

Table 9: 3-step Ack based Occasion Duration per case

Cases for Nack-based: 2-step

| Occasion Duration case | Message | Summation of Duration Labels | Duration [us] |
| --- | --- | --- | --- |
| Empty Occasion | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Collision Occasion | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |
| Payload Failure | RN16AndDeviceID+ Ack | T_RN16AndDeviceID+ T_ReaderRxTx+ T_Ack+ | 313 |
| Payload Success | RN16AndDeviceID | T_RN16AndDeviceID+ T_ReaderRxTx | 150 |

Table 10: 2-step Nack based Occasion Duration per case

Cases for Nack-based: 3-step

| Occasion Duration case | Message | Summation of Duration Labels | Duration [us] |
| --- | --- | --- | --- |
| Empty Occasion | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Collision Occasion | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Random Access Failure | RN16 | T_RN16+ T_ReaderRxTx | 50 |
| Random Access Ack Failure | RN16+ Ack | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_ReaderRxTx | 263 |
| Payload Failure | RN16+ Ack+ DeviceID+ Ack | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_DeviceID+ T_ReaderRxTx+ T_Ack | 534 |
| Payload Success | RN16+ Ack+ DeviceID | T_RN16+ T_ReaderRxTx+ T_Ack+ T_ReaderTxRx+ T_DeviceID+ T_ReaderRxTx | 371 |

Table 11: 3-step Nack based Occasion Duration per case

The above are depicted in the following diagrams, where the duration is [us], RxTx and TxRx and the transmission times between Rx and Tx:

Cases for no acknowledgement: 2-step and 3-step

Figure 1: Occasion duration of RA protocol within a given occasion for no acknowledgement

Cases for Ack-based: 2-step and 3-step

Figure 2: Occasion duration of RA protocol within a given occasion for Ack-based acknowledgement

Cases for Nack-based: 2-step and 3-step

Figure 3: Occasion duration of RA protocol within a given occasion for Nack-based acknowledgement

#### 2.2.2 Performance of the Acknowledgement based protocol schemes

We now assess the performance of the different acknowledgement-based protocol schemes:

The assumed bit-error rate from impairments is as follows:

|  | Value | Comment |
| --- | --- | --- |
| Uplink bit error rate | 1e-3 |  |
| Downlink bit error rate | 1e-2 | Increased bit error rate due to low-powered receiver |

Table 12: Uplink and Downlink bit error rate

The assumed interference is as follows:

|  | Interference packet duration [us] | Interference packet density [%] | Comment |
| --- | --- | --- | --- |
| Uplink interference | 200 | 1 |  |
| Downlink interference | 200 | 2 | Low-power receiver has wider frequency selectivity |

Table 13: Uplink and Downlink interference

The packet error rate is as follows:

| Messages | Uplink/ Downlink | Packet Error Rate from impairments [%] | Packet Error Rate from impairments and interference [%] |
| --- | --- | --- | --- |
| PagingCmd | Downlink | 43.04 | 45.60 |
| RoundCmd | Downlink | 31.74 | 34.33 |
| OccasionInd | Downlink | 28.94 | 31.52 |
| Ack | Downlink | 28.94 | 31.52 |
| RN16 | Uplink | 2.37 | 3.47 |
| DeviceID | Uplink | 9.88 | 11.27 |
| RN16AndDeviceID | Uplink | 11.31 | 12.75 |

Table 14: Uplink and Downlink packet error rate

Based on the mentioned packet error rate the following are the probabilities for different cases of RA protocol within a given occasion assuming no collisions within the given occasion.

The following diagrams using the following legend:

Per case, if a retransmission is performed, then it is stated if the retransmission is:

- Valid: The Device should perform the retransmission as the packet was not received at the Reader

- Redundant: The Device should not perform the retransmission as the packet was received at the Reader.  For this case, the Device redundantly performs the retransmission

- Missed: The Device should perform a retransmission, but doesn’t, as the Nack from the Reader is sent, but is not received correctly, thus the retransmission is not performed.

- None: The Device does correctly not perform a retransmission

Cases for no acknowledgement:

Figure 4: Different cases of RA protocol within a given occasion for no acknowledgement, when assuming no collisions

|  | Case | Probability [%] | Retransmission |
| --- | --- | --- | --- |
| 2-step | RN16AndDeviceID Failure | 12.75 | Valid |
| 2-step | RN16AndDeviceID Success | 87.25 | Redundant |
| 3-step | RN16 Failure | 3.47 | Valid |
| 3-step | RN16 Success + Ack Failure | 30.43 | Valid |
| 3-step | RN16 Success + Ack Success + DeviceID Failure | 7.45 | Valid |
| 3-step | RN16 Success + Ack Success + DeviceID Success | 58.65 | Redundant |

Table 15: Probabilities for different cases of communication protocol within a given occasion for no acknowledgement

Cases for Ack-based:

Figure 5: Different cases of RA protocol within a given occasion for Ack-based acknowledgement, when assuming no collisions

|  | Case | Probability [%] | Retransmission |
| --- | --- | --- | --- |
| 2-step | RN16AndDeviceID Failure | 12.75 | Valid |
| 2-step | RN16AndDeviceID Success + Ack Failure | 27.50 | Redundant |
| 2-step | RN16AndDeviceID Success + Ack Success | 59.75 | None |
| 3-step | RN16 Failure | 3.47 | Valid |
| 3-step | RN16 Success + Ack Failure | 30.43 | Valid |
| 3-step | RN16 Success + Ack Success + DeviceID Failure | 7.45 | Valid |
| 3-step | RN16 Success + Ack Success + DeviceID Success + Ack Failure | 18.49 | Redundant |
| 3-step | RN16 Success + Ack Success + DeviceID Success + Ack Success | 33.90 | None |

Table 16: Probabilities for different cases of communication protocol within a given occasion for Ack-based

Cases for Nack-based:

Figure 6: Different cases of RA protocol within a given occasion for Nack-based acknowledgement, when assuming no collisions

|  | Case | Probability [%] | Retransmission |
| --- | --- | --- | --- |
| 2-step | RN16AndDeviceID Success | 87.25 | None |
| 2-step | RN16AndDeviceID Failure + Nack Failure | 4.02 | Missed |
| 2-step | RN16AndDeviceID Failure + Nack Success | 8.73 | Valid |
| 3-step | RN16 Failure | 3.47 | Valid |
| 3-step | RN16 Success + Ack Failure | 30.43 | Valid |
| 3-step | RN16 Success + Ack Success + DeviceID Success | 58.65 | None |
| 3-step | RN16 Success + Ack Success + DeviceID Failure + Nack Failure | 2.35 | Missed |
| 3-step | RN16 Success + Ack Success + DeviceID Failure + Nack Success | 5.10 | Valid |

Table 17: Probabilities for different cases of communication protocol within a given occasion for Nack-based

#### 2.2.3 Retransmission statistics

Based on the probabilities for different cases of communication protocol within a given occasion, assuming that there are no collisions, the probability of retransmission, valid and redundant and the probability of missed retransmission are as follows:

2-step:

| Ack-based protocol | Retransmissions |  |  | Missed Retransmissions [%] |
| --- | --- | --- | --- | --- |
|  | Total [%] | Valid [%] | Redundant [%] |  |
| None | 100 | 12.75 | 87.25 | 0 |
| Ack | 40.26 | 12.75 | 27.50 | 0 |
| Nack | 8.73 | 8.73 | 0 | 4.02 |

Table 18: Probabilities of retransmission, valid and redundant and probabilities of missed retransmission for 2-step

3-step:

| Ack-based protocol | Retransmissions |  |  | Missed Retransmissions [%] |
| --- | --- | --- | --- | --- |
|  | Total [%] | Valid [%] | Redundant [%] |  |
| None | 100 | 41.35 | 58.65 | 0 |
| Ack | 59.84 | 41.35 | 18.49 | 0 |
| Nack | 39 | 39 | 0 | 2.35 |

Table 19: Probabilities of retransmission, valid and redundant and probabilities of missed retransmission for 3-step

### 2.3 Analysis Parameters

The following parameters used for the MAC RA simulation:

| Parameter | Value | Comment |
| --- | --- | --- |
| Number of devices | 943 | See Table 1 |
| Number of new devices per round | 4 | It is assumed that 4 new devices of the 943 devices are added every round. This is a result of the energizing of the devices and their distance from the energy source. In addition to these new devices, retransmissions from devices from previous rounds also participate in round |
| Number of base occasions per round | 16 | It is assumed that each round has 16 occasions. However, if in the previous round there are 25% or more occasions with bad reception, either from collision, impairment or interference, then the number of occasions in the round are doubled. Once there are less than 25% occasions in the current round with bad reception then the number of occasions is returned to 16 |
| Downlink bitrate | 252kbps |  |
| Uplink bitrate | 960kbps |  |
| Number of sessions repeated | 3 | Defining a session as a paging cycle, then <99% of the devices may be read per session. To overcome this, a session is repeated up to 3 times |
| Max transmissions per device |  | The maximum number of packets that contain the DeviceID. For 2-step this is the maximum number of times MSG1 is sent to the Reader. For 3-step this is the maximum number of times MSG3 is sent to the Reader. |

Table 20: Parameters used for the MAC RA simulation

### 2.4 Results

This section details the results for 2-step/3-step and the different acknowledgement-based methods.

The results represent the accumulated duration to perform the required number of paging sessions to read >99% of the devices.  Per session the duration is calculated by summing the required duration of all performed rounds in the given session.  These results assume that the energizing of the devices are not the limiting factor.

Per round a constant number of new devices are added, joining retransmission of devices from previous rounds.  Prior to each round both the Paging command and Round command are issued as new devices may not have received previous Paging commands due to not being energized.

Results for 2-step: (the optimal results are emphasized in green)

| Ack-based protocol | Max transmissions per device | Accumulated successful transmissions received at reader [%] |  |  | Accumulated total duration for all sessions [ms] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Session 1 | Session 2 | Session 3 | Session 1 | Session 2 | Session 3 |
| None | 3 | 88.12301 | 98.0912 | 100 | 1689.239 | 3348.438 | 5042.684 |
|  | 4 | 90.66808 | 99.46978 | 99.89396 | 1884.931 | 3744.828 | 5604.725 |
| Ack-based | 2 | 84.72959 | 97.77306 | 100 | 1457.86 | 2921.052 | 4383.963 |
|  | 4 | 96.07635 | 99.78791 | 100 | 1611.078 | 3232.882 | 4881.51 |
|  | 6 | 98.72747 | 99.89396 | 100 | 1740.304 | 3490.709 | 5221.146 |
|  | 8 | 99.57582 | 100 | 100 | 1805.817 | 3640.712 | 5458.144 |
| Nack-based | 2 | 84.41145 | 97.34889 | 99.57582 | 1302.875 | 2625.126 | 3935.734 |
|  | 3 | 90.98621 | 99.15164 | 99.89396 | 1333.019 | 2656.736 | 3986.703 |
|  | N/A | 96.39449 | 99.89396 | 100 | 1316.918 | 2649.182 | 3972.899 |

Table 21: Results for 2-step, detailing the max transmissions per device, the accumulated successful transmissions received at reader [%] and the accumulated total duration for all sessions[ms]

Results for 3-step: (the optimal results are emphasized in green)

| Ack-based protocol | Max transmissions per device | Accumulated successful transmissions received at reader [%] |  |  | Accumulated total duration [ms] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Session 1 | Session 2 | Session 3 | Session 1 | Session 2 | Session 3 |
| None | 3 | 73.48887 | 92.57688 | 99.25769 | 1814.169 | 3620.726 | 5445.586 |
|  | 4 | 78.26087 | 95.44008 | 99.57582 | 2091.874 | 4313.141 | 6463.58 |
| Ack-based | 2 | 64.26299 | 88.44115 | 96.39449 | 1525.599 | 3021.257 | 4541.517 |
|  | 4 | 84.5175 | 97.66702 | 99.15164 | 1944.225 | 3871.896 | 5759.437 |
|  | 6 | 91.30435 | 99.57582 | 100 | 2155.59 | 4273.563 | 6485.155 |
|  | No limit | 100 | 100 | 100 | 2723.977 | 5538.698 | 8369.59 |
| Nack based | 2 | 66.80806 | 89.50159 | 95.97031 | 1354.874 | 2723.224 | 4058.29 |
|  | 3 | 78.04878 | 96.07635 | 99.15164 | 1442.423 | 2928.381 | 4431.732 |
|  | No limit | 96.50053 | 99.89396 | 100 | 1737.367 | 3395.502 | 5109.345 |

Table 22: Results for 3-step, detailing the max transmissions per device, the accumulated successful transmissions received at reader [%] and the accumulated total duration for all sessions [ms]

## Conclusions

Observation 1: 2-step Ack-based protocol results in the shortest accumulated total duration time to read >99% of the devices taking 1805.817ms.

Observation 2: 2-step Nack-based protocol results in the shortest accumulated total duration time read >96% of the devices taking 1316.918ms.

Proposal 1: 2-step is recommended over 3-step.

Proposal 2: For read-rate of >99% it is recommended to use an Ack-based protocol.

Proposal 3: For read-rate of >96% it is recommended to use an Nack-based protocol.

## Reference

[1] Draft_RAN2_127b_Meeting_Report_v3.docx