3GPP TSG-RAN2 Meeting #128	R2-24xxxx

Orlando, USA, 18th - 22nd Nov 2024

- Title:	Discussion on MCSt

- Source:	ZTE Corporation, Sanechips

- Agenda Item:	7.6.3

- Document for:	Discussion, Decision

# Introduction

In this paper, we discuss MCSt related issues.

# RAN2 agreement

RAN2 123bis：

5. For a resource pool configured with PSFCH resource, UE can NOT select consecutive slots (i.e., MCSt) for transmissions of a single TB.

RAN2 124：

1. For remaining slot(s) in case transmission is successful for one TB in MCSt (multiple TB case), the UE still performs retransmission for this TB in the remaining slot(s).

# MAC specification

| NOTE 3Ag: When the MAC entity selects the time and frequency resources from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], it is up to the UE implementation whether to randomly select resources for transmission opportunities from the resources indicated by the physical layer or to select resources in consecutive slots by UE implementation from the resources indicated by the physical layer. NOTE 3Ah: For a resource pool configured with PSFCH resource, UE cannot select consecutive slots for SL transmissions of a single TB for Multi-consecutive slots transmission. |  |
| --- | --- |
| 1> if a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.1a, except a positive acknowledgement to Multi-consecutive slots transmission (i.e., multiple TBs case) of the MAC PDU and there is remaining slot(s) for this MAC PDU: 2> clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant. |  |
| 1> if a positive acknowledgement to this transmission of the MAC PDU was received according to clause 5.22.1.3.2, except a positive acknowledgement to Multi-consecutive slots transmission (i.e., multiple TBs case) of the MAC PDU and there is remaining slot(s) for this MAC PDU; or 1> if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for this transmission of the MAC PDU according to clause 5.22.1.3.2: 2> flush the HARQ buffer of the associated Sidelink process. |  |

Observation1：current agreement and specification show that MCSt for multiple TBs is supported in pool with PSFCH.

Observation2：current agreement and specification show that MCSt for single TB is not supported in pool with PSFCH.

# Scenario

## Resource pool with PSFCH

| Case1a: | N = 1 is indicated to PHY layer or no value N is indicated to PHY layer. | Based on MAC implementation, UE only select consecutive slot for single TB |
| --- | --- | --- |
| Case1b: |  | And based on MAC implementation, UE select consecutive slot for single TBs and multiple TBs |
| Case1c: |  | Based on MAC implementation, UE select consecutive slot for only multiple TBs |
| Case2a: | N=3 is indicated to PHY layer | Based on MAC implementation, UE only select consecutive slot for single TB |
| Case2b: |  | UE select MCSt for single TB. Based on MAC implementation, UE select consecutive slot for multiple TBs |

Figure 1: MCSt for pool with PSFCH.

Question1: Check companies view：

| Case | Supported or not supported scenario | Reason |
| --- | --- | --- |
| 1a, 1b, 2a | Not supported. | Since it can not ensure the minimum time gap, current specification already captured this. No specification change is needed for these cases. |

Question2: Check companies view：

| Case | Supported or not supported scenario | Reason |
| --- | --- | --- |
| 2b | FFS | Supported companies: Not supported companies: |

Draft Proposal1: If 2b is not supported, then as long as N>1 is indicated to PHY layer, for a resource pool configured with PSFCH resource, UE can NOT select consecutive slots (i.e., MCSt) for transmissions of a nultiple TBs. Adopt following statement in MAC specification:

NOTE 3Ah:	For a resource pool configured with PSFCH resource, as long as N>1 is indicated to PHY layer, UE cannot select consecutive slots for SL transmissions of a single TB or multiple TBs for Multi-consecutive slots transmission.

Question2: Check companies view：

| Case | Supported or not supported scenario | Reason |
| --- | --- | --- |
| 1c | FFS | Supported companies: Not supported companies: |

Draft Proposal2: If 1c is supported, then as long as N=1 or no value N is indicated to PHY layer, for a resource pool configured with PSFCH resource, UE can select consecutive slots (i.e., MCSt) for transmissions of a nultiple TBs.

## R2-2410073(ZTE)

According to current specification, in case multiple TBs MCSt, UE can keep retransmission if HARQ ACK is received for these TBs, and corresponding HARQ buffer and SL grant will not be flushed or cleared.

However, without flushing or clearing behaviour, HARQ based transmission becomes useless and the system performance will be decreased, event if the retransmission TBs belongs to another set of MCSt resource.

Additionally, the benefit for keeping retransmission is that UE can keep using LBT type2 for retransmission slot within the COT for TBs within MCSt. However, it is possible that the retransmission slots belongs to another COT, for these retransmission slots, keeping re-transmission is not needed.

For example, considering following case:

Figure 2: MCSt retransmission

To solve this issue, we suggest to adopt the R2-2410073.

Draft proposal3: suggest to adopt following statement:

NOTE *: If a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

NOTE *:	If a positive acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may flush the HARQ buffer of the associated Sidelink process.

## Resource pool without PSFCH

Figure 1: MCSt for pool without PSFCH.

I believe above cases are all supported;

Draft proposal4: Update the stage2 specification as following:

A UE using mode 2 resource allocation and operating in a pool of resources without PSFCH configured supports resource selection for multiple consecutive slot transmission (MCSt). A UE using mode 2 resource allocation and operating in a pool of resources with PSFCH configured supports resource selection for multiple consecutive slot transmission (MCSt), if the N=1 or no value N is indicated to PHY layer. A UE autonomously determines whether to use MCSt, and the number of consecutive slots in an MCSt up to the maximum COT duration for a specific SL-CAPC as defined in TS 37.213 [37]. MCSt can be used for transmission of a single TB or multiple TBs. For each TB transmitted in an MCSt, the UE triggers resource (re)selection only when LBT failure is detected on the resources for the initial transmission and all retransmissions of the TB.

# Conclusion

# Reference

R2-2313617	Summary of [AT124][112][V2X/SL] MAC details (LG)	LG	discussion	Rel-18