3GPP TSG-RAN WG2 Meeting #128                                                                                       R2-24xxxxx
Orlando, USA, November 18th – 22nd, 2024

Title:	[Draft] Reply LS on security handling for inter-CU LTM in non-DC cases

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	CATT [To be RAN2]

To:	SA3

Cc:

- Contact Person:	Rui Zhou

- E-mail Address:	zhourui@catt.cn

1. Overall Description:

RAN WG2 thanks SA WG3 for the reply to the RAN2 LS on the security handling for inter-CU LTM in non-DC cases.Regarding the requirement on avoiding RRC signalling between cell switches,RAN2 discussed and would like to provide the response as follows,

RAN2 thinks avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection.

Besides, RAN2 would like to clarify the definition of subsequent inter-CU LTM as follows,

Subsequent inter-CU LTM is done by repeating the early synchronization, inter-CU LTM cell switch execution, and inter-CU LTM cell switch completion steps without releasing, reconfiguring, or adding other LTM candidate configurations after each inter-CU LTM cell switch completion.

2. Actions

To SA3 group

ACTION: 	RAN2 respectfully asks SA3 to take the above RAN2 response into account.For the sake of completing WI,RAN2 expects to receive the feedback from SA3 until RAN2#130(19 - 23 May 2025).

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #129	17 - 21 February 2025	             Athens, GR

TSG-RAN2 Meeting #129bis	07 - 11 April        2025	             China, CN