3GPP TSG RAN WG2#127bis	R2-2407904

Hefei, China, 14th - 18th October 2024

3GPP TSG-CT WG1 Meeting #150	C1-245040

# Maastricht, Netherlands, 19-23 August 2024

# Title:	Reply on LS on the maximum number of devices supported in SLPP

# Response to:	LS on the maximum number of devices supported in SLPP (R2-2405988)

# Release:	Rel-18

# Work Item:	NR_pos_enh2, Ranging_SL

Source:	CT1

To:	RAN2

Cc:	CT4, SA2

Contact Person:

Name:	Yizhong Zhang

Tel. Number:

E-mail Address:	yizhong.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C1-245018

1. Overall Description:

CT1 thanks RAN2’s LS regarding to the maximum number of devices that is supported in the SLPP messages (R2-2405988). CT1 would like to provide the following feedback:

For ranging and sidelink positioning, the information transmitted over NAS via payload container IE has the limitation of a maximum length of 65536 octets from CT1 protocol point of view (the maximum size of a NAS message carrying payload container IE is specified in subclause 7.2.2 of TS 24.501 for further information). The related information includes e.g. SLPP messages, the result of the positioning request. So CT1 finds out that:

- The payload container IE may not be able to encapsulate a complete SLPP message with information for other UEs (UE2-UEn) with the maximum number of devices is 256 as defined by RAN2.

- The payload container IE may also not be able to encapsulate the absolute location result for other UEs (UE2-UEn) with the maximum number of devices is 256.

This limitation also applies to the messages exchanged between UEs.

To address the above issue, for RSPP supplementary messages, CT1 evaluated and achieved the consensus that the supported maximum number of other UEs (UE2-UEn) shall not be greater than 63 in R18 (see attachment), considering the associated information to be exchanged between UEs or between UE1 and LMF for ranging and sidelink positioning.

2. Actions:

To RAN2

ACTION:	CT1 kindly asks RAN2 to take above information into account, update the corresponding specification if needed, and keep CT1 informed of the final decision.

3. Date of Next CT1 Meetings:

CT1#151	14th October- 18th  October 2024	Hefei, CN

CT1#152	18th November- 22nd  November 2024	Orlando, US