3GPP TSG-RAN WG3 Meeting #125                                                              R3-244143

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda item: 		12.3

Source: 		Qualcomm Incorporated

Title: 	(Draft Reply LS to SA2) Discussion on Reply LS to SA2 on Support of UE move between CAG cell of 5G Femto and CSG cell

Document for:		Other

# 1	Introduction

SA2 sent an LS to RAN3 on Support of UE move between CAG cell of 5G Femto and CSG cell in R3-243020/S2-2405813 with the following content:

1. Overall Description: As part of the FS_5G_Femto study (SP-231797), the following aspects are being studied: - WT#1: Based on RAN3 outcome, enhance the overall architecture and enable the required functional and procedural changes for supporting 5G NR Femto deployment. - WT#2: How to enable interworking between CAG and CSG cells. In TR 23.700-45, the WT#2 (mapped to KI#1 in the TR) investigates the support of the UE moving between CAG cell of 5G Femto and CSG cell, with no impact on the RAN. Therefore, some companies proposed below solutions for this KI: - The UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814). - RAN recognizes the target CSG cell (or the target CAG cell) as an open cell during the handover (e.g., via local configuration) and the core network performs access control as described in pCR (S2-2405789). Question 1: SA2 would like to know whether the two solutions mentioned above have any impact on the RAN (e.g., for RAN procedures)? Question 2: SA2 has reserved the time units for the normative work of WT#1 based on the result of RAN3 work (RP-234041), which is expected to start in SA2 from SA2#164. Therefore, SA2 requests to confirm the conclusion of RAN3 on overall architecture, etc., which will be used as the basis for SA2's normative work. 2. Actions: SA2 asks RAN2 and RAN3 to answer the above questions.

This contribution discusses the replies to SA2’s questions and contains a draft LS in the Annex.

# 2	Discussion

On Question 1:

The first solution, which is based on S2-2405814, has RAN impact for the following reasons:

- The solution proposes that the Rel-19 UE at a 5G RAN should incorrectly report the measurement of a 4G cell candidate as that of a 5G cell candidate. Also, a Rel-19 UE at a 4G RAN should incorrectly report the measurement of 5G cell candidate as that of a 4G cell.

- Based on this misrepresentation of cell types, the 5G RAN node may be tempted to apply the intra-system SON Configuration Transfer procedure, e.g., to discover the proclaimed neighbor 5G RAN node for the establishment of Xn. This procedure will fail since the target RAN Node is an eNB and not a gNB.

- The 5G RAN node may further receive conflicting information, i.e., a report from a Rel-19 UE incorrectly indicating the 4G cell as a 5G cell and a report from a legacy UE correctly indicating the 4G cell as a 4G cell. The RAN would have to be able to cope with such conflicting information.

- The above examples indicate that the RAN’s behavior in response to such incorrect reporting would have to be defined via specification.

- RAN3 may want to emphasize that procedures should be followed according to their intended use.

- RAN3 should emphasize that this solution has UE impact while RAN3’s parallel SI on NR Femto explicitly precludes impact on the UE.

Observation 1a: Solution 1 has RAN impact due to the incorrect use of measurement reports, which can cause malfunction of NG-RAN and E-UTRAN.

Observation 1b: Solution 1 has UE impact since the UE needs to perform CSGCAG mapping and use measurement reports in a manner unintended by legacy specification.

The second solution is based on S2-2405789 and is not expected to have RAN or UE impact if designed properly:

- This solution has no RAN impact from RAN3 perspective since access control is performed by the AMF.

- In the last RAN2 and RAN3 meetings, some companies claimed that solution 2 might lead to elevated handover failure rates. This claim was also included in RAN2’s Reply LS to SA2 in R3-244012. The claim for an elevated handover failure rate has been attributed to the more time-intensive procedure to conduct access control at the AMF rather than at the source gNB/eNB. This argument cannot withstand further scrutiny since an equivalent procedure has been defined for 4G S1 handover from eNB to HgNB. In this 4G procedure, access control is conducted at the MME, i.e., in the CN, as opposed to at the source eNB. There have been no concerns or reports on performance degradation for this 4G procedure. Consequently, the equivalent procedure proposed by second solution should work fine.

- For the 5G->4G handover, it is worth emphasizing that the AMF needs to identify the CSG ID and the UE’s Member Ship for this CSG and include this information in the Relocation Request to the MME so that it can be delivered to the target eNB. Solution 2 will have EUTRAN impact if these identifiers are not provided to the target eNB. RAN3 may want to raise this aspect to SA2.

Observation 2a: For 4G  5G handover, Solution 2 has no impact on RAN or UE.

Observation 2b: For 5G  4G handover, Solution 2 has no impact on the UE. It also has no impact on RAN in case the AMF includes the target cells CSG and the UE’s membership ID in the Relocation Request to the MME.

Based on this analysis, RAN3 to reply to SA2 the following:

Proposal 1: On question 1, RAN3 to reply to SA2:

“Reply to question 1:

Solution 1 has both RAN and UE impacts due to the incorrect use of measurement reports, which would cause malfunction of the NG-RAN and E-UTRAN. RAN3 prefers to have UEs follow the procedures as intended by the specification.

Solution 2 has no RAN or UE impact. RAN3 emphasizes that for 5G  4G handover of this solution, the AMF needs include the target cell’s CSG ID and the UE’s membership ID in the Relocation Request to the MME.”

On question 2:

RAN3 can provide recommendations on the NR Femto architecture as soon as the study on Additional Topological Enhancements (RP-234041) has concluded. RAN3 is planning to conclude the in RAN3 #125.

Proposal 2: On question 2, RAN3 to reply to SA2: “On question 2:

RAN3 can provide recommendations on the NR Femto architecture as soon as the study on Additional Topological Enhancements (RP-234041) has concluded. RAN3 is planning to conclude the in RAN3 #125.”

A draft Reply LS has been provided in the Annex.

Proposal 3: Agree on draft Reply LS in the Annex.

# 3	Conclusion

This contribution discussed the content of the Reply LS to SA2 on the support of UE moving between CAG cell of 5G Femto and CSG cell. The following proposals have been made:

Observation 1a: Solution 1 has RAN impact due to the incorrect use of measurement reports, which can cause malfunction of NG-RAN and E-UTRAN.

Observation 1b: Solution 1 has UE impact since the UE needs to perform CSGCAG mapping and use measurement reports in a manner unintended by legacy specification.

Observation 2a: For 4G  5G handover, Solution 2 has no impact on RAN or UE.

Observation 2b: For 5G  4G handover, Solution 2 has no impact on the UE. It also has no impact on RAN as long as the AMF includes the target cells CSG and the UE’s membership ID in the Relocation Request to the MME.

Proposal 1: On question 1, RAN3 to reply to SA2: “Reply to question 1:

Solution 1 has both RAN and UE impacts due to the incorrect use of measurement reports, which would cause malfunction of the NG-RAN and E-UTRAN. RAN3 prefers to have UEs follow the procedures as intended by the specification.

Solution 2 has no RAN or UE impact. RAN3 emphasizes that for 5G  4G handover of this solution, the AMF needs include the target cell’s CSG ID and the UE’s membership ID in the Relocation Request to the MME.”

Proposal 2: On question 2, RAN3 to reply to SA2: “Reply to question 2:

RAN3 can provide recommendations on the NR Femto architecture as soon as the study on Additional Topological Enhancements (RP-234041) has concluded. RAN3 is planning to conclude the study in RAN3 #125.”

Proposal 3: Agree on draft Reply LS in the Annex.

# Annex: Draft Reply LS to SA2

3GPP TSG RAN WG3#125	R3-24xxxx

Maastricht, The Netherlands, August 19 – 23, 2024

Title:	[Draft] Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell

Response to:	LS in R3-243020 (S2-2405813) on Support of UE move between CAG cell of 5G Femto and CSG cell

Release:	Rel-19

Study Item:	FS_WAB_5GFemto_NR

Source:	Qualcomm [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on Support of UE move between CAG cell of 5G Femto and CSG cell (R3-243022/S2-2405813).

RAN3 has discussed the questions provided by SA2 and would like to provide the following replies:

Question 1: SA2 would like to know whether the two solutions mentioned above have any impact on the RAN (e.g., for RAN procedures)?

Reply to question 1:

Solution 1 has both RAN and UE impacts due to the incorrect use of measurement reports, which would cause malfunction of the NG-RAN and E-UTRAN. RAN3 prefers to have UEs follow the procedures as intended by the specification.

Solution 2 has no RAN or UE impact. RAN3 emphasizes that for 5G  4G handover of this solution, the AMF needs include the target cell’s CSG ID and the UE’s membership ID in the Relocation Request to the MME.

Question 2:  SA2 has reserved the time units for the normative work of WT#1 based on the result of RAN3 work (RP-234041), which is expected to start in SA2 from SA2#164. Therefore, SA2 requests to confirm the conclusion of RAN3 on overall architecture, etc., which will be used as the basis for SA2's normative work.

Reply to question 2:

RAN3 can provide recommendations on the NR Femto architecture as soon as the study on Additional Topological Enhancements (RP-234041) has concluded. RAN3 is planning to conclude the study in RAN3 #125.

RAN3 asks SA2 to take the above feedback into account.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #125-bis, October 14 to 18, 2024       Hefei, China

TSG-RAN3 Meeting	 #126, November 18 to 22, 2024       Orlando, FL, USA