3GPP TSG-RAN WG3 Meeting #125	R3-244366

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	15.2

Source:	Huawei

Title:	Support store and forward for IoT NTN

Document for:	discussion

# 1	Introduction

According to the Rel-19 IoT NTN WID, RAN aims to support the Store & Forward (S&F) satellite operation as follows [1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (split) core network on satellite) etc.; further coordination with CT1 might be required.

SA2 and RAN2 discussed this topic for several meetings. SA2 concluded the SI work in TR 23700-29-100 [2], while RAN2-126 meeting made the following agreements.

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In this paper we provide our considerations in RAN3 perspective based on the two architectures concluded by SA2 TR 23.700-29.

# 2	Discussion

## 2.1	S&F architectures and attach procedure

The characteristic of S&F satellite operation is that service link and feeder link are not available at the same time, i.e., the satellite cannot connect with any NTN gateway when it serves the UE, and the satellite cannot serve the UE when it connects with the NTN gateway.

SA2 has concluded two architectures for S&F operation, namely split MME on-board and whole CN on-board in TR 23700-29-100  [2].

For the split MME on-board arch, the eNB and split MME is on-board, and other CN components are on the ground. The UE attach procedure is shown by Fig 1 based on the TR 23700-29-100 [2].

Fig 1. UE attach in split MME architecture

- T1: The UE sends attach request to the MME1 on satellite1, and the UE would receive an attach reject message from satellite1 at first time, since the HSS is on the ground which is currently un-accessible. The MME1 on satellite1 stores the received information included in the attach request.

- T2: The satellite1 connects with the NTN gateway, and the MME on-ground performs UE verification.

- T3: If the UE verification succeeds, the MME on-ground sends the UE registration information to the MME2 on satellite2 which will cover the UE at next time. MME2 stores the UE registration information. Here, it is noted that in T1 the UE may be configured by MME1 a waiting timer for the time of satellite2 coming.

- T4 and after: The satellite2 serves the UE. The UE initializes attach request again, and the MME2 sends attach accept to the UE since the UE registration information has been stored at MME2. When the satellite2 leaves UE, the UE would stay in EMM-REGISTERED. At a later time, the UE registration information would be sent from the MME on-ground to the next satellite (e.g., satellite3). When satellite3 serves the UE, the UE can send service request rather than attach request to MME3.

For the whole CN on-board arch, things are much simpler. All CN components are on-board, the UE attach procedure can be directly finished as legacy, as shown in Fig 2. When satellite1 leaves UE, the UE is de-registered, and when satellite2 comes, the UE attach procedure is implemented again.

Fig 2. UE attach in whole CN on-board architecture

As illustrated above, both architectures are applicable at least at the stage of attach. RAN3 can assume both architectures can be based for RAN study, until further progress made in SA2 work item, e.g., if any of them is excluded.

- RAN3 consider both whole CN on-board and split MME architectures are applicable for S&F IoT NTN, until further progress made in SA2.

## 2.2	CIoT CP and UP signalling reduction optimisations in S&F operation

In legacy CIoT, both UP solution and CP solution are specified in TS 36.300. In CP solution, the UE data is transferred via NAS message and handled by MME. And in UP solution, the UE data is transferred via S1-U tunnels and handled by SGW.

RAN2 #125b meeting made the following agreements in [3]:

- The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In the whole CN architecture, there is no issues for supporting the CP and UP solutions. The store and forward operation can be implemented by the MME on-board and MME on the ground (for CP solution), and SGW on-board and proxy on the ground (for UP solution).

In the split MME arch, for the CP solution, the data is stored on the MME on the ground and the MME on-board; for the UP solution, the data is stored on the SGW on the ground and the eNB on-board, since there is no SGW on-board.

- In the split MME arch, for the CIoT CP solution, the data is stored on the MME on the ground and the MME on-board; while for the CIoT UP solution, the data is stored on the SGW on the ground and the eNB on-board, since there is no SGW on-board.

Based on the Fig 1 above, it can be observed that that for CP solution, the UE’s data can be sent to the on-board MME together with the UE registration information, e.g., in T3.

However, for the UP solution, since the SGW is on the ground, and only MME is on-board, it needs to study how to send the UE DL data to the on-board eNB which has not stored or acquired the UE context yet. It can be observed that this issue happens not only during the attach process, but also during later service setup procedure. Each time the UE access the satellite, the accessed satellite may be different from the previous one. In the split MME architecture, RAN3 needs to discuss how to support the CIoT UP solution, i.e. how to setup the GTP-U tunnel and send the UE’s DL UP data to the eNB which has not been accessed by the UE.

- In the split MME architecture, RAN3 to discuss and acknowledge the issue on how to support the CIoT UP solution, i.e. how to setup the GTP-U tunnel and send the UE’s DL UP data to the eNB which has not been accessed by the UE.

## 2.3	Stage 2 and Stage 3 TP

The stage 2 TP can be envisioned to include two aspects for IoT NTN: the regenerative payload, and the S&F operation.  The BLCR for NR NTN was agreed in [4], which provides a good baseline for the IoT NTN regenerative payload. Since it is to be updated based on the NR NTN discussion, it may be a little early to have the BLCR for IoT NTN, we could wait NR NTN becomes more stable. Regarding the S&F operation, a simple way is to refer to SA2 spec, without many details in RAN specification.

On stage 3, now nothing specific to S&F seems to be required, however the regenerative aspects should be transposed to LTE, even if the MME or CN will be on-board. These are deployment option indeed, but the standard should explain the role of the interface and feature for e.g. testing component of the deployments …

- RAN3 to consider the stage 2 IoT NTN TP to include the regenerative payload (which is a mirror of NR NTN), and the S&F operation. No need to endorse it at this meeting. RAN3 also to discuss and acknowledge the stage 3 specification needed for regenerative aspects, if any in NR NTN.

# 3	Conclusion

This contribution discusses the store and forward operation for the IoT NTN. It is proposed:

- RAN3 consider both whole CN on-board and split MME architectures are applicable for S&F IoT NTN, until further progress made in SA2.

- In the split MME arch, for the CIoT CP solution, the data is stored on the MME on the ground and the MME on-board; while for the CIoT UP solution, the data is stored on the SGW on the ground and the eNB on-board, since there is no SGW on-board.

- In the split MME architecture, RAN3 to discuss and acknowledge the issue on how to support the CIoT UP solution, i.e. how to setup the GTP-U tunnel and send the UE’s DL UP data to the eNB which has not been accessed by the UE.

- RAN3 to consider the stage 2 IoT NTN TP to include the regenerative payload (which is a mirror of NR NTN), and the S&F operation. No need to endorse it at this meeting. RAN3 also to discuss and acknowledge the stage 3 specification needed for regenerative aspects, if any in NR NTN.

# 4	References

- RP-234077, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Deutsche Telekom.

- TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3.

- R2-2403733, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 125bis.

- R3-243949 Support for Regenerative Payload in NR NTN, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics