3GPP RAN WG3 Meeting #125	R3-244506

# Maastricht, Netherlands, 19th – 23rd August 2024

Source:	CATT, Nokia, Huawei, CMCC

Title:	Clarification on indication of cell re-activation

Agenda Item:	9.1

Document for:	Discussion

- Introduction

This document proposes to clarify how an eNB owning a dormant cell indicate to the neighbour node that the energy saving cell is re-activated.

- Discussion

Based on current 36.300, in case the eNB owning the dormant cell decides to switch on the concerned cell, it should inform all peer node by an indication on the X2 interface.

The eNB owning the dormant cell should normally obey a request. The switch-on decision may also be taken by O&M. All peer eNBs are informed by the eNB owning the concerned cell about the re-activation by an indication on the X2 interface. The eNB indicates the re-activation action to a GERAN and/or UTRAN node by means of the eNB Direct Information Transfer procedure over S1.

However, in 36.423, there is no indication which is used to indicate the re-activation of energy saving cell. And there is only an optional IE which indicates the de-activation of the corresponding cell in ENB CONFIGURATION UPDATE message.

| >Deactivation Indication | O |  | ENUMERATED(deactivated, …) | Indicates that the concerned cell is switched off for energy saving reasons | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Meanwhile, for EN-DC scenario, it is clearly stated that the absence of NR Deactivation Indication IE in EN-DC CONFIGURAION UDPATE message is used to indicate that the concerned cell is activated as below:

| >>>NR Deactivation Indication | O |  | ENUMERATED(deactivated, …) | Indicates that the concerned NR cell is switched off for energy saving reasons. If this IE is not included, indicates that the concerned cell is activated. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

So, a feasible solution to indicate the re-activation of a dormant cell between two eNBs is to send an ENB CONFIGRUATION UDPATE message with the absence of Deactivation Indication IE. However, without clearly statement in the spec, there maybe different implementations on indication of re-activation of dormant cells which would result in failure in multi-vendor deployment scenarios.

With that, we propose to introduce the similar statement in Deactivation Indication IE as the one in NR Deactivation Indication IE to make it clear that absent of this IE is to indicate the concerned cell is activated.

Proposal 1: In 36.423, add description in Deactivation Indication IE that If this IE is not included, indicates that the concerned cell is activated.

- Conclusion

Proposal 1: In 36.423, add description in Deactivation Indication IE that If this IE is not included, indicates that the concerned cell is activated.

The corresponding CR is provided in [1]

- Reference

[1] R3-244610 Clarification on indication of dormant cell reactivation   CATT, Nokia, Huawei, CMCC