3GPP TSG-RAN WG3#126 	R3-247102

Orlando, USA, 18-22 November 2024

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-gNB-CU LTM Procedure

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-242356 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC o Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint • Specify RRM requirements related to the above objectives as necessary [RAN4]

This paper discusses the basic required mobility procedures for inter-CU L1/L2 triggered mobility (LTM) and its potential impact to the disaggregated gNB architecture signalling.

# 2	Stage-2 Inter-CU LTM for non-split gNB Architecture

In Rel-18, LTM was specified with limitation that LTM mobility could be configured and carried out only to cells within the same gNB-CU (i.e. intra-gNB LTM). In Rel-19, this feature expands to also cover inter-gNB-CU (i.e. inter-gNB LTM) mobility.

For Inter-CU LTM, we foresee the following as necessary to be considered.

- Rel-18 LTM framework and LTM handover phases for intra-gNB-CU LTM are basis also for inter-gNB-CU LTM. That is, inter-CU LTM also has the following phases:

(a) Preparation phase, (b) Early synchronization phase, (c) Execution phase, and (d) Completion phase.

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. RAN3#125bis: Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered. WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The LTM Configuration IDs are allocated by the source CU. WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS. Continue to work on the solutions comparison and details…

## 2.1	Preparation Phase

The following steps are needed during the preparation phase and similar to L3 handover as defined in TS 38.300.

Step 0: The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

Step 1: The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

Step 2: The source gNB decides to configure LTM based on measurement reports.

Candidate cell information exchange

RAN3 has agreed that SSB related information can be made available to the Source gNB from the target gNB nodes via Xn interface setup and configuration update procedures.

Additionally, for Rel-19 Inter-CU LTM, given that TCI states and cell switch for LTM can also be based on CSI-RS, there is need for signalling CSI-RS related information regarding possible target cells to the Source gNB. One approach is to follow a similar method as with SSB signalling. However, since CSI-RSs are typically configured as UE-specific resources (unlike SSBs, which are cell-specific), using non-UE-associated signalling may not be ideal for providing appropriate CSI-RS configurations specific to a UE. Another approach is to use UE-associated signalling for periodic/semi-periodic CSI-RS configurations. However, it has a drawback that it would incur an additional class-1 procedure based on RAN2 agreement i.e. common CSI-Resource Configuration is generated by source CU. The procedure will consist of following four steps:

Step # 1: Source CU sends HANDOVER REQUEST message to target CU based on L3 measurements.

Step # 2: Target CU sends CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

Step # 3: Source CU will generate common CSI-Resource Configuration based on CSI-RS configuration received from all target cells and sends to target CU in  LTM CONFIGURATION UPDATE message.

Step # 4: Target CU will send CSI-Report Configuration to source CU in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Proposal 1: RAN3 to decide whether CSI-RS configuration to be sent by UE associated or non-UE associated signalling.

Step 3: The source gNB issues a HANDOVER REQUEST message(s) to the candidate target gNB(s).

Single vs. Parallel messages

In Rel-18, the aspect of whether a single message could configure multiple LTM cells, or whether multiple parallel messages should be used was thoroughly discussed in context of intra-CU LTM, with RAN3 opting for the multiple message approach. For Rel-19, we suggest taking this principle as baseline and have of separate messages per each LTM cell being configured also for inter-gNB LTM. Hence, turn the following Working Assumption into agreement,

Proposal 2: Turn into agreement that “WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.”

Step 4: Admission control is performed by target gNB(s) according to legacy procedure.

Step 5: The candidate target gNB(s) sends an LTM response via the HANDOVER REQUEST ACKNOWLEDGE message including configurations of LTM candidate cell to the source gNB.

Step 6: The source gNB sends an RRCReconfiguration message to the UE, containing the configuration of LTM candidate cells.

Step 7: The UE sends an RRCReconfigurationComplete message to the source gNB.

## 2.2	Early Synchronization Phase

RAN3 agreed at last meeting to “introduce a new procedure on Xn to transfer the TA information”. However, left inconclusive the details regarding procedure’s Class and whether it is non UE-associated.

The early synchronization can be supported for inter-CU LTM taking Rel-18 approach for intra-CU case as baseline in a straightforward manner. Hence, a new non-UE associated procedure is required over Xn from Target gNB to Source gNB. Likewise, in case of disaggregated architecture, existing DU-CU/CU-DU TA INFORMATION TRANSFER messages can be reused.

Proposal 3: The XnAP TA INFORMATION TRANSFER procedure is a non-UE associated procedure.

Proposal 4: Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario.

Step 8: Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300.

Step 9: The Target gNB send a TA INFORMATION TRANSFER message to the UE indicating the acquired TA values.

## 2.3	LTM Execution Phase

Following steps are performed during execution phase.

Step 10-11: Based on L1 measurement reports, source gNB decides on the target cell and sends Cell Switch Command (CSC) towards the UE.

Step 12: The Source-CU sends CELL SWITCH NOTIFICATION to the Target gNB indicating the Target Cell ID and TCI State related information.

Step 13: Source gNB-CU sends a SN STATUS TRANSFER message to target gNB and data forwarding is performed.

Step 14: UE marks LTM handover completion according to RAN2 specified indication (e.g., on reception of grant from target gNB).

## 2.4	LTM Completion Phase

During the Completion Phase, the following steps are performed.

Step 15: RRCReconfigurationComplete message is sent to target gNB.

Step 16: The Target gNB sends the HANDOVER SUCCESS message to Source gNB.

Step 17-19: Path switch procedure is performed.

Step 20: Target gNB may send UE CONTEXT RELEASE message to release the UE context at the Source gNB.

## 2.5	Stage-2 Signalling Flow for non-split Inter-CU LTM

Based on the above procedure, the stage-2 signaling flow for Inter-CU LTM is shown below. For subsequent LTM scenario, additional signalling may be required between Source and Target gNBs and to be FFS.

Figure 1: Inter-gNB-CU LTM Signaling with non-split architecture

# 3	Support for Subsequent inter-CU LTM Aspects

Support for Subsequent LTM will require additional information to be provided to Candidate gNB(s) that have LTM candidates. The following items are several aspects to be considered. Similarly, a signalling flows depicting these proposals is shown below.

## 3.1	Preparation Phase

Xn Interface between Candidate gNBs:

In case of Subsequent inter-CU LTM, there is need for the new Source gNB to be able to exchange messages with the Candidate gNBs. That is, there is need for an Xn control interface between all Candidate gNBs configured. Likewise, RAN3 has agreement that “Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM.”. This implies that a prerequisite for inter-CU LTM shall be that an Xn control interface link exists between all Candidate gNBs. That is, a Candidate-CU that becomes the Source-CU after mobility shall not trigger additional Xn-C setup procedures with Candidate-CUs if a Xn control interface had not been established beforehand.

Proposal 5: It is a pre-requisite for Inter-CU LTM candidate configuration that Candidate gNBs have an Xn control interface established between each other. Otherwise, the Inter-CU LTM preparation shall not be accepted.

## 3.2	LTM Execution Phase

Data Forwarding and TNL Information Update:

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB. Otherwise, the Target gNB (which becomes Source gNB after LTM mobility) will lack the necessary TNL information to execute data forwarding for subsequent inter-CU LTM. One possible alternative to address this issue is to signal this information after cell switch has been triggered toward the UE and include this information as part of the XnAP CELL SWITCH NOTIFICATION message. Nevertheless, the signalling design can be flexible to also accommodate providing this information at an earlier step.

Proposal 6: The Source gNB includes the PDU Session Resource Information List information of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

Proposal 7: The Source gNB may also provide PDU Session Resource Information List information of all Candidate gNBs prior to LTM cell switch operation.

TA Forwarding:

In the Rel.18 DU-CU and CU-DU CELL SWITCH NOTIFICATION messages, the TA information list IE may be included to transfer obtained TA values to the target (i.e., new serving) cell. However, RAN3 should study whether TA values is enough to be sent from source gNB to target gNB or more information is needed in order to determine the TA validity at the target gNB. Note that TA validity was left up to source DU implementation in case of Rel. 18 intra-CU LTM. We believe, in case of Rel. 19 inter-CU LTM, it cannot be left up to target DU/gNB implementation as the Xn link delay needs to be accounted for. Earlier in Rel.18, RAN3 had discussed introducing TA validity timer sent from target DU without agreement. Another approach, which can be linked to UE signal experienced rather than time itself, is to provide information such as the RSRP which the UE experienced when TA was acquired. This would allow the Target-gNB/DU to better understand the signal conditions associated with a TA value, rather than a “time” value tied to the TA acquisition itself. This information will support the Target-gNB/CU determine whether a given TA received should be consider valid.

Proposal 8: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP/XnAP LTM CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Based on the received RSRP value along with TA value(s) and the new RSRP value, the new source DU can determine the validity of the TA value. Note that such scheme is already possible for SDT and should be considered for LTM.

Figure 1: Execution Phase Considerations for Subsequent Inter-gNB-CU LTM

## 3.5	LTM Completion Phase

Handling of Xn UE AP Identifiers:

When inter-CU LTM is firstly configured, a UE-associated connection will be established between the Source gNB and every Candidate gNB (e.g., when XnAP HANDOVER REQUEST and HANDOVER REQUEST ACKNOWLEDGE messages are exchanged). Hence, the XnAP Identifiers corresponding to a specific UE context will always point toward the Source gNB that firstly initiated the inter-CU LTM configuration. In other words, one pair of identifiers is linked by a parameter given by the Source gNB and another given by the Candidate gNB.

However, in case of Subsequent inter-CU LTM there is issue that after mobility from Source gNB to Target gNB, XnAP identifiers for the UE context become outdated, since the Source gNB has changed. Therefore, the UE-association should be updated with parameter that points toward the new Source gNB.

RAN3 discussed this aspect and reach agreement but left open the aspect with respect to timing of carrying out this identifier exchange, which was captured as follows “Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.”

In our view, it is not necessary to inform all XnAP UE identifiers to all Candidate-gNBs at preparation phase, given that there is no guarantee this information will be of use during the lifetime of the UE context. Therefore, another alternative for carrying out this task is that after inter-CU LTM, the new Source gNB sends a UE-associated message toward all Candidate gNBs. This message will serve the purpose of updating the XnAP identifiers for the UE context. This has benefit to ensuring that the information sent to Candidate-gNBs will be of use, since all Candidate-gNBs will need to know which node has become the new Source-gNB/CU, and avoids sending unnecessary information.

Proposal 9: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM CONFIGURATION UPDATE message to update the information related to a given UE context.

Candidate Triggered gNB release:

At prior meeting, RAN3 agreed that a Candidate gNB can initiate release of LTM candidate cell(s) of its own and has introduced the LTM CANCEL procedure for this purpose at last meeting. However, after inter-CU LTM mobility, there is problem that the new Source gNB-CU is unknown to all Candidate gNB(s) other than the gNB that initiated the mobility (i.e. the old Source gNB). Therefore, for a Candidate gNB to be able to trigger release of its own LTM candidate cells, it always needs to be aware of which gNB is the current Source gNB.

One way to solve this issue is to have the new Source gNB signal to all Candidate gNBs that it has become the new Source gNB. This can take place via the XnAP LTM CONFIGURATION UPDATE message, which can serve both to indicate the change of Source-gNB as well as exchange Xn AP UE IDs as proposed above.

Proposal 10: Above proposed, Xn LTM CONFIGURATION UPDATE message is also used by new Source gNB to signal the change of gNB.

Figure 1: Completion Phase Considerations for Subsequent Inter-gNB-CU LTM

## 3.4	Subsequent Inter-CU LTM Signalling

Taking the prior proposals into account, the resulting signalling flow is the following.

Figure 1: Inter-gNB-CU LTM Signaling considering Subsequent LTM between Source gNB and Candidate gNBs with non-split architecture

# 4	Stage 2 Inter-CU LTM for split gNB Architecture

Similar to Section 2, this section describes the stage-2 signalling flow for inter-CU LTM considering disaggregated gNB architecture.

As starting baseline, existing procedures in Rel-18 in F1AP, E1AP and XnAP should be reused and applicable also for Rel-19 inter-CU LTM.

Inclusion of source DU and source CU/gNB identity:

In Rel-18, CFRA resources to be used for random access can be shared only among UEs belonging to same DU. This allows the CFRA resource sharing among UEs without additional burden of signalling exchange for coordination among DUs. This is because, to have collision free random access only one CFRA resource (preamble) can be used at a given time.

In our view, this holds true also for inter-CU case and we should allow the possibility of CFRA resource sharing. Hence, both the source gNB-DU ID IE and source gNB ID IE are included in the HANDOVER REQUEST message.

Observation 1: CFRA resource sharing is allowed in intra-CU LTM and is beneficial for dense deployments where CFRA resources are scarce.

Proposal 11: Both source gNB ID (global ID) and source gNB-DU ID (local ID per CU) are provided to candidate target gNBs.

Proposal 12: The candidate target gNB allocates CFRA resources based on the source gNB ID and source gNB-DU ID.

The resulting stage-2 signaling flow for Inter-CU LTM with disaggregated architecture is shown below.

Figure 2: Inter-gNB-CU LTM Signaling with split architecture

# 5	Conclusions

Base signalling

Proposal 1: RAN3 to decide whether CSI-RS configuration to be sent by UE associated or non-UE associated signalling.

Proposal 2: Turn into agreement that “WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.”

Proposal 3: The XnAP TA INFORMATION TRANSFER procedure is a non-UE associated procedure.

Proposal 4: Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario.

Subsequent LTM Considerations

Proposal 5: It is a pre-requisite for Inter-CU LTM candidate configuration that Candidate gNBs have an Xn control interface established between each other. Otherwise, the Inter-CU LTM preparation shall not be accepted.

Proposal 6: The Source gNB includes the PDU Session Resource Information List information of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

Proposal 7: The Source gNB may also provide PDU Session Resource Information List information of all Candidate gNBs prior to LTM cell switch operation.

Proposal 8: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP/XnAP LTM CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Proposal 9: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM CONFIGURATION UPDATE message to update the information related to a given UE context.

Proposal 10: Above proposed, Xn LTM CONFIGURATION UPDATE message is also used by new Source gNB to signal the change of gNB.

Disaggregated gNB Considerations

Observation 1: CFRA resource sharing is allowed in intra-CU LTM and is beneficial for dense deployments where CFRA resources are scarce.

Proposal 11: Both source gNB ID (global ID) and source gNB-DU ID (local ID per CU) are provided to candidate target gNBs.

Proposal 12: The candidate target gNB allocates CFRA resources based on the source gNB ID and source gNB-DU ID.

# References

- 3GPP TS 38.401

- RP-242356 NR mobility enhancements Phase 4, Intel (moderator)

# Annex: Text Proposal for TS 38.401

### 8.9.X	Inter-gNB-CU LTM Handover

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 8.2.1.X-1 shows the inter-gNB-CU LTM procedure for intra-NR.

Figure 8.9.X-1: Inter-gNB-CU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells.

2.	The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the source gNB-CU.

3.	The source gNB-CU determines to initiate LTM configuration.

4.	The source gNB-CU issues a HANDOVER REQUEST message(s) to the target gNB-CU(s) with necessary information to prepare the handover at the target side.

5.	Admission Control is performed by the target gNB-CU(s).

6.	The target gNB-CU sends a UE CONTEXT SETUP REQUEST message to the target candidate gNB-DU(s) for each candidate cell, containing one target candidate cell ID, the LTM configuration ID of the candidate cell, and LTM configuration ID mapping list. The target gNB-CU may request PRACH resources from the candidate gNB-DU(s). The target gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

7.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

8.	The target gNB(s) sends a HANDOVER REQUEST ACKNOWLEDGE message including configurations of LTM candidate cells to the source gNB-CU.

9.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other gNB-DU(s). The source gNB-CU may send the updated CSI resource configuration to the source gNB-DU.

10.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

11.	The source gNB-CU sends a LTM CONFIGURATION UPDATE message to the target gNB-CU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells.

Editor’s note: Details are FFS.

12.	The target gNB-CU a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

13.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the candidate gNB-DU.

14.	The target gNB-CU responds with a LTM CONFIGURATION UPDATE ACKNOWLEGDE message.

Editor’s note: Details on whether this step is mandatory or optional is FFS.

15.	The source gNB-CU generates the RRC Reconfiguration.

16.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

17.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

18.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

19.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

20.	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

21.	Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300 [2].

22.The candidate gNB-DU sends the TA value, the associated CFRA resource information, the candidate cell ID source gNB-DU ID, and the Source gNB ID to the source gNB-DU in the DU-CU TA INFORMATION TRANSFER message to the target gNB-CU.

23.The target gNB-CU sends the TA INFORMATION TRANSFER message including the TA related information received from the candidate gNB-DU to the Source gNB-CU.

24.	The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

25.	The UE sends the L1 measurement result to the source gNB-DU.

26.	The source gNB-DU decides to execute LTM to a candidate target cell.

27.	The source gNB-DU sends the Cell Switch command to the UE.

28.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the source gNB-CU to indicate the initiation of the Cell Switch command to the UE, for which the message includes the target cell ID and the TCI state ID.

NOTE: The Source gNB may initiate early data forwarding at an earlier timing than LTM cell change being triggered.

29.	The source gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-CU in the LTM CELL SWITCH NOTIFICATION message.

30.	The target gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

31.	The source gNB-CU sends a SN STATUS TRANSFER message to the target gNB-CU.

32.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

33.	The target gNB-DU sends an ACCESS SUCCESS message to the source gNB-CU with the target cell ID.

34.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

35.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

36.	The target gNB-CU sends a HANDOVER SUCCESS message to the source gNB-CU.

NOTE : Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Editor’s note: Subsequent LTM and its associated signaling and messages between source gNB-CU, target gNB-CU and the target candidate gNB-DU(s) to support it are FFS.

37. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

38.	The target gNB-CU may send the UE CONTEXT RELEASE message to the source gNB-CU to release the resources of prepared cells.

39.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

40.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.