3GPP TSG-RAN WG3 Meeting #126	R3-247109

Orlando, FL, USA, November 18th – 22nd 2024

Agenda Item:	12.2

Source:	Ericsson

Title:	(TP for WAB BL CR for TS 38.401): Functional Aspects of WAB-Nodes

Document for:	Agreement

# Introduction

In this paper we continue the normative discussion on the functional aspects of WAB-nodes.

A TP for WAB BL CR for TS 38.401 is presented in the Annex.

# Discussion

We discuss the following functional aspects of WAB-nodes:

- NG connection management.

- WAB authorization.

- Mobility handling.

- Xn connection management.

- Prevention of multihop and prevention of Xn setup between WAB-gNBs.

- Resource coordination.

- Handling of backhaul link degradation.

- Packet delay budget (PDB) handling.

## NG connection management

Based on the SI conclusions, there exists a RAN3 consensus for supporting NG connection removal. According to the TR 38.799, NG connection removal is needed at least in the following cases:

- WAB-node mobility, e.g., when a WAB-node is about to leave the service area of one AMF (or AMF set) and is about to enter the service area of another AMF (or AMF set).

- WAB-gNB authorization status changes from “authorized” to “not authorized”. In this case, NG connection removal is executed after all served UEs have been handed over and/or released.

As stated by the WID, the WAB discussions on NG connection management should consider the corresponding NTN conclusions.

Proposal 1-1: The NG connection(s) of a WAB-gNB can be removed upon WAB-node mobility, or when the authorization status of the WAB-gNB becomes “not authorized”.

Two major differences between WAB-gNBs and static gNBs are:

- NG connections of WAB-gNB are carried over a BH that is wireless.

- WAB-gNBs can be mobile or nomadic.

These two special properties of WAB-gNBs that differentiate them from static gNBs, impose additional challenges that may require special handling by the core network. While most of “bells and whistles” for this special handling can be left to implementation, the standard should at least allow the awareness at the core network that a gNB is a WAB-gNB (and not a static gNB). One example where the WAB awareness at the core network would be used for an essential functionality is positioning. For example, if the core network knows that a gNB is a WAB-gNB, then, for the LCS service, the LMF will try to obtain not only the UE location but also the WAB-gNB location.

A straightforward way to enable the core network’s awareness of a WAB-gNB is to introduce a WAB-gNB indication in the NG SETUP REQUEST message.

Proposal 1-2: Introduce a “WAB-gNB” indication in the NG SETUP REQUEST message.

## WAB authorization

In the SI, RAN3 concluded that the authorization of the WAB-gNB and the authorization of WAB-MT should be decoupled. Authorization of the WAB-gNB refers to the right of the WAB-gNB to serve UEs and it is managed by the WAB-gNB’s OAM. WAB-MT authorization refers to the right of the WAB-MT to support backhauling of WAB-gNB’s traffic via BH PDU session(s) and it is managed by the BH-5GC.

With respect to the normative work on WAB-node authorization, the following should be considered:

- Common understanding in RAN3 is that WAB-MT authorization is in the remit of SA2, but RAN3 needs to specify the actions taken by the WAB-node when the authorization status of the WAB-MT and WAB-gNB changes.

- SA2’s understanding is that WAB-MT authorization (for providing BH PDU sessions) is based on Allowed S-NSSAIs. SA2 also assumes that the authorization changes of the WAB-MT and WAB-gNB part are (somehow) coordinated. This is captured in the following bullets from the Conclusions section of TR 23.700-6:

>>>>>>>>>>>>>>>>>>Start of excerpt from TR 23.700-6<<<<<<<<<<<<<<<<<<<<

  - MWAB-gNB's OAM is assumed to be aware of the MWAB-UE authorization limitations (pre-determined authorizations) and may (pre-)configure/trigger MWAB-gNB to turn on/shut down accordingly.

…

NOTE 2: It is assumed that the OAM ensures the de-authorization of a MWAB-gNB (as described in bullet 4 above) is not conflicting with the de-authorization of MWAB-UE e.g. the allowed NSSAI for the BH PDU sessions of the MWAB-UE is maintained long enough for the MWAB-gNB to moves the connected UE away. The coordination of the MWAB-gNB service authorization/configuration from OAM and the update of subscription data (e.g. slice/DNN) of the MWAB-UE is handled by OAM/management system.

>>>>>>>>>>>>>>>>>>>>>>>End of excerpt from TR 23.700-6<<<<<<<<<<<<<<<<<<<<<

According to the above text, the OAM of the WAB-gNB's PLMN and the OAM of the WAB-MT's PLMN (where humans may be a part of the loop, as well) should have the coordinated information about the authorization statuses of the two. So, when the BH-5GC triggers the removal of BH PDU session due the removal/invalidation of the S-NSSAIs for BH, the WAB-gNB will already have moved away the UEs and removed its NG/Xn connections, owing to the fact that it will get notified by its OAM about the imminent authorization status change in advance.

To begin the normative work, RAN3 should discuss the principal steps of authorization handling. The principal steps that we foresee are captured in the below proposal.

Proposal 2: When the authorization status of a WAB-gNB changes from “authorized” to “not authorized” (along with the authorization status of its co-located WAB-MT):

- The WAB-gNB node attempts to hand over and/or releases the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- Optionally, some or all PDU sessions of the WAB-MT may be released, and the WAB-MT may be de-registered from the network.

PDU session removal and WAB-MT de-registration are meaningful if the “not authorized” status is expected to hold for a long time.

## Mobility handling

At the end of the SI, it was concluded that, for UE mobility, legacy procedures will be reused. One unresolved issue is how to support UE’s AMF change for a (moving) WAB-gNB. The corresponding WI objective states:

- Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB.

For this scenario, RAN3 needs to describe the overall set of actions that the network needs to take upon UE’s AMF change. This set of actions includes:

- The establishment of NG connectivity to the new AMF (set).

- Optional update of the TAC/RANAC.

- Consequent handling of RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE UEs

- The removal of NG connection(s) to the old AMF (set).

To support WAB-gNB mobility with UE’s AMF change, RAN3 captured 4 different solutions in TR 38.799, clause 4.3.4.3.3:

- One solution with two logical WAB-gNBs.

- Three solutions with single logical WAB-gNB:

  - Option 1: Single WAB-gNB with a single cell using mobility registration update due to TAC change.

  - Option 2: Single WAB-gNB with two cells with different TACs, using NG-based HO.

  - Option 3: Single WAB-gNB single cell without TAC change.

Given that RAN3 confirmed the feasibility of the solution with two logical WAB-gNBs, we conclude that all pre-requisites for fulfilling the corresponding WI objective are in place, and that RAN3 may proceed to normative work based on this solution only. At this point, we do not see any reason to proceed with any additional solution.

Proposal 3: Support only the two-logical-gNB solution for UE’s AMF change.

## Xn connection management

As per the SI conclusions, WAB-nodes should support Xn connectivity, and the relevant WI objective states:

- Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

During the WAB-node integration procedure, the WAB-MT will first connect to the BH-gNB, followed by NG connection establishment between the WAB-gNB and the UE’s AMF(s), and Xn interface establishment between the WAB-gNB and the BH-gNB.

Common understanding from the SI discussions is that Xn interface establishment between the WAB-gNB and the BH-gNB will be needed at least for the sake of resource multiplexing between the WAB-MT and WAB-gNB. Nevertheless, for the resource multiplexing to work, i.e., for the PHY resources of the WAB-MT and the co-located WAB-gNB to be configured in a non-interfering way, it is necessary for the BH-gNB to be aware of the co-location of the WAB-MT and the WAB-gNB. This is captured in clause 4.3.5 of TR 38.799, as follows:

For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

A straightforward way to support co-location discovery would be to include an identifier of the WAB-MT into the XN SETUP REQUEST message sent from the WAB-gNB to the BH-gNB. In addition, if the WAB-gNB has already established an Xn connection with the WAB-MT’s target BH-gNB, the NG-RAN NODE CONFIGURATION UPDATE message can be used. This is relevant for the scenario when the WAB-MT is handed over from the current BH-gNB to one of the surrounding gNBs. Which WAB-MT identifier to include into the XnAP message needs to be discussed.

Proposal 4-1: The WAB-gNB includes an ID of the co-located WAB-MT in the XN SETUP REQUEST or in the NG-RAN CONFIGURATION UPDATE message sent to the BH-gNB.

The underlying assumption with respect to prevention of Xn setup between WAB-gNBs is that WAB-nodes will be moving constantly and may be in each other’s vicinity only briefly. In such a scenario, setting up Xn connection(s) between WAB-nodes is indeed not meaningful. However, we can foresee the scenarios in which it may be necessary to set up Xn interface between WAB-gNBs. Some example scenarios are:

- Multiple WAB-nodes moving together, due to being deployed onboard the same vessel.

- A nomadic WAB scenario, where multiple static WAB nodes are temporarily deployed to provide coverage in a disaster-hit area.

Based on the above, we think that it should be possible to establish Xn interface connections between WAB-gNBs, i.e., that the solution for prevention of inter-WAB Xn interface setup should be optional.

Proposal 4-2: Xn connection between WAB-gNBs can be established.

With respect to Xn setup, the specifications should not mandate whether a WAB-gNB should establish Xn with all or with some of its surrounding gNBs. This should rather be an implementation choice. In that sense, in a scenario where the WAB-MT is handed over to a target BH-gNB that had no Xn connection with the WAB-gNB prior to the HO, service interruption may occur because resource coordination cannot be done without an Xn connection between the WAB-gNB and the BH-gNB, that has not yet been set up. Therefore, service interruption could be reduced by setting up the Xn connection between the WAB-gNB and the target BH-gNB is set up as early as possible. One way to speed up Xn interface setup towards the target BH-gNB is to notify the WAB-gNB as soon as possible about the target (BH-)gNB for the WAB-MT HO.

In case the WAB-gNB already has an Xn connection towards the target BH-gNB (e.g., because the target BH-gNB is a neighbour of the source BH-gNB), the WAB-gNB needs to know which of its Xn neighbours is the new BH-gNB, to be able indicate to the target BH-gNB that it is co-located with the incoming WAB-MT (e.g., for enabling the resource multiplexing for the WAB-node). This also speaks in favour of notifying the WAB-gNB about the target BH-gNB for the WAB-MT HO.

Proposal 4-3: The WAB-gNB should be notified about the target BH-gNB for the WAB-MT HO.

According to the WID, the BH link can be a terrestrial or a non-terrestrial link. This is captured in the WID as follows:

NOTE 5: Backhaul link for WAB-MT can be TN or NTN.

In our view, the WAB-gNB should be aware of whether the BH link is TN or NTN. This information could be of interest for the WAB-gNB’s PLMN for, e.g., setting the QoS of the PDU sessions of the served UEs. In addition, in resource multiplexing, the WAB-gNB can benefit from this knowledge in case it is the one determining the HSNA configuration.

Proposal 4-4: The WAB-gNB should be aware of whether the BH link for the WAB-MT is TN or NTN.

## Prevention of multihop and prevention of Xn setup between WAB-gNBs

At the RAN3#125-bis meeting, solutions for multihop prevention in WAB topologies were discussed, This does not work, if the BH-gNB is not WAB-aware (i.e., does not know how to add the indication).

What if NG-based HO? Impact on CN/AMF when source and/or target AMF need to know the WAB indication?

without a conclusion. However, a list of candidate solutions was established, as follows:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

We analyse each solution below.

Solution 1 incurs minimal impact. The only impact seems to be that the OAM needs to configure the WAB-gNB with the knowledge of the dedicated WAB-gNB OTA parameters that would deter other WAB-MT from connecting to this WAB-gNB.

In fact, Solution 1 could equally well be used for prevention of Xn setup between WAB-gNBs. For example, a dedicated frequency used by a WAB-gNB cell can indicate that the cell is served by a WAB-gNB, while a dedicated PCI value may indicate whether the WAB-gNB accepts incoming Xn setup requests from other WAB-gNBs.

Observation 5-1: Solution 1 for multihop prevention, can also ensure prevention of Xn setup between WAB-gNBs.

Solution 2 has some potential shortcomings:

- Sending the allowed S-NSSAIs in msg5 corresponds to the Access Stratum Connection Establishment NSSAI Inclusion Mode d), which is not the default mode. The original purpose of indicating the allowed S-NSSAIs in msg5 is AMF selection, meaning that Solution 2 introduces an additional purpose of sending the Allowed S-NSSAIs in msg5. This is not a “clean” approach.

- RRC msg5 is sent unencrypted, leaving this approach vulnerable to rogue WAB-MTs.

Solution 3 the main drawback of this solutions is that it incurs impact on the Uu interface, so off-the-shelf UEs could not be used for the WAB-MT function.

Solution 5 works only for HO, but it does not work for initial access. This means that it needs to be used in conjunction with another candidate solution. Given that all other solutions work for both initial access and HO, Solution 5 seems redundant. This solution also incurs BH-RAN impact, meaning that WAB-unaware BH-RAN will not support it. Namely, the source BH-gNB needs to be able to set the “WAB-MT” indicator in HO signalling and the target BH-gNB needs to understand its meaning. Finally, in case of NG-based HO, the BH-AMF(s) also need to be able to understand the WAB-MT indication.

Based on the above, we propose to adopt Solution 1 for prevention of multihop and prevention of Xn setup between WAB-gNBs.

Proposal 5: Solution 1 (dedicated OTA parameters) is used for preventing multihop in WAB topologies and for prevention of Xn setup between WAB-gNBs.

## WAB-specific XnAP and NGAP cause values

RAN3 may want to consider the introduction of WAB-specific cause values. Below are some scenarios in which WAB-specific cause values could be considered:

- Removal of WAB-gNB's NG connection may fail, and a corresponding cause value needs to be provided in the NGAP failure removal message.

- Given that, for WAB, NG connection removal can be requested either due to WAB-gNB mobility or WAB-node authorization status change, RAN3 should also consider introducing a cause value indicating the reason for NG connection removal.

- The WID calls for a solution for preventing Xn setup between WAB-gNBs. As explained above, this solution should be optional. If a WAB-gNB1, that is configured to set up Xn connections towards other WAB-gNBs (and, hence, is not configured to support the solution for Xn setup prevention), attempts to set up Xn towards WAB-gNB2 that does not allow it, the latter can respond with the existing Xn SETUP FAILURE message. For that case, RAN3 should consider introducing a corresponding cause value to be included in the XN SETUP FAILURE message.

- The SI conclusion states that the WAB-MT reuses the legacy UE mobility procedures. With respect to the HO, two possible scenarios defined in legacy are HO failure and HO cancellation. In these cases, the Xn connection towards the source BH-gNB must be retained. In other words, if the WAB-MT remains connected to the BH-gNB due to, e.g., HO failure or cancellation, the BH-gNB should reject the removal request from the WAB-gNB. For this case, RAN3 should discuss introducing a proper cause value in the XN REMOVAL FAILURE message.

- When a WAB-node moves sufficiently far from the group of gNBs with which it maintains Xn connections, these connections need to be removed. For this scenario, the existing Xn Removal procedure can be reused. Given that, for WAB, Xn removal can be requested either due to WAB-gNB mobility or WAB-node authorization status change, RAN3 should also consider introducing a cause value indicating the reason for Xn interface removal.

Proposal 6: Discuss the introduction of WAB-specific cause values for XnAP and NGAP.

## Resource coordination

The WID calls for supporting radio resource coordination between WAB-node’s access and BH links. It should be noted that this radio resource coordination problem does not apply to all WAB scenarios, i.e., it applies only for in-band scenarios with 3GPP BH, and only if WAB-MT and the WAB-gNB are served by the same PLMN.

In Rel-17, RAN3 specified XnAP signalling for IAB-node radio resource coordination. Although the problem of radio resource coordination for IAB-nodes is essentially identical to the corresponding problem for WAB-nodes (i.e., resource coordination for a RAN node and an MT in the same physical box), the procedure defined for IAB cannot be directly reused for WAB. This is not only because the existing IAB message and IE names are IAB-specific, but also because certain existing IEs in these messages are not applicable to WAB-nodes.

Hence, we propose to define a new XnAP procedure for radio resource coordination between access and BH links. As said above, the procedure pertains only to in-band scenarios, and only if the WAB-MT and WAB-gNB are served by the same PLMN.

Proposal 7-1: For in-band backhauling in non-roaming scenarios, introduce a new XnAP procedure for a WAB-gNB and its BH-gNB to coordinate the resources of this WAB-gNB and its co-located WAB-MT.

Although the XnAP IAB procedures cannot be directly reused for radio resource coordination between access and BH links of WAB-nodes, the IAB IEs in clauses 9.2.2.94-97 of TS 38.423 are a good starting point for the discussion.

Proposal 7-2: For in-band backhauling, discuss which parts of XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423 should be used in the procedure for WAB resource coordination.

Given that Xn interface can be established only between RAN nodes of the same PLMN, it follows that Xn-based resource coordination in roaming scenarios is not possible. Hence, RAN3 should assume out-of-band BH in case the WAB-gNB and WAB-MT are served by different PLMNs.

Proposal 7-3: RAN3 assumes out-of-band backhauling when the WAB-gNB and the WAB-MT are served by different PLMNs.

## Handling of backhaul link degradation

The traffic to and from the UEs served by the WAB-gNB will traverse two wireless links:

- The backhaul (BH) link between the BH-gNB and the WAB-MT.

- The access link between the WAB-gNB and the UE.

The BH link, like any other wireless link, is prone to degradation, failures, and congestion. If the BH link is in bad conditions (poor coverage, congestion etc.), all PDU sessions of the WAB-MT used for backhauling of traffic to and from the WAB-gNB may be affected. In the worst case, the NG connection(s) of the WAB-gNB may fail, resulting in the WAB-gNB and its served UEs being cut-off from the network.

Observation 8-1: When BH link degradation occurs, avoiding NG connection failure is crucial.

Several issues need to be considered. First, we notice that, when it comes to communication in the WAB architecture, there exist multiple control loops. These loops are overlapping, but they may be managed by different, possibly unrelated, entities. For example, one loop, which encompasses the UE PDU sessions established between the UEs and the UE’s 5GC, is controlled by the UE’s 5GC. The access link for delivering this traffic to/from the UE is controlled by the WAB-gNB. On the other hand, the UE PDU sessions are carried inside PDU sessions of the WAB-MT (managed by the BH-5GC), over the radio resources on the BH link (managed by the BH-gNB). The WAB-MT’s PDU sessions are managed by the BH-5GC.

Observation 8-2: In the WAB architecture, there are multiple overlapping control loops:

- Radio resources on the BH link are controlled by the BH-gNB.

- Radio resources on the access link are controlled by the WAB-gNB.

- PDU sessions of the WAB-MT are controlled by the BH-5GC.

- UE data traffic flow is controlled by the UE’s 5GC.

Second, the existing tools defined for PDU session management (e.g., on the NG interface) are not suitable for dealing with BH link degradation because:

- The WAB architecture has not yet been specified, meaning that the existing tools against link degradation have not been tailored to the WAB architecture. For example, the design of existing tools did not consider the scenario where WAB-MT’s PDU sessions are used to tunnel the PDU sessions between the WAB-gNB and the UE’s 5GC.

- The NG protocol (including NG-C and NG-U) is designed for wired networks. Meanwhile, in WAB architecture the NG traffic is carried over the wireless BH link.

The existing user plane failure handling can only deal with the GTP-U failure by using Error Indication defined for the GTP-U protocol in TS 29.281. For UEs served by a WAB-node, this indication is sent to the UE’s 5GC. Nevertheless, this indication is intended for different types of protocol failures and session context unavailability, rather than for indicating link degradation. A similar conclusion holds for the NGAP Error Indication procedure. This means that, as of today, the NG signalling cannot be used to indicate that the cause of problems is wireless BH link degradation. Also, a procedure to recover from poor link conditions is not specified. Hence, we conclude that the existing tools cannot support handling of BH link degradation in the WAB architecture.

Observation 8-3: The NG protocol is designed for wired networks, while the NG connection between the WAB-gNB and the UE’s 5GC is carried over non-stationary wireless BH.

Third, when the BH wireless link deteriorates, the BH-gNB and the WAB-MT are almost immediately aware of it. Meanwhile, the WAB-gNB and the UE’s 5GC are likely unaware of whether the problem occurred on the BH link or in the wired network between the BH-gNB and the UE’s 5GC. They may observe that there is an issue, but they cannot infer whether they can do something about it.

As observed earlier, in this case, it is of utmost importance to prevent the failure of the WAB-gNB NG-C connection. Unfortunately, the BH-gNB may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic and may not be able to prevent the failure of NG connection in the best way. On the other hand, the WAB-gNB and the UE’s 5GC may help the BH-gNB in dealing with BH link degradation and avoiding NG-C connection failure. For example:

- The WAB-gNB and the UE’s AMF(s) may prioritize the sending of NG control plane traffic and the SCTP traffic (e.g., heartbeat messages and pings), to prevent NG connection failure.

- The WAB-gNB may reduce the amount of traffic of the NG and/or Uu interfaces, by e.g., prioritizing some traffic and disconnecting low-priority traffic, so that the remaining available bandwidth is used for control plane traffic.

Observation 8-4: The BH-gNB is able to determine degradation of wireless BH, but it may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic, and it may not be able to prevent the failure of WAB-gNB NG-C connection.

Observation 8-5: The WAB-gNB and UE’s AMF can take actions that can contribute to avoiding NG-C connection failure.

Based on the above, we conclude that, to be able to counteract BH link degradation and to avoid NG-C connection failure, the access and the BH network should be able coordinate for mitigating BH link degradation, so that appropriate action can be taken.

Proposal 8: The access and the BH network can coordinate for mitigating BH link degradation.

## PDB handling

Fig. 1: The WAB architecture

In legacy scenarios involving “normal” gNBs (i.e., non-WAB-gNBs), there is no wireless BH – in this case, there is only one wireless hop on the way to the UE, i.e., the Uu interface between the gNB and the UE. In that case, the total end-to-end (i.e., WAB-UPF-to-UE) PDB pertaining to a 5QI consists of the RAN part (5G-AN PDB from TS 23.501) and the CN part (CN PDB from TS 23.501).

With respect to the PDB, in legacy scenarios, during the UE PDU session and QoS setup, the UE’s 5GC should provide to the NG-RAN the CN PDB, either via NGAP signalling, or a static value can be preconfigured/hard-coded at the NG-RAN node. The access network (AN, i.e., the NG-RAN) uses it to calculate the 5G-AN PDB and considers this value in its scheduling decisions. In other words, the PDB of the RAN (5G-AN PDB from TS 23.501) equals the PDB indicated in the configured 5QI minus the CN PDB (either signalled or predefined):

5G-AN PDB = (Total end-to-end PDB) – CN PDB

According to the WAB architecture, shown in Fig. 1, the traffic to/from the UEs served by the WAB-gNB will traverse two wireless links:

- The first link between the BH-gNB and the WAB-MT, i.e., the BH link.

- The second link between the WAB-gNB and the UE, i.e., the access link.

Hence, in the WAB architecture, compared to the legacy scenario, there exists an additional wireless hop between the BH-gNB and the WAB-MT – the packet towards a UE traverses the UE’s 5GC, the BH-5GC, the BH-gNB, the wireless BH link to the WAB-MT, the WAB-gNB and, finally, the wireless access link to the UE. Given that the WAB-gNB and the WAB-MT may be served by two different core networks, the time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Observation 9-1: The time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Applying today’s tools to the WAB architecture, when the 5QI/QoS is configured for the PDU sessions of the UEs served by the WAB-gNB, the existence of wireless BH, and the QoS of the WAB-MT’s PDU sessions (that carry the PDU sessions of UEs served by the WAB-gNB), are not considered. Meanwhile, the BH-5GC, that sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured for UE PDU sessions.

Observation 9-2: The BH-5GC that serves the WAB-MT and sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured by the UE’s 5GC for UE PDU sessions, and vice versa.

This leads to inefficient resource planning in the BH network. For services with high performance requirement, such as XR, the WAB-gNB must compensate for the delay incurred at the BH part of the network, to ensure the end-to-end service quality.

This means that, in WAB scenarios, the end-to-end PDB consists of the following:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

- The PDB of the WAB-gNB.

Consequently, the “CN PDB”, from WAB-gNB point of view, i.e., the PDB for the path between the WAB-UPF and the WAB-gNB consists of the following:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

Hence, in case of WAB architecture, additional factors with respect to legacy architecture need to be considered. Namely, the BH-5GC PDB and the BH-gNB PDB, i.e., the time between the time the data comes out from WAB-UPF to the time when it reaches the WAB-gNB, need to be subtracted from the end-to-end PDB when calculating the 5G-AN PDB of the WAB-gNB (herein referred to as the WAB-gNB PDB). Otherwise, the WAB-gNB PDB will not be correctly calculated and the data for the PDU session may not be correctly scheduled.

In the simplest case, the WAB-gNB, should calculate the PDB of the WAB-gNB (the WAB counterpart of the legacy 5G-AN PDB) according to the following formula:

WAB-gNB PDB = (Total end-to-end PDB) – (UE’s 5GC PDB) – (BH PDU Session/QoS PDB)

Proposal 9: Calculation of the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501) considers the PDB of the BH network.

Certain aspects of the above analysis are captured in the TP for the WAB BL CR for TS 38.401, provided in the Annex.

Proposal 10: Agree the TP for WAB BL CR for TS 38.401, provided in the Annex.

# Conclusion

This paper discusses the functional aspects of WAB-node. The following is observed and proposed:

NG interface aspects

Proposal 1-1: The NG connection(s) of a WAB-gNB can be removed upon WAB-node mobility, or when the authorization status of the WAB-gNB becomes “not authorized”.

Proposal 1-2: Introduce a “WAB-gNB” indication in the NG SETUP REQUEST message.

WAB authorization

Proposal 2: When the authorization status of a WAB-gNB changes from “authorized” to “not authorized” (along with the authorization status of its co-located WAB-MT):

- The WAB-gNB node attempts to hand over and/or releases the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- Optionally, some or all PDU sessions of the WAB-MT may be released, and the WAB-MT may be de-registered from the network.

Mobility handling

Proposal 3: Support only the two-logical-gNB solution for UE’s AMF change.

Xn interface aspects

Proposal 4-1: The WAB-gNB includes an ID of the co-located WAB-MT in the XN SETUP REQUEST or in the NG-RAN CONFIGURATION UPDATE message sent to the BH-gNB.

Proposal 4-2: Xn connection between WAB-gNBs can be established.

Proposal 4-3: The WAB-gNB should be notified about the target BH-gNB for the WAB-MT HO.

Proposal 4-4: The WAB-gNB should be aware of whether the BH link for the WAB-MT is TN or NTN.

Prevention of multihop and prevention of Xn setup between WAB-gNBs

Observation 5-1: Solution 1 for multihop prevention, can also ensure prevention of Xn setup between WAB-gNBs.

Proposal 5: Solution 1 (dedicated OTA parameters) is used for preventing multihop in WAB topologies and for prevention of Xn setup between WAB-gNBs.

WAB-specific XnAP and NGAP cause values

Proposal 6: Discuss the introduction of WAB-specific cause values for XnAP and NGAP.

Resource coordination

Proposal 7-1: For in-band backhauling in non-roaming scenarios, introduce a new XnAP procedure for a WAB-gNB and its BH-gNB to coordinate the resources of this WAB-gNB and its co-located WAB-MT.

Proposal 7-2: For in-band backhauling, discuss which parts of XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423 should be used in the procedure for WAB resource coordination.

Proposal 7-3: RAN3 assumes out-of-band backhauling when the WAB-gNB and the WAB-MT are served by different PLMNs.

Handling of backhaul link degradation

Observation 8-1: When BH link degradation occurs, avoiding NG connection failure is crucial.

Observation 8-2: In the WAB architecture, there are multiple overlapping control loops:

- Radio resources on the BH link are controlled by the BH-gNB.

- Radio resources on the access link are controlled by the WAB-gNB.

- PDU sessions of the WAB-MT are controlled by the BH-5GC.

- UE data traffic flow is controlled by the UE’s 5GC.

Observation 8-3: The NG protocol is designed for wired networks, while the NG connection between the WAB-gNB and the UE’s 5GC is carried over non-stationary wireless BH.

Observation 8-4: The BH-gNB is able to determine degradation of wireless BH, but it may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic, and it may not be able to prevent the failure of WAB-gNB NG-C connection.

Observation 8-5: The WAB-gNB and UE’s AMF can take actions that can contribute to avoiding NG-C connection failure.

Proposal 8: The access and the BH network can coordinate for mitigating BH link degradation.

Packet delay budget (PDB) handling

Observation 9-1: The time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Observation 9-2: The BH-5GC that serves the WAB-MT and sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured by the UE’s 5GC for UE PDU sessions, and vice versa.

Proposal 9: Calculation of the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501) considers the PDB of the BH network.

TP for the WAB BL CR for TS 38.401

Proposal 10: Agree the TP for WAB BL CR for TS 38.401, provided in the Annex.

# Annex: TP for WAB BL CR for TS 38.401

-------------------------------------------Start of changes-------------------------------------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [34].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [34].

Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

BH-5GC: The 5GC serving the WAB-MT.

BH-AMF: The AMF serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-RAN-node: The NG-RAN node serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

WAB-gNB: The gNB functionality, as defined in TS 38.300 [2], that provides NR access interface towards the UE.

WAB-MT: The WAB-node’s function that terminates the Uu interface to the BH-RAN-node using the procedures and behaviours specified for UEs. Corresponds to the MWAB-UE function defined in TS 23.501 [3].

WAB-node: An NG-RAN node comprising the WAB-MT and the WAB-gNB functionality.

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

UL	Uplink

WAB	Wireless Access Backhaul

-------------------------------------------Next change-------------------------------------------

# 6	NG-RAN architecture

## 6.1	Overview

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 6.1.x	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-RAN-node. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.x-1 shows the WAB architecture for 5GS.

Figure 6.1.x-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.  The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.x-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.x-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.x-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

Figure 6.1.x-3: Protocol stacks for Xn Control plane and Xn User plane transport

-------------------------------------------Next change-------------------------------------------

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

## X.2	Configuration of WAB-node

### X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node. Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

### X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic. In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

## X.3 	NG connection management

The UE’s AMF can become aware that the gNB is a WAB-gNB during the setup or the update of the NG connection. The NG connection(s) of the WAB-gNB can be removed, either due to inter-AMF mobility of a WAB-gNB or when the authorization status of the WAB-gNB changes from “authorized” to “not authorized”.

## X.4 	Xn connection management

A WAB-gNB can establish Xn connectivity with the BH-gNB serving the co-located WAB-MT. The BH-gNB can become aware about the co-location of the WAB-MT and the WAB-gNB during the setup or the update of the Xn connection between the WAB-gNB and the BH-gNB.

Xn connection can also be established between WAB-gNBs.

## X.5 	WAB-node authorization

WAB-node authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB.

The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the traffic of the co-located WAB-gNB via BH PDU session(s). The WAB-MT is authorized based on one or more S-NSSAIs and DNN dedicated for WAB.

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

WAB-node authorization can be steered by policies provided to the node by the OAM or by the SeGW (location-, time- or event-based policies). Alternatively, the OAM can enforce the policies by configuring the WAB-gNB only when the WAB-gNB is authorized.

The WAB-gNB’s service authorization status may change. The OAM ensures the de-authorization of a WAB-gNB is not conflicting with the de-authorization of WAB-MT, meaning that the allowed NSSAI for the BH PDU sessions of the WAB-MT are maintained long enough for the WAB-gNB to hand over or release the UEs. The coordination of the WAB-gNB service authorization/configuration from OAM and the update of subscription data (e.g. slice/DNN) of the WAB-MT are handled by OAM/management system.

In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to hand over, or releases, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- Some or all PDU sessions of the WAB-MT may be released, and the WAB-MT may be de-registered from the network.

-------------------------------------------End of changes-------------------------------------------