3GPP TSG-RAN3 Meeting # 126	R3-247122

# Orlando, US, 18th – 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR with rethinking on Rel-18 group paging design |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom, CMCC, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Previous RAN3 discussion for eMBS followed a principle that whether to enable the RRC_INACTIVE reception feature should be per gNB decision, i.e., one gNB should not tell another gNB how to do scheduling on which one, RRC_INACTIVE or RRC_CONNECTED based multicast reception. That is why RAN3 did not introduce the enhanced group paging on XnAP during Rel-18 standardization. In this way, we maintain the independence of scheduling between different gNBs. However, there are cases we had spotted that might violate above principle, to introduce interruptions across gNB, and even make the RRC_INACTIVE reception introduced in Rel-18 eMBS obsolete. (In following cases, UE is released by gNB1 with RNA area that covers cells in both gNB1 and gNB2. UE reselects cell that belongs to gNB2.) Case 1. The session is inactive. In the RRCRelease from gNB1, UE does not receive any indication to enable the RRC_INACTIVE reception for any session. - UE re-selects to cell under gNB2. UE stays in RRC_INACTIVE, regardless of the existence of MCCH/PTM configuration, as it behaves as Rel-17 specification. - UE receives Rel-18 based group paging with the session it had joined. - UE resumes, regardless of gNB2’s intention that is to keep UE in RRC_INACTIVE to mitigate the congestion. This basically makes gNB2’s scheduling decision to enable RRC_INACTIVE reception obsolete, if there are a lot of UE that are about to resume. Case 2. The session is inactive. In the RRCRelease from gNB1, UE receives the indication that RRC_INACTIVE reception is enabled. - UE re-selects to cell under gNB2. UE stays in RRC_INACTIVE, if it finds in the MCCH that the session is still indicated as inactive. - UE receives Rel-18 based group paging with the session it had joined. However, the intention of gNB1 is to resume UE to RRC_CONNECTED (i.e., to delivery the session for UE in RRC_CONNECTED), while gNB2 is to adopt the Rel-18 RRC_INACTIVE reception. This is quite reasonable and possible, based on the understanding of independent scheduling of each gNB. - gNB2 has to make a choice upon receving the XnAP group paging from gNB1 the session activation from 5GC. - Deliver enhanced group paging. UE stays in RRC_INACTIVE and start monitoring gNB1 won’t be able to resume UE it once released. gNB1 might try the group paging for multiple rounds, which will resume UEs in cells of gNB2 unintentionally, and again, makes gNB2’s scheduling decision to enable RRC_INACTIVE reception obsolete. Or gNB1 might try unicast paging and resumes UE one by one. But who knows which paging strategy gNB1 might take? - Deliver normal group paging, which of course violates gNB2’s intention. Based on the above elaboration on the cases, we have observed that such independence was compromised by the design of the Rel-18 group paging mechanism. Please also note such compromise can not be mitigated by implementation. A simple solution, better late than never, is to confine related UE’s RNA area to be within the scope of one gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in stage 2 description to confine the RNA configuration: “The decision to enable data reception of MBS multicast session in RRC_INACTVE state is in a per cell granularity, by each gNB the cell belongs to. The gNB may decide to confine the RNA area of the UE that is released to RRC_INACTIVE for MBS multicast session reception to be within the scope of one gNB, to avoid the scheduling conflicts between different gNBs.” Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The principle that to enable the RRC_INACTIVE reception feature should be per gNB decision is compromised. And one gNB’s scheduling decision on multicast session might make another gNB’s scheduling to enable RRC_INACTIVE based reception obsolete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.10.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

// change starts.

16.10.5	Multicast Handling

16.10.5.1	Session Management

There are two delivery modes as specified in TS 23.247 [45]:

-	5GC Shared MBS traffic delivery;

-	5GC Individual MBS traffic delivery.

As specified in TS 23.247 [45], if the gNB supports MBS, the network shall use the 5GC Shared MBS traffic delivery in which case an MBS Session Resource context for a multicast session is setup in the gNB when the first UE joins the multicast session.

For 5GC Shared MBS traffic delivery mode, shared NG-U resources are used to provide MBS user data to the gNB. The gNB initiates the Multicast Distribution Setup procedure towards the 5GC, to allocate shared NG-U resources for a multicast session. In case multiple MBS session areas as specified in TS 23.247 [45] are associated with the same multicast session for location dependent MBS services, multiple NG-U shared resources are established for the same multicast session per MBS Area Session ID served by the gNB.

A shared NG-U resource applies one of the following transport options:

-	unicast transport;

-	multicast transport.

For 5GC Shared MBS traffic delivery an MBS Session Resource comprises one or several MRBs. If minimisation of data loss is applied for a given MRB, synchronisation of allocation of PDCP COUNT values is applied by either or a combination of the following methods:

-	derivation of the PDCP COUNT values by means of a DL MBS QFI Sequence Number provided on NG-U. Synchronisation in terms of MBS QoS flow to MRB mapping and PDCP SN size of the corresponding MRB among gNBs are achieved by means of network implementation.

-	deployment of a Shared NG-U Termination at NG-RAN, shared among gNBs, which comprises a common entity for assignment of PDCP COUNT values. Synchronisation in terms of MBS QoS flow to MRB mapping and PDCP SN size of the corresponding MRB among gNBs may be achieved by means of network implementation.

If PDCP COUNT values are derived from a DL MBS QFI Sequence Number provided on NG-U and only one QoS Flow is mapped to an MRB, the gNB shall set the PDCP COUNT value of PDCP PDU to the value of the DL MBS QFI Sequence Number provided with the received packet over NG-U. If PDCP COUNT values are derived from a DL MBS QFI Sequence Number provided on NG-U and multiple QoS Flows are mapped to an MRB, the gNB may derive the PDCP COUNT value of the PDCP PDU from the sum of the DL MBS QFI Sequence Numbers of the QoS Flows mapped to this MRB.

NOTE:	Synchronisation of PDCP COUNT values in case user data for MBS QoS flows mapped to the same MRB arrive over NG-U at different gNBs in different order or in case of loss of data over NG-U, and related handling of minimisation of data loss is left to implementation.

As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS Assistance Information associated with a multicast MBS session for a UE, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE is expected to require dedicated resources very frequently. Based on this information, the gNB may decide the RRC state of the UE. The QoS requirements of the multicast session apply regardless of the RRC state within which the UE receives multicast session data.

The decision to enable data reception of MBS multicast session in RRC_INACTVE state is in a per cell granularity, by each gNB the cell belongs to. The gNB may decide to confine the RNA area of the UE that is released to RRC_INACTIVE for MBS multicast session reception to be within the scope of one gNB, to avoid the scheduling conflicts between different gNBs.

// change ends.