3GPP TSG-RAN WG3 Meeting #126	R3-247126

Orlando, USA, 18 – 22 November 2024

Agenda Item:	13.2

Source:	Google

Title:	(TP to BL CR to 38.300 and 38.423) Support for reference configurations in the inter-CU MCG LTM

Document for:	Discussions & Approval

# 1	Discussions

In Rel-19 LTM agenda in RAN3#125bis, the following To Be Continued item is captured.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

In the meantime, RAN2 sent us a LS (R3-247111) carrying all the agreements related to inter-CU LTM for the past two meetings. The part specifically related to the Inter-CU MCG LTM is excerpted below:

RAN2#127:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

Context for the above agreement:

### 8.6.2 Inter-CU LTM Remaining open issues and further details of LTM preparation phase (e.g. single or multiple reference configuration(s)?, details of RRC signaling structure, etc.), early sync phase, execution phase (e.g. support of mixture of intra-CU and inter-CU, etc.), and LTM cell switch completion phase. Initial discussion on inter-CU LTM with DC (high-level spec impacts and solutions). Single or multiple reference configuration(s)? P1 in R2-2406694 (Apple) Proposal 1: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations are supported. - P1 in R2-2406694 is agreed. P1 in R2-2407269 (LG) Proposal 1. Multiple reference configurations are provided for inter-CU LTM, where each reference configuration is CU specific. [Samsung]: Multiple reference configuration gives more flexibility. NW still can achieve single reference configuration, which is up to NW. [Huawei]: Assume it does not happen often and saved bits are not significant. It is considered as optimization for some cases, which is not essential. [ZTE]: With multiple reference configurations, it simplifies inter-CU signalling. [Xiaomi]: With multiple reference configurations, it increases UE complexity and it requires more UE memory capability. [Ericsson]: Single reference configuration works as baseline. [Nokia]: With single reference configuration, the reference configuration setup procedure needs to be repeated whenever any CU reconfigures some configuration. [Session chair]: Check companies’ positions. If there is no clear majority companies, we can stick to what we already have (single reference configuration). - Multiple reference configurations: LG, Samsung, CMCC, Nokia, Fujitsu, Rakuten, ITRI, KDDI, ZTE, BT, IDC, KT, ETRI (13) - Single reference configuration: Google, Honor, CATT, Lenovo, Vivo, Apple, OPPO, Spredtrum, Xiaomi, NEC, Intel, MediaTek, Qualcomm, China Telecom, Ericsson, Sharp (16)

As also mentioned in the RAN3 offline discussion, in the RAN2#127-bis there were a further discussion on whether to adopt multiple reference configurations for inter-CU LTM; however, as excerpted below from the RAN2 chairman’s note, it was not pursued so the above agreement should be held and we can reflect it in the RAN3 specifications if there is no further dispute on this topic.

R2-2409031 Multiple reference configuration for inter-CU LTM KDDI Corporation, LG Electronics, Nokia, ZTE, Rakuten, KT, CMCC, ETRI, ITL, ITRI, Fujitsu, SK telecom, Uplus discussion Noted. [Session chair]: With the consideration of comparison with the last RAN2 meeting, it seems not much changed in the point of companies’ positions. Hope to understand that we sometimes should make hard decision. Suggest noted without repeating the discussion. [LG]: Acceptable, however please note there may be similar discussion in RAN3 for their perspective.

For the inter-CU MCG LTM, there could be ways for the source gNB to obtain the reference configuration. It can be based on the serving/source gNB configuration so it is available from the beginning of the inter-CU MCG LTM preparation. Or alternatively, the source gNB can request the reference configuration from the first candidate gNB and provide it to the other candidate gNB(s) in the following preparations. Therefore, the source gNB, if available, may provide a reference configuration in a Handover Request message. Or instead the source gNB can request one by the Handover Request message and obtain one in the Handover Request Acknowledge message from a candidate gNB.

Proposal 1: The source gNB can either request a reference configuration or, if available, provide a reference configuration for LTM in a Handover Request message.

Proposal 2 : A candidate gNB, if requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message.

Proposal 3 : If a reference configuration for LTM is provided in the Handover Request message, the candidate gNB indicates whether the LTM candidate configuration is a full/complete candidate configuration in the Handover Request Acknowledge message.

# 2	Conclusion

Proposal 1: The source gNB can either request a reference configuration or, if available, provide a reference configuration for LTM in a Handover Request message.

Proposal 2 : A candidate gNB, if requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message.

Proposal 3 : If a reference configuration for LTM is provided in the Handover Request message, the candidate gNB indicates whether the LTM candidate configuration is a full/complete candidate configuration in the Handover Request Acknowledge message.

- Text Proposal to Inter-CU LTM BL CR to TS 38.300

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB, if available, may provide the reference configuration to the one or more candidate gNB(s). The source gNB may request a candidate gNB to provide the reference configuration.

Editor’s Note: Working Assumption: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB may provide the reference configuration in the HANDOVER REQUEST ACKNOWLEDGE message if being requested by the source gNB.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s).

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Working Assumption: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

- Text Proposal to Inter-CU LTM BL CR to TS 38.423

#### 9.2.1.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target NG-RAN node To Source NG-RAN node Transparent Container | FFS |  | FFS |  |
| Reference Configuration | O |  | OCTET STRING |  |
| Complete Configuration Indicator | O |  | ENUMERATED (complete, ...) | (FFS) or the RRC Config Indication in the 9.2.3.72 can be re-used |

<< End of Changes >>