3GPP TSG-RAN WG3 Meeting #126	R3-247138

Orlando, USA, 18-22 Nov 2024

Agenda Item:	11.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Network Slicing

Document for:	Discussions & Approval

- Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in AI/ML-assisted Network Slicing, as captured in TR 38.743 [2], we present our analysis of the normative work for the network slicing use case.

Following is the agreements and left issues achieved in the previous meeting:

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

In this contribution, we further provide our understanding and analysis on the working assumption and left issues, and provide the corresponding TP to BL CR 38.423.

- Discussion

In the last meeting, we agreed that introduce the slice ID information in the DATA COLLECTION REQUEST message, while whether the failure of predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node is FFS.

During the Rel-18 phase, the DATA COLLECTION RESPONSE message was used to indicate which requested information failed to be initiated. With the introduction of slice-level information, it naturally follows that the receiving node needs to inform whether the requested slice-level information was successfully initiated. The ability of the receiving node to provide predicted slice radio resource status and predicted slice available capacity depends on its AI/ML capabilities. In some cases, the node may be able to generate the predicted cell radio resource status but may not be able to generate the predicted slice radio resource status.

Therefore, it is better to explicitly introduce Slice Measurement Initiation Result List IE to indicate that predicted radio resource status per slice or predicted slice available capacity can not be initated.

- Introduce Slice Measurement Initiation Result List IE in the DATA COLLECTION UPDATE message to indicate the predicted radio resource status per slice or predicted slice available capacity can not be initiated.

In the DATA COLLECTION REQUEST message, the Report Characteristics for Data Collection IE should be extended to include predicted slice radio resource status and predicted slice available capacity. After reviewing the legacy IE design, the 'First Bit = Predicted Radio Resource Status' can be reused to request the predicted slice radio resource status, along with the slice ID information in the request message. A new bit should be introduced for requesting the predicted slice available capacity.

- First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status.

- Introduce new bit for predicted slice available capacity in the Report Characteristics for Data Collection IE.

Regarding the DATA COLLECTION UPDATE message, the current specification already includes the Predicted Radio Resource Status IE, which refers to the legacy Radio Resource Status IE. This IE already includes the Slice Radio Resource Status. Additionally, semantic description captured is that the Predicted Radio Resource Status IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE.

Therefore, to include the predicted radio resource status per slice in the DATA COLLECTION UPDATE message for Rel-19, the semantic description of the Predicted Resource Status IE needs to be modified as follows:

“The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE.”

- Modify the description of the Predicted Resource Status IE to “The IE includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE.”

Next, predicted slice available capacity IE is introduced in Cell Info Result for Data Collection Item IE in the DATA COLLECTION UPDATE message and refers to the Slice Available Capacity IE

- Predicted slice available capacity IE is introduced in Cell Info Result for Data Collection Item IE and refers to the Slice Available Capacity IE.

  - Per Slice UE performance

In Rel-18, RAN3 agreed to introduce UE-level performance feedback to evaluate the performance of decisions made after handover. Currently, UE performance feedback includes metrics such as Average UE Throughput (UL/DL), Average Packet Delay(UL/DL), and Average Packet Loss (DL). During the Rel-19 SI phase, it was acknowledged that finer granularity UE performance feedback should be introduced for specific slice.

As described in TS 28.552 [3], average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

5.1.1.1	Packet Delay

5.1.1.1.1	Average delay DL air-interface

a)	This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

5.1.1.1.3	Average delay UL on over-the-air interface

a)	This measurement provides the average (arithmetic mean) over-the-air packet delay on the uplink. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

5.1.1.3	UE throughput

5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

5.1.1.3.3	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

5.1.3.1.3	DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI.

- Average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

When a UE initiates a PDU Session, the UE selects an appropriate S-NSSAI based on the application requirements to specify the needed network slice. The network uses the S-NSSAI to determine which slice is best suited to carry the PDU Session. For example, if a PDU Session is intended for eMBB services, it might select an S-NSSAI that identifies a network slice specifically configured for high-bandwidth, low-latency services. A PDU Session is mapped to a specific network slice.

Therefore, following option is proposed to reflect the slice-level UE performance:

-	Option 1: UE performance per S-NSSAI per PDU Session

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDU Session List |  | 0..1 |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |
| >PDU Session ID | M |  | 9.2.3.18 |  |
| >S-NSSAI |  |  |  |  |
| >Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >Average Packet Delay | O |  | 9.2.3.187 |  |
| >Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

-	Option 2: UE performance per QoS Flow per PDU Session

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDU Session List |  | 0..1 |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |
| >PDU Session ID | M |  | 9.2.3.18 |  |
| >QoS Flow List |  | 1 |  |  |
| >>QoS Flow Item |  | 1..<maxnoofQoSFlows> | 9.2.3.10 |  |
| >>QoS Flow ID |  |  |  |  |
| >>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

-	Option 3: UE performance per S-NSSAI

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE performance List |  | 0..1 |  |  |
| > Slice UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

During the discussion in the last meeting, there is a working assumption that UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. Upon reviewing this working assumption, we believe that UE performance per PDU session per S-NSSAI is also not an appropriate granularity. According to the SA5 definition, UE performance at the slice level can be measured and calculated, but it does not explicitly define how to measure UE performance at the per PDU session level. Therefore, we consider the most straightforward approach to be directly transmitting UE performance at the per-slice level (i.e., option 3).

- SA5 specification only provides a calculation granularity of UE performance per slice instead of per PDU session.

From another perspective, the source node needs to compare the slice-level UE performance for this UE before and after the handover. Therefore, it is essential to ensure that the source node and target node use consistent measurement and calculation methods, enabling the source node to make a valid comparison.

- The source and target nodes need to maintain a consistent measurement and calculation method to ensure that the source node can interpret the reported UE performance and make comparisons accurately.

- Modify the WA to: UE performance per S-NSSAI is reported from the target node to the source node. And turn it to agreement.

Corresponding TP to BL CR 38.423 is provided in R3-247139.

3	Conclusion

Following is our proposals and observations:

Predicted information exchange:

- Introduce Slice Measurement Initiation Result List IE in the DATA COLLECTION UPDATE message to indicate the predicted radio resource status per slice or predicted slice available capacity can not be initiated.

- First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status.

- Introduce new bit for predicted slice available capacity in the Report Characteristics for Data Collection IE.

- Modify the description of the Predicted Resource Status IE to “The IE includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE.”

- Predicted slice available capacity IE is introduced in Cell Info Result for Data Collection Item IE and refers to the Slice Available Capacity IE.

Per Slice UE performance:

- Average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

- SA5 specification only provides a calculation granularity of UE performance per slice instead of per PDU session.

- The source and target nodes need to maintain a consistent measurement and calculation method to ensure that the source node can interpret the reported UE performance and make comparisons accurately.

- Modify the WA to: UE performance per S-NSSAI is reported from the target node to the source node. And turn it to agreement.

Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] TS 28.552