3GPP TSG-RAN WG3 Meeting #126	R3-247144

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on requirements for Continuous MDT solution

Agenda item:	11.4

Source:	Telecom Italia, Jio, BT, Deutsche Telekom

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #125bis, the Rel-19 WI on Enhancements for AI/ML for NG-RAN started, also including a set of Rel-18 leftovers [1], based on the conclusions captured in TR 38.743 [2]. Solutions for Continuous MDT collection targeting the same UE across RRC states is part of such leftovers, and the following agreement was reached at the last RAN3 meeting.

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Based on the above agreement, management-based MDT is considered for Continuous MDT, where UEs selection is performed by RAN nodes. For the Continuous MDT to be actually feasible, the network should be able to re-select the same UE (previously selected by a RAN node) after either mobility in RRC_CONNECTED mode (i.e., handover) or after transition from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED. However, there are currently no standardized means for the network to re-select again the same UE for subsequent MDT data collection.

In this contribution we provide our views on requirements that should be considered for the selection of a solution to support Continuous MDT.

# 2	Discussion

Minimization of Drive Test (MDT) campaigns play an important role for network operators, as they allow for the acquisition of detailed information regarding the state of the network. Through these campaigns operators can monitor network performance and identify critical areas that require optimization. However, to ensure reliable information and draw meaningful conclusions, it is essential to have a sufficient number of measurements.

The more data collected through MDT campaigns, the greater the accuracy of analyses and strategic decisions. It is therefore crucial to expand the number of measurements taken since a broader statistical base provides a clearer and more detailed view of network performance. However, one of the current limitations is the availability of UEs that support MDT functionality, where less than 10% of UEs can be exploited (and some of them also without related positioning reporting). The limited number of compatible devices can negatively impact the effectiveness of the campaigns, reducing the variety and scope of the collected data.

Observation 1: It is important to have a large number of terminals that support MDT to obtain reliable statistics on network performance.

The introduction of enhancements for MDT represents a significant strategic opportunity for operators, as it contributes to a better characterization of the network and the traffic it manages. These innovations allow for the collection of more accurate and detailed data, thereby facilitating in-depth analyses and informed strategic decisions. However, to ensure the effectiveness of these campaigns, it is crucial that the new functionalities do not limit the number of devices that can be exploited.

For this purpose, the introduction of the Continuous MDT functionality should not be limited to Rel-19 terminals only. It is essential that it is also backwards compatible with legacy terminals that currently already support MDT. This approach would not only increase the number of available measurements but also enrich the scope of collected data, thereby allowing for a more comprehensive and representative assessment of network performance. Only through greater availability of devices supporting MDT it is possible to ensure continuous and reliable improvements in the quality of services offered to users and in the management of network resources.

Proposal 1: RAN3 to agree to introduce the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

In today's context, where technological innovation advances at a rapid pace, it is also essential that the deployment of new features occurs with reduced complexity and sustainable costs. This approach not only facilitates the adoption of innovative solutions but also ensures their effective integration into existing networks. It follows that, also for Continuous MDT, it is important to analyse the various solutions, among the ones ensuring compatibility with legacy devices, by evaluating the relevant complexity so to identify and select the one with less complexity and reduced specification impact.

Proposal 2: RAN3 to agree to support a Continuous MDT solution, among the ones applicable to legacy UEs, with reduced complexity and specification impact.

# 3	Conclusion

In this paper we provide the following proposals:

Proposal 1: RAN3 to agree to introduce the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

Proposal 2: RAN3 to agree to support a Continuous MDT solution, among the ones applicable to legacy UEs, with reduced complexity and specification impact.

# 4	References

[1]	RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, 3GPP RAN#105

[2]	3GPP TR 38.743, “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (Rel.19)