3GPP TSG-RAN WG3 Meeting #126	R3-247164

Orlando, US, 18 - 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0442 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SDT network-based solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Ericsson, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The network-based solution was discussed in RAN3 to support the scenario in which the network moves the UE to RRC_CONNECTED mode by triggering RRC setup towards the UE, in case e.g., DL non-SDT data arrive during an on-going RA-SDT without anchor relocation (after SDT partial context transfer). In this network-based solution, there is a need to add new indication in XnAP: RETRIEVE UE CONTEXT RESPONSE to trigger the receiving gNB to send RRCSetup towards the UE, and update procedural text and stage2 call flows accordingly. In case of CP UP separation, if the other gNB-CU-CP further provides the full UE context, the gNB-CU-CP triggers Bearer Context Setup towards the gNB-CU-UP, triggers UE Context Modification towards the gNB-DU, and sends RRCSetup message to move the UE to RRC_CONNECTED state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of network-based solution into the existing RACH based SDT section. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it introduces network-based solution for SDT, to allow the gNB to use another way to terminate the ongoing SDT procedure and moves the UE to RRC_CONNECTED mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support network-based solution in case of split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 draftCR TS 38.423 CR 1427 TS 38.473 CR 1527 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.18.1	RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.18.1-1.

Figure 8.18.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends the RRCResumeRequest message together with UL SDT data and/or UL SDT signalling.

2.	The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

3.	The step 3 is as defined in step 4 in clause 8.6.2, including an indication of SDT access. The gNB-DU may also provide SDT assistance information.

4-5.	If UE context is successfully retrieved as specified in TS 38.300 [2], the steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2. The UL SDT data, if any, is forwarded to the gNB-CU-UP, and the UL signalling, if any, is forwarded to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1:	In case that full UE context is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP first establishes the UE context in the gNB-CU-UP via the Bearer Context Setup procedure and F1-U UL TEIDs are retrieved before step 4. The BEARER CONTEXT SETUP REQUSET message may include an indication to suspend non-SDT bearers, and in this case, the BEARER CONTEXT MODIFICATION REQUEST message in step 6 does not include resume indication for SDT DRBs.

NOTE 2:	In case that only partial UE context for SDT including F1-U UL TEIDs is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP uses those F1-U UL TEIDs for steps 4-5, and the subsequent steps 6-7 are not executed. The F1-U DL TEIDs received from the gNB-DU in step 5 should be forwarded to the other gNB-CU-CP, to be used for transferring of the DL SDT data. In addition, the UL SDT data, if any, is forwarded from the gNB-DU to the gNB-CU-UP of the other gNB-CU-CP for which the partial context is retrieved, and the UL signalling, if any, is forwarded from the gNB-CU-CP to the other gNB-CU-CP (the last serving gNB-CU-CP) via the XnAP RRC TRANSFER message. And if the other gNB-CU-CP further provides the full UE context as specified in TS38.300 [2], the gNB-CU-CP triggers Bearer Context Setup towards the gNB-CU-UP, triggers UE Context Modification towards the gNB-DU, and sends RRCSetup message to move the UE to RRC_CONNECTED state.

NOTE 3:	The other gNB-CU-UP may need to buffer the UL SDT data if received before the SDT bearer(s) are resumed.

6.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message including an resume indication for SDT DRBs. The gNB-CU-CP also includes the F1-U DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-UP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.

NOTE 4:	void.

Upon receiving the UE INACTIVITY NOTIFICATION message without SDT volume threshold crossed indication from the gNB-DU, the gNB-CU, if serving the UE and deciding to terminate the ongoing SDT procedure, shall transmit the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

If CG-SDT is (re-)configured, the gNB-CU may request the gNB-DU to keep CG-SDT configuration and resources in the UE CONTEXT RELEASE COMMAND message.

NOTE 5:	void.

Upon receiving BSR from the UE, in case that UL SDT data size in the BSR is larger than the threshold configured from the gNB-CU-CP, the gNB-DU sends the UE INACTIVITY NOTIFICATION message with the SDT volume threshold crossed indication to the gNB-CU-CP. Upon receiving such indication, the gNB-CU-CP may terminate the ongoing SDT procedure, by sending the RRCResume message to move the UE to RRC_CONNECTED, or by sending the RRCRelease message to move the UE to RRC_INACTIVE, or by sending the RRCRelease message with resume indication to trigger the UE to initiate RRC Resume procedure to move to RRC_CONNECTED state as specified in TS 38.300 [2].

Upon receiving non-SDT data, the gNB-CU-UP shall send the DL DATA NOTIFICATION message to the gNB-CU-CP. The gNB-CU-CP shall terminate the ongoing SDT procedure as specified in TS 38.300 [2].

If the amount of the received DL SDT data is above the data size threshold configured by the gNB-CU-CP, the gNB-CU-UP shall send the DL DATA NOTIFICATION message with the SDT data size threshold crossed indication. The gNB-CU-CP may terminate the ongoing SDT procedure as specified in TS 38.300 [2].