3GPP TSG RAN WG3 Meeting #126	R3-247167

Orlando, U.S.A., 18 - 22 November, 2024

Title: 	Discussion on the support of semi-persistent CSI-RS resource for LTM candidate cells

Source: 	ZTE Corporation

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

In this meeting, RAN3 received an LS (R3-247006/R1-2409283) from RAN1, detail as below.

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective 1. Overall Description: The following working assumption was made in RAN1#118bis for candidate cell L1-RSRP measurement based on CSI-RS: RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case, and hence RAN3’s view is needed to confirm the working assumption. Therefore, RAN1 would like to ask RAN3 for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

This paper tries to answer the question raised in this LS.

- Discussion

In the last RAN3 #125bis meeting, we have the agreement as below.

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

However, different from SSB information, CSI-RS information is a UE-specific RS other than a cell-specific RS, so it shall use UE-associated XnAP/F1AP signalling.

CSI-RS configuration

Because CSI-RS information shall be configured before LTM execution phase, source gNB shall request the candidate gNB(s) to provide CSI-RS for specified UE in the Xn: Handover request message, then the candidate gNB(s) provide the CSI-RS via Xn: Handover request acknowledge message.

Observation 1: CSI-RS information is a UE-specific RS, it can be configured before LTM execution, by Xn: Handover procedure.

CSI-RS activation

Similar to SSB, CSI-RS is also used for early synchronisation. More, it shall be activated before LTM execution.

In detail, the source gNB shall coordinate with each of candidate gNBs to activate full/part of configured CSI-RS, then indicate UE to perform L1-RSRP measurement by MAC-CE.

Observation 2: source gNB shall coordinate with candidate gNB to active full/part of configured CSI-RS information, by Xn signalling/procedure.

CSI-RS deactivation

The activated CSI-RS will consume network/UE’s resource and power, so that, it is necessary to deactivate it if not used, so that, the activated CSI-RS shall be deactivated after LTM execution

Moreover, CSI-RS activation is triggered by source gNB, thus, for subsequence LTM, it is the new source gNB to (re)activate CSI-RS if needed.

Observation 3: The activated CSI-RS shall be deactivated after LTM execution.

Observation 4: After LTM execution, other candidate gNB(s) shall be informed of LTM execution and new source node, then it/they can deactivate CSI-RS if activated. How to inform other candidate gNB(s) of subsequent LTM execution is still discussing in RAN3.

- Summary

Based on above analysis, RAN3’s normative work and workload are acceptable.

Meanwhile, since both RAN2 and RAN1 have already studied how to support periodic CSI-RS, semi-persistent CSI-RS, it is valuable for RAN3 to support it.

Proposal 1: RAN3 shall support CSI-RS in Rel-19, to align with other group’s normative work.

Proposal 2: RAN3 shall respond RAN1 that is the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s).

- Conclusion

Based on the above analysis, we have the following observations and proposals.

Observation 1: CSI-RS information is a UE-specific RS, it can be configured before LTM execution, by Xn: Handover procedure.

Observation 2: source gNB shall coordinate with candidate gNB to active full/part of configured CSI-RS information, by Xn signalling/procedure.

Observation 3: The activated CSI-RS shall be deactivated after LTM execution.

Observation 4: After LTM execution, other candidate gNB(s) shall be informed of LTM execution and new source node, then it/they can deactivate CSI-RS if activated. How to inform other candidate gNB(s) of subsequent LTM execution is still discussing in RAN3.

Proposal 1: RAN3 shall support CSI-RS in Rel-19, to align with other group’s normative work.

Proposal 2: RAN3 shall respond RAN1 that is the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s).