3GPP TSG RAN WG3 Meeting #126	R3-247174

Orlando, U.S.A., 18 - 22 November, 2024

Title: 	Discussion on L3 measurement based LTM support over F1

Source: 	ZTE Corporation

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

In this meeting, RAN3 received a reply LS (R3-247013/ R2-2409373) from RAN2, detail as below.

1. Overall Description: RAN WG2 would like to thank RAN WG3 for their LS in R2-2409382. RAN WG2 understands that early synchronisation is also applicable to L3 measurements based LTM. It is also RAN WG2 understanding that for L3 measurements based LTM, the CU decides early synchronisation according to the results of L3 measurements and indicates that decision with the corresponding beam information to a DU and then the DU triggers early synchronisation. 2. Actions: RAN2 kindly asks RAN3 to take the information above into account.

This paper addresses to provide a method for L3 measurement based LTM support over F1

- Discussion

Regarding how to support LTM triggering based on L3 measurements, there are two option:

- Option 1: The CU decides the target candidate cell based on L3 measurement results, and inform the selected target candidate cell to the DU via F1 interface;

- Option 2: The CU sends L3 measurement results to the DU via F1 interface, and the DU selects the target candidate cell based on the received L3 measurement results.

In option 1, the CU can directly decide the target candidate cell according to received L3 measurement results, similar to the legacy L3 handover. And then the CU informs the selected target candidate cell to the DU, to enable the DU generate and send the LTM cell switch MAC CE to the UE. In this way, only one new IE is required to be introduced in an F1AP message (e.g. UE context modification message, CU-DU cell switch notification message, or a new F1AP message), to indicate the target candidate cell ID (e.g. the candidate configuration index, or NR CGI).

In option 2, the L3 measurement results for all related candidate cells need to be transferred to the DU completely. Considering that there is no F1AP message to transfer such L3 measurements from the CU to the DU in the current spec, a new F1AP message shall be required to include the L3 measurement results of candidate cells. The message should include the candidate cell ID, and the corresponding SSB or CSI-RS based L3 measurement results (e.g.  RSRP, RSRQ and SINR measurements). Compared to option 1, larger signalling overhead via F1AP is required in option 2. Besides, the F1AP message carrying L3 measurement results may need to be sent frequently, e.g. every time when the CU receives the L3 measurement report from the UE, considering that L3 measurement results on candidate cells may be triggered at different time point and one report may also include results on partial candidate cells.

Early Synch

In the last meeting, RAN3 mainly discussed whether the early synchronization should be supported or not for L3 measurements based LTM and sent an LS (R3-245710) to RAN2. Latter, RAN2 responded in reply LS (R3-247013/ R2-2409373) that the CU decides early synchronisation according to the results of L3 measurements and indicates that decision with the corresponding beam information to a DU and then the DU triggers early synchronisation.

Observation 1: RAN2 agreed that the CU does not need to transfer to the results of L3 measurements to candidate DU for decision on the early synchronisation

Observation 2: RAN2 agreed that it is the CU to decide early synchronisation according to the results of L3 measurements.

Proposal 1: The CU can according to the results of L3 measurements, indicate candidate DU(s) to perform early synchronisation by F1AP signalling.

LTM execution

Since the source CU decides early synchronisation, it is straightforward that the source CU also decides the target candidate cell according to the results of L3 measurements.

Proposal 2: The CU can according to the results of L3 measurements, decide the target candidate cell to perform LTM execution.

Coexistence between L1 measurement results and L3 measurement results

Proposal 3: The source gNB-DU can initiate Early TA acquisition either by its own decision or by receiving indicating from gNB-CU.

Proposal 4: Introduce a new Class 2 message, i.e., LTM Handling Notification, which is sent from gNB-CU to gNB-DU.

- Conclusion

Based on the above analysis, we have the following observations and proposals.

Observation 1: RAN2 agreed that the CU does not need to transfer to the results of L3 measurements to candidate DU for decision on the early synchronisation

Observation 2: RAN2 agreed that it is the CU to decide early synchronisation according to the results of L3 measurements.

Proposal 1: The CU can according to the results of L3 measurements, indicate candidate DU(s) to perform early synchronisation by F1AP signalling.

Proposal 2: The CU can according to the results of L3 measurements, decide the target candidate cell to perform LTM execution.

Proposal 3: The source gNB-DU can initiate Early TA acquisition either by its own decision or by receiving indicating from gNB-CU.

Proposal 4: Introduce a new Class 2 message, i.e., LTM Handling Notification, which is sent from gNB-CU to gNB-DU.

We also provide both stage 2/stage 3 CRs and draft LS, to support L3 measurement based LTM support over F1.