3GPP TSG RAN WG3 Meeting #126	R3-247177

Orlando, U.S.A., 18 - 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0156 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support L3 measurement based LTM support over F1 - TS38.470 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the previous RAN2 #127 meeting and RAN2 #127bis meeting, RAN2 sent LS R3-245008/ R2-2407606 and LS R3-247013/ R2-2409373 on L3 measurement based LTM support over F1. Regarding how to support LTM triggering based on L3 measurements, there are two options: - Option 1: The CU decides the target candidate cell based on L3 measurement results, and inform the selected target candidate cell to the DU via F1 interface; - Option 2: The CU sends L3 measurement results to the DU via F1 interface, and the DU selects the target candidate cell based on the received L3 measurement results. After discussion, RAN3 agreed to use Option 1 to support L3 measurement based LTM support over F1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Class 2 procedure named LTM Handling Notification, to be sent from the gNB-CU to the gNB-DU. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change affects only the LTM function. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L3 measurement based LTM support over F1 cannot be supported in split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1521 TS 38.401 CR 0443 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------Start of the change---------------------------------

6.1.2	Context Management procedures

The F1 Context management procedures are listed below:

-	UE Context setup procedure;

-	UE Context Release Request (gNB-DU initiated) procedure;

-	UE Context Release (gNB-CU initiated) procedure;

-	UE Context Modification (gNB-CU initiated) procedure;

-	UE Context Modification Required (gNB-DU initiated) procedure;

-	UE Inactivity Notification procedure;

-	Notify procedure;

-	Access Success procedure;

-	DU-CU Cell Switch Notification procedure;

-	CU-DU Cell Switch Notification procedure;

-	LTM Handling Notification procedure.

-----------------------End of the change---------------------------------