3GPP TSG RAN WG3 Meeting #126	R3-247179

Orlando, U.S.A., 18 - 22 November, 2024

Title: 	Discussion on SDT signalling optimization for partial context transfer

Source: 	ZTE Corporation

Agenda item:	8.3

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #25bis meeting, we have discussed the network based solution on SDT signalling optimization for partial context transfer, then made the following progress.

Whether MAC reset is needed for network-based solution needs to be further checked. No issue on AMF redirecting NG connections (i.e., NG connection from old gNB to new gNB), reuse the legacy procedure. Call flow may need to be checked. No issue for DL data loss by data forwarding. The network based solution does not apply to UL data/signaling and DL MBS configuration scenario. To be continued in AI8.3.

This paper discusses whether there is any issue yellow highlight as above.

- Discussion

  - Signalling procedure for network-based solution

The following figure 1 is updated from R3-245677.

Figure 1. Overall procedure for Network-based solution

For step 9b, it can refer to the legacy signalling procedure in TS38.300, as below.

[TS 38.300] The following figure describes the UE triggered transition from RRC_INACTIVE to RRC_CONNECTED in case of UE context retrieval failure: Figure 9.2.2.4.1-2: UE triggered transition from RRC_INACTIVE to RRC_CONNECTED (UE context retrieval failure) 1. The UE resumes from RRC_INACTIVE, providing the I-RNTI, allocated by the last serving gNB. 2. The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context data. 3. The last serving gNB cannot retrieve or verify the UE context data. 4. The last serving gNB indicates the failure to the gNB. 5. The gNB performs a fallback to establish a new RRC connection by sending RRCSetup. 6. A new connection is setup as described in clause 9.2.1.3. Figure 9.2.1.3-1: UE triggered transition from RRC_IDLE to RRC_CONNECTED

Observation 1: For network-based solution, after receiving RRCSetup message (step 8b in figure 1), the UE is changed into RRC connected and CM-IDLE mode, and performs step 2a-9 described in Figure 9.2.1.3-1 in TS38.300.

After new RRC connection is established between UE and Network, for the same UE, another NG connection between receiving gNB and CN is established.

[TS23.502] 4.2.3.2 UE Triggered Service Request ------------------<Skip unrelated part>-------------------- The AMF may receive a Service Request to establish another NAS signalling connection via a new NG-RAN while it has maintained an old NAS signalling connection for UE still via an old NG-RAN. The new NG-RAN and the old NG-RAN can be the same NG-RAN node. In this case, AMF shall trigger the AN release procedure toward the old NG-RAN to release the old NAS signalling connection as defined in clause 4.2.6 and: - For the PDU Sessions indicated by the UE in the List Of PDU Sessions To Be Activated, the AMF requests the SMF to activate the PDU Session(s) immediately by performing this step 4; and NOTE 5: This activates the UP of PDU Session(s) using resources of the new NG-RAN. - For the PDU Sessions indicated by the old NG-RAN in the "List of PDU Session ID(s) with active N3 user plane" but not in the List Of PDU Sessions To Be Activated sent by the UE, the AMF requests the SMF to deactivate the PDU Session(s). NOTE 6: This deactivates the UP of PDU Session(s) that are no more needed by the UE.

Observation 2: For network-based solution, it is allowed for CN to establish two NG connections for this UE. However, in this case, the CN shall first trigger AMF initiated UE Context Release procedure to release the old gNB (i.e., last serving gNB). Thus, the CN first releases the old NAS signalling connection before establishing a new one.

  - Support of data forwarding

In the previous meeting, RAN3 common understand that DL data forwarding shall be supported for network-based solution.

Based on this common understanding, for both last serving gNB (i.e., old gNB) and receiving gNB (i.e., new gNB), they shall be configured the same PDU session/QoS flow profile.

[TS38.331] 5.3.3.4 Reception of the RRCSetup by the UE The UE shall perform the following actions upon reception of the RRCSetup: 1> if the RRCSetup is received in response to an RRCReestablishmentRequest; or 1> if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1: 2> if the UE is NCR-MT: 3> indicate to NCR-Fwd to cease forwarding; 2> if sdt-MAC-PHY-CG-Config is configured: 3> instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running; 3> instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG indicated by tag-Id, if it is not running; 2> if srs-PosRRC-Inactive is configured: 3> instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running; 2> if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 3> instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT, if it is running; 2> if the UE is configured to receive MBS multicast in RRC_INACTIVE: 3> reset MAC; 2> discard any stored UE Inactive AS context and suspendConfig; 2> discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key; 2> release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP; 2> release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs; 2> indicate to upper layers fallback of the RRC connection; ------------------<Skip unrelated part>--------------------

[TS23.502] 1. UE to (R)AN: AN message (AN parameters, Service Request (List Of PDU Sessions To Be Activated, List Of Allowed PDU Sessions, security parameters, PDU Session status, 5G-S-TMSI, [NAS message container], Exempt Indication, [Release Request indication], [Paging Restriction Information], [Reject Paging Indication])). The NAS message container shall be included if the UE is sending a Service Request message as an Initial NAS message and the UE needs to send non-cleartext IEs, see clause 4.4.6 of TS 24.501 [25]. 2. (R)AN to AMF: N2 Message (N2 parameters, Service Request). ------------------<Skip unrelated part>-------------------- 11. [Conditional] SMF to AMF: Nsmf_PDUSession_UpdateSMContext Response (N2 SM information (PDU Session ID, QFI(s), QoS profile(s), CN N3 Tunnel Info, S-NSSAI, User Plane Security Enforcement, UE Integrity Protection Maximum Data Rate, RSN, PDU Session Pair ID), N1 SM Container, Cause) to the AMF. If the UPF that connects to RAN is the UPF (PSA), the N3 CN Tunnel Info is the UL CN Tunnel Info of the UPF (PSA). If the UPF that connects to RAN is the new intermediate UPF, the CN N3 Tunnel Info is the UL Tunnel Info of the intermediate UPF. 12. AMF to (R)AN: N2 Request (N2 SM information received from SMF, security context, Mobility Restriction List, UE-AMBR, List of UE-Slice-MBR(s) (optional and for 3GPP access type only), MM NAS Service Accept, list of recommended cells / TAs / NG-RAN node identifiers, UE Radio Capability, Core Network Assistance Information, Tracing Requirements, UE Radio Capability ID). The Allowed NSSAI for the Access Type for the UE and Partially Allowed NSSAI are included in the N2 message. If the subscription information includes Tracing Requirements, the AMF includes Tracing Requirements in the N2 Request.

UE side

UE is changed into CM-IDLE mode, it will discard any stored UE context and release all of DRBs. So, when it sends Service request message to CN via NAS-PDU, it cannot be guarantee that new PDU session profile is the same with the old one within Service request message.

Observation 3: For network-based solution, UE cannot guarantee that PDU session profile in this Service Request message is the same with that in the previous Service Request message.

CN side

Although CN can establish two NG connection with this UE, it will establish new PDU session/QoS flow profile and new UE context for this UE. Thus, from the network perspective, a new session is created and hence association between the old data and the new session are lost on the network side.

It should also be noted that whilst the RRCSetup procedure is initiated, the UE may initiate new services and hence piggy back entirely new service request procedure in the initial UE message in UL. Thus, retaining the old PDU session/QoS flow profile is not possible anymore and hence the CN cannot guarantee that the new PDU/QoS flow profile at new gNB is the same with the old PDU/QoS flow profile at old gNB.

Observation 4: For network-based solution, CN cannot guarantee that PDU session/QoS flow profile in the new gNB is the same with that in the old gNB.

Proposal 1: RAN3 can send LS to SA2/CT1 to check observation 3/4.

Since both UE and CN cannot guarantee to establish the same PDU session/QoS flow profile with the old one, DL data forwarding cannot be supported.

Observation 5: Since both UE and CN cannot guarantee to establish the same PDU session/QoS flow profile with the old one, the data forwarding cannot be always supported.

Proposal 2: If observation 5 is correct, RAN3 shall not agree with network-based solution.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: For network-based solution, after receiving RRCSetup message (step 8b in figure 1), the UE is changed into RRC connected and CM-IDLE mode, and performs step 2a-9 described in Figure 9.2.1.3-1 in TS38.300.

Observation 2: For network-based solution, it is allowed for CN to establish two NG connections for this UE. However, in this case, the CN shall first trigger AMF initiated UE Context Release procedure to release the old gNB (i.e., last serving gNB). Thus, the CN first releases the old NAS signalling connection before establishing a new one.

Observation 3: For network-based solution, UE cannot guarantee that PDU session profile in this Service Request message is the same with that in the previous Service Request message.

Observation 4: For network-based solution, CN cannot guarantee that PDU session/QoS flow profile in the new gNB is the same with that in the old gNB.

Observation 5: If both UE and CN cannot guarantee to establish the same PDU session/QoS flow profile with the old one, the data forwarding cannot be always supported.

Proposal 1: RAN3 can send LS to SA2/CT1 to check observation 3/4.

Proposal 2: If observation 5 is correct, RAN3 shall not agree with network-based solution.

For proposal 1, the corresponding draft LS is R3-247180.