3GPP TSG-RAN WG3 Meeting #126          	          R3-247188

Orlando, USA, 18 Nov - 22 Nov 2024

Title: 	(TP for TR38.769) Locating an AIoT device

Source: 	ZTE Corporation

Agenda item:	16.4

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #124 meeting, we have achieved the following progress and have agreed the pCR in R3-243972.

Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. Use cases for locating an AIoT device: - Find an appropriate “reader” close to the A-IoT device; - Find where the A-IoT device is. Support locating the A-IoT device at “reader” granularity. RAN3#124: AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. How to know the “reader” location is FFS. Whether and how to utilize more than one “readers” for location purposes is FFS depending on involved complexity. Signalling details on how to provide the location information to the A-IoT CN node.

In the last RAN3 #125 meeting, a bit progress on Locating an AIoT device is agreed to capture in the pCR R3-244840.

This paper further discusses the issue on locating AIoT device.

- Discussion

  - How to know the “reader” location

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

AIoT device location information is mainly used for the upcoming CN initiated inventory/command within an accurate area other than a blindly large area. So that, the AIoT device location information is benefit to store it at AIoT CN.

Before CN initiates inventory/command procedure, it shall select suitable readers. It means CN shall know each AIoT device to be inventoried/commanded, is controlled by which reader, i.e., establish/store association between each device and reader. This association is within device context, stored in the CN, i.e., device context includes {device id, reader id} association. The reader id is UE id or gNB/BS/cell/TRP id.

Through inventory procedure, reader can respond inventory result and forward it to the AIoT CN.

If the “reader” location is already acquired by the AIoT CN, through the new inventory procedure, the “reader” location can be updated.

Proposal 1: The AIoT CN shall acquire, store and update the “reader” location by inventory procedure.

According to the agreed RAN3 agreement as below, before CN initiates command procedure, it shall know the reader location as well as the association between reader and device, i.e., the reader id/reader id pair.

(a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

Based on above, reader shall provide its latest location to the AIoT CN. Otherwise, the AIoT CN cannot initiate command procedure with the suitable reader.

Observation 1: Before initiating command procedure, the AIoT CN shall select the suitable reader.

In the SA2 discussion, it is suggested that AIoT CN always firstly initiates inventory, after receiving the latest “reader” location, and then it secondly initiates command procedure.

It is conservative way, however, if the AIoT CN always stores the latest “reader” location, it can actually initiate command procedure directly. For this reason, the reader shall autonomously provide its latest location to the AIoT CN.

For example, the reader is a UE. The UE (reader) can move to another area. In this case, the UE (reader) shall initiate a new inventory procedure and then provide reader location and association between reader and device to the AIoT CN.

Proposal 2: Reader can autonomously initiate inventory procedure to provide its latest location to the AIoT CN.

  - Signalling details on how to provide the location information to the A-IoT CN node.

As we discuss as above, location information is provided by inventory procedure, we prefer to reuse the inventory signaling other than dedicate signaling.

Proposal 3: The reader responds inventory result to the AIoT CN, including its location.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: Before initiating command procedure, the AIoT CN shall select the suitable reader.

Proposal 1: The AIoT CN shall acquire, store and update the “reader” location by inventory procedure.

Proposal 2: Reader can autonomously initiate inventory procedure to provide its latest location to the AIoT CN.

Proposal 3: The reader responds inventory result to the AIoT CN, including its location.

- Text Proposal for TR38.769

================================Start of change===============================

6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

The AIoT CN shall acquire, store and update the “reader” location by inventory procedure.

Optional, the reader can autonomously initiate inventory procedure to provide its latest location to the AIoT CN

For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device.

Editor’s Note 1: Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysing the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

================================End of change===============================