3GPP TSG-RAN WG3 Meeting #126	R3-247205

Orlando, US, 18th – 22nd November 2024

Agenda Item:		11.4

Source:		NEC

Title:			Continuous MDT collection for AI/ML

Document for:		Discussion and decision

# Introduction

In RAN#105 meeting, a new WI on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN was approved based on the study results from SI and the detailed objectives are captured as below [1]:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]:

  - Mobility Optimization for NR-DC

  - Split architecture support for Rel-18 use cases

  - Continuous MDT collection targeting the same UE across RRC states

Regarding continuous MDT collection, two problems for continuous MDT collection, as below, were captured in the TR38.743 [2].

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

This contribution further discusses the above two items, the possible solutions and the corresponding impacts on RAN3 specification.

# Discussion

In Rel-18 discussion, RAN3 reached the following agreement on continuous MDT collection:

The existing MDT framework is used as baseline for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

According to TR37.817 [3] and TR38.743 [4], the following data need to be collected from UE for AI/ML:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

- UE Mobility History Information

In order to limit the impact on UE power consumption and processing, continuous collection and logging of UE measurement should as much as possible rely on the measurements that are already supported in UE and so we only consider to continuous collect the above data.

## 2.1	Problem A (measurement continuity)

We already agreed that “Continuous collection of MDT traces is beneficial only for AI/ML training in OAM.”. Training data is the initial dataset used to train machine learning algorithms on how to make predictions. In order to increase the precision of a ML model, the process of training an ML model needs large volume of input data and a rather long training time. And the provision of training data is not time critical.

As in legacy, immediate MDT can be configured to UE for measurements in RRC Connected, while logged MDT in – RRC Idle and RRC Inactive. Regarding to the gap during RRC state transition, it is a extremely short period of time comparing to the long training time and the measurements during the transition can be negligible.

Besides, the collected raw data may still need to be processed and translated into usable information for training by data processing technology, which can also handle the sparse missing data issue, due to e.g. RRC state transition or other reasons.

Therefore, we think that, for AI/ML training, the measurement continuity during the same RRC state can be guaranteed in legacy way and the measurement discontinuity during RRC state transition can be negligible.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

## 2.2	Problem B (trace correlation)

RAN3 sent an LS to SA5 [5] to enquire whether S-TMSI as a “Measured UE Identifier” in TS28.558 that is included in all the UE level measurements can be used to identify which UE sends the measurement report and to do the trace correlation in TCE.

In the reply LS [6], SA5 clarified that

“So, the “Measured UE Identifier” parameter specified in TS28.558 is not applicable to NG-RAN UE level (MDT) measurements. Therefore, SA5 has agreed to add a clarification in TS28.558 that the measurement bullet g) is only applicable for management-based activation of 5GC UE level measurement collection, and the bullet g) in NG-RAN UE level measurements are set to “N/A” instead of “S-TMSI…”

Besides, SA5 also mentioned that

“SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE.”

As confirmed by SA5, an identifier that is unique within the lifetime of the measurement session is needed to indicate the measurement was done in the same or different UE.

5G-S-TMSI as a possible identifier has been discussed but the following concerns raised [5]:

- 5G-S-TMSI is not available in the gNB-DU and in the gNB-CU-UP.

- 5G-S-TMSI may not be always available in the gNB-CU-CP.

- 5G-S-TMSI can not be used if inter-AMF mobility happens.

Therefore, to address this issue, we think OAM or AMF can allocate a specific UE identifier for continuous MDT and UE can include this identifier in measurement reports.

This UE identifier can also be included in logged MDT measurement configuration, which to implicitly request UE to perform continuous MDT, and then UE will continue the measurements across RRC states and even during RRC states transition.

Proposal 1: For continuous MDT, OAM or AMF allocates a specific UE identifier and UE can include it in measurement reports for trace correlation in TCE.

As we agreed in last meeting, “RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN”, we suggest to send an LS to SA5 and to inform them this agreement and ask them to finalize the solution for continuous MDT collection. This can be achieved by OAM e.g. to allocate a new UE identifier, to add a new Job Type value for continuous MDT mode, etc.

## 2.3 User consent

Comparing to legacy MDT, continuous MDT collection needs to set new requirements for UEs, e.g. continuous measurements collection capability, UE memory size, high energy consumption, etc, and users need to be asked and confirm the use of this function. Therefore, a new user consent is preferred to be used for continuous MDT or for specific use cases or specific UE measurements of continuous MDT.

Proposal 2: A new user consent is needed for continuous MDT.

A draft LS to SA5 capturing the abovementioned issues is included in Annex.

Proposal 3: To agree the LS to SA5 on continuous MDT collection in Annex.

# Proposals

In this contribution, we provide our further analysis on continuous MDT collection for AI/ML and the following observation and proposals were drawn from the analysis.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

Proposal 1: For continuous MDT, OAM or AMF allocates a specific UE identifier and UE can include it in measurement reports for trace correlation in TCE.

Proposal 2: A new user consent is needed for continuous MDT.

Proposal 3: To agree the LS to SA5 on continuous MDT collection in Annex.

# Reference

[1] R3-243952, (TP for TR 38.743) Continuous Management-based MDT Data Collection, Nokia, Huawei, Qualcomm, NEC, Samsung, ZTE, CMCC, InterDigital, Ericsson

[2] RP-242358, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[3] TR37.817, Study on enhancement for Data Collection for NR and EN-DC

[4] TR38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[5] R3-243941, Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics, RAN3

[6] R3-245021, Reply LS on improved KPIs involving end-to-end data volume transfer time analytics, SA5

# Annex LS to SA5

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, US, 18th – 22nd November 2024

Title:	[draft] LS on Continuous MDT collection

Response to:

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh

Source:	NEC (to be RAN3)

To:	SA5

Cc:

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In the Rel-18 and Rel-19 AI/ML for NG-RAN, RAN3 discussed how to support continuous MDT collection targeting the same UE across RRC states.

RAN3 reached the following agreements:

The existing MDT framework is used as baseline for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

And two problems for continuous MDT collection, as below, were captured in the TR38.743.

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

To address the above issues, some proposed enhancements may have OAM impacts. However, RAN3 lacks knowledge on whether such designs are possible and seeks to understand the following questions:

Question 1: For continuous MDT, whether OAM can allocate a specific UE identifier for UE to includes in measurement reports for trace correlation in TCE?

Question 2: As assumed by RAN3 that OAM configures Management Based MDT for Continuous MDT towards the NG-RAN, whether a new Job Type value for continuous MDT needs to be defined?

Question 3: Comparing to legacy MDT, continuous MDT collection sets new requirements for UEs, e.g. continuous measurements collection capability, UE memory size, high energy consumption, etc. Whether a new user consent is needed for continuous MDT or for specific use cases or specific UE measurements of continuous MDT?

ACTION: RAN3 kindly asks SA5 to provide answers to the questions above.

# 2	Actions

To SA5:

ACTION: RAN3 kindly asks SA5 to provide answers to the questions above.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127		2025-02-17 - 2025-02-21		Athens, GR

RAN3#127bis		2025-04-07 - 2025-04-11		Wuhan, CN