3GPP TSG-RAN WG3 Meeting #126	R3-247208

Orlando, US, 18th – 22nd November 2024

Agenda Item:		12.3

Source:		NEC

Title:			Open issues for NR Femto

Document for:		Discussion and decision

# Introduction

In RAN#105 meeting, a new WI on additional topological enhancements for NR was approved based on the study results from SI and the following objectives of 5G Femto are captured [1]:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

In last RAN3 meeting, a CR to TS38.300 [2] was agreed to include NR Femto architecture and protocol stacks.

In this contribution, we further discuss the open issues for NR femto, including functional split, protocol stacks and access control.

# Discussion

## 2.1	Functional split

A NR Femto hosts the same functions as a gNB, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	A NR Femto shall only connect to a single NR Femto GW at one time:

-	The NR Femto will not simultaneously connect to another NR Femto GW or another AMF.

-	The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto. Upon reception of the GUAMI from a UE, the NR Femto shall include it in the INITIAL UE MESSAGE message; upon reception of the GUAMI Type from the UE, the NR Femto shall also include it in the message if supported and supported by the NR Femto GW.

-	NR Femtos may be deployed without network planning. A NR Femto may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NG application part messages between the AMF serving the UE and the NR Femto serving the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto and towards the AMF.

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

-	Optionally terminating NG-U interface with the NR Femto and with the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

In case of open, closed and hybrid access modes are supported by NR Femto, the AMF hosts the following functions in addition to legacy functions:

-	Access control for UEs that are members of CAG.

-	Membership Verification for UEs in case of attachment to hybrid cells.

-	Membership status signalling to the NG-RAN in case of attachment to hybrid cells and in case of the change of membership status when a UE is served by a CAG cell or a hybrid cell.

-	Supervising the NG-RAN action after the change in the membership status of a UE.

-	In case of a NR Femto directly connected:

- verifying that the identity used by the NR Femto is valid when receiving the NG SETUP REQUEST message and determining whether the access mode of the NR Femto is closed or not;

- verifying, for a closed NR Femto, that the indicated cell access mode and CAG ID are valid when receiving the NG INITIAL UE MESSAGE message;

-	The AMF may send two transport layer addresses of different versions only in case of NR Femto GW which does not terminate user plane.

## 2.2	Interfaces

In the agreed CR [2], only Protocol Stacks for NG Control Plane is added. Below we also provide the Protocol Stack for NG User Plane and Xn User/Control Plane.

#### 2.2.1	Protocol Stack for NG User Plane

The NG-U data plane is defined between the NR Femto node, NR Femto GW and the UPF. The figures below show the NG-U protocol stack with and without the NR Femto GW  .

Figure 1: User plane for NG-U interface for NR Femto node without NR Femto GW

Figure 2: User plane for NG-U interface for NR Femto node with NR Femto GW

The NR Femto GW may optionally terminate the user plane towards the NR Femto node and towards the UPF, and relay User Plane data between the NR Femto node and the UPF.

#### 2.2.2	Protocol Stack for Xn User Plane and Xn Control Plane

The protocol stack for Xn User Plane and Xn Control Plane is reported in clause 7 of TS 38.420 [3].

Proposal 1: To include the description on functional split and protocol stacks for NG User Plane and Xn User/Control Plane in section 2.1 and section 2.2 to BLCR TS38.300.

## 2.3	Access control

As concluded in the SI phase [4], CAG mechanism is reused for NR Femtos deployments. With the existing CAG mechanism as described in TS38.300 clause 16.7.4, the open, hybrid and closed access modes, can be supported as follows:

-	To support the open access mode: The NR Femto activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

As for whether to define “open, hybrid and closed access modes”, it can depend on operators’ deployment requirements.

Proposal 2: Whether to define “open, hybrid and closed access modes” can depend on operators’ deployment requirements.

# Proposals

In this contribution, we provided considerations on functional split, protocol stacks and access control, and the following proposals were drawn:

Proposal 1: To include the description on functional split and protocol stacks for NG User Plane and Xn User/Control Plane in section 2.1 and section 2.2 to BLCR TS38.300.

Proposal 2: Whether to define “open, hybrid and closed access modes” can depend on operators’ deployment requirements.

# Reference

[1] RP-242395, New WID on additional topological enhancements for NR

[2] R3-245864, (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects

[3] TR 38.420, Xn general aspects and principles

[4] TR 38.799, Study on additional topological enhancements for NR