3GPP TSG-RAN WG3 Meeting #126	R3-247209

Orlando, US, 18th – 22nd November 2024

Agenda Item:		17.2

Source:		NEC

Title:			Support on-demand SSB operation

Document for:		Discussion and decision

# Introduction

One objective on on-demand SSB (OD-SSB) is captured in WID [1] as follows:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

In previous RAN3 meetings, we had some discussions on Xn impact and F1 impact but no conclusion due to lack of relevant agreements in other WGs. The following issues were listed FFS:

No Xn impact?

- Normal cell to request NES cell to provide on-demand SSB over Xn for layer3 measurement?

F1 impact?

- NES gNB-CU can indicate NES gNB-DU the allowed cell list for on-demand SSB?

- On-demand SSB operation is decided by CU or DU?

- The gNB-CU indicates the on-demand SSB mode to gNB-DU?

- The information provided from DU to CU, e.g., on-demand SSB state?

In this contribution, we further discuss the above issues and analyse potential RAN3 impacts.

# Discussion

## RAN1 agreements

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Below are the agreements made in RAN1#118 meeting that may influence RAN3 discussion.

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Below are the new agreements made in RAN1#118bis meeting.

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

  - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

  - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

  - FFS: Whether OD-SSB and always on SSB have same beam or not

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

## RAN2 agreements

Below are the agreements made in RAN2#127 meeting that may influence RAN3 discussion.

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Below are the new agreements made in RAN2#127bis meeting.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

## RAN3 impacts

## Xn impact

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger OD-SSB SCell operation and we think this should be an intra-gNB issue.

In last meeting, one issue was brought up about how to address L3 measurement of an OD-SSB Cell, whether NES cell can be triggered by a neighbor cell over Xn for OD-SSB activation? Considering the existing RAN1 agreement, On-demand SSB on the cell is not located on synchronization raster; On-demand SSB on the cell is non-cell-defining SSB. Besides, RAN2 is also working on how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration and their initial assumption is that UE does not expect to measure SSB when OD-SSB transmission is deactivated if no always-on SSB on the cell.

Therefore, from the above two points of view, no Xn impact is foreseen for OD-SSB SCell operation for now. But this issue can be kept FFS in case RAN1 or RAN2 reaching any new agreements that will change the above working assumptions.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

## F1 impact

RAN1 agreed to Support on-demand SSB SCell operation triggered by gNB, so in this case, for split architecture, it is an open issue about which node, e.g. gNB-CU or gNB-DU, can decide OD-SSB configuration and trigger OD-SSB transmission.

As in legacy, gNB-DU decides the SSB configuration, it is reasonable that OD-SSB configuration is still decided by gNB-DU.

Proposal 2: gNB-DU decides OD-SSB configuration.

Considering, after CA is set up, gNB uses MAC CE (SCell Activation/Deactivation) to activate or deactivate data transmission for SCell and, as RAN2 agreed, this MAC CE will also be used for OD-SSB indication for scenario 2A, gNB-DU is the suitable node to activate/deactivate OD-SSB transmission.

Proposal 3: gNB-DU activates/deactivates OD-SSB transmission.

Since RAN2 agreed that RRC based OD-SSB transmission indication can be used to indicate the activation/deactivation state of OD-SSB configuration, in this case, gNB-DU needs to send the OD-SSB activation/deactivation indication to gNB-CU.

Proposal 4: If RRC based OD-SSB transmission indication is used, gNB-DU needs to send the OD-SSB activation/deactivation indication for each SCell to gNB-CU.

Considering gNB-CU is more knowledgeable on the SSB operation status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance to gNB-DU for OD-SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest to gNB-DU e.g. OD-SSB allowed cells and, for each SCell, SSB operation mode, OD-SSB transmission periodicity, OD-SSB transmission duration, etc.

Proposal 5: gNB-CU can provide suggested OD-SSB operation information to gNB-DU to assist the OD-SSB operation.

# Proposals

In this contribution, we summarized the latest RAN1 and RAN2 progress on OD-SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

Proposal 2: gNB-DU decides OD-SSB configuration.

Proposal 3: gNB-DU activates/deactivates OD-SSB transmission.

Proposal 4: If RRC based OD-SSB transmission indication is used, gNB-DU needs to send the OD-SSB activation/deactivation indication for each SCell to gNB-CU.

Proposal 5: gNB-CU can provide suggested OD-SSB operation information to gNB-DU to assist the OD-SSB operation.

# Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR