3GPP TSG-RAN WG3 Meeting #126	R3-247210

Orlando, US, 18th – 22nd November 2024

Agenda Item:		17.3

Source:		NEC

Title:			Support on-demand SIB1

Document for:		Discussion and decision

# Introduction

In RAN#105 meeting, a way forward on on-demand SIB1 (OD SIB1) was made, i.e. support Case-2 as part of normative work on OD SIB1 for Rel-19 NES, and the WID was updated accordingly as below [1]:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In RAN3#125 meeting, the following agreements were achieved for case 2 of OD SIB1:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

In last RAN3 meeting, we further agreed that:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

This contribution further discusses the RAN3 impact of OD SIB1 based on the above study scope and agreements.

# Discussion

## UL WUS configuration provision

In case 2, a UE receives UL WUS configuration of a NES cell from Cell A, and then the UE sends an UL WUS to the NES cell to request it to transmit SIB1 for a specified duration. Once receiving the UL WUS, the NES cell can send SIB1 in the legacy way and for one or more legacy SIB1 periodicity.

Before the NES cell starts on demand SIB1 or when the UL WUS configuration changes, the NES cell needs to provide its UL WUS configuration to cell A. RAN3 has already reached the following agreement on inter-node exchange of UL WUS configuration from NES cell to Cell A.

UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed.

Regarding the procedure for UL WUS config exchange over Xn interface, we agreed to use a new class1 defined procedure. And upon the reception of the UL WUS configuration from NEC cell, Cell A can provide feedback to NES Cell on whether the WUS configuration transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). An example procedure is as below.

Figure 1: UL WUS Configuration Provision procedure

The purpose of this UL WUS Configuration Provision procedure is to request Cell A to broadcast the UL WUS configuration for the NES cell.

In the UL WUS Configuration Provision Request message from NES cell gNB to Cell A gNB, a list NES cell IDs and their related UL WUS configurations are included. In the UL WUS Configuration Provision Response message from Cell A gNB to NES cell gNB, the feedback on whether Cell A can accept the UL WUS Configuration Provision for each NES cells are included. Regarding the detailed IEs in the request message, we can discuss after RAN2 defining the UL WUS configuration parameters.

Proposal 1: To define an UL WUS Configuration Provision procedure over Xn for UL WUS configuration provision from NES cell to Cell A.

There is one possible scenario that multiple Cell A are applicable for a single NES cell. In this case, the NES cell can choose which Cell A for UL WUS configuration provision and it can be only one Cell A or some Cell A or all applicable Cell A. The requests to different Cell A can be sent one by one or at the same time, this can be left to implementation. If one of the receiving Cell A accepts the request, the NES cell can go to OD SIB1 Mode.

Proposal 2: In case of multiple Cell A for a single NES cell, NES cell can choose which Cell A for UL WUS configuration provision.

Proposal 3: If one of the Cell A that received the UL WUS configuration for a NES cell accepts the request, the NES cell can go to OD-SIB mode.

Regarding F1 impact, according to the agreement below, NES Cell DU needs to send the UE WUS configuration to NES Cell CU.

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU.

Since this UL WUS configuration transfer is just for neighbour node exchange, the existing GNB-DU CONFIGURATION UPDATE message and F1 SETUP REQUEST message can be reused for UL WUS configuration transfer from NES Cell DU to NES Cell CU.

Proposal 4: NES Cell DU sends UL WUS configuration to NES Cell CU in GNB-DU CONFIGURATION UPDATE message and F1 SETUP REQUEST message.

## OD SIB1 activation/deactivation

In last RAN3 meeting, three options for OD SIB1 activation was discussed:

- Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.

- Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of an explicit “UL config provided” message from Cell A gNB.

- Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs. NES cell goes to OD-SIB Mode and Cell A gNB broadcasts received UL WUS configuration via its SIB upon transmission/reception of an explicit “UL WUS configuration broadcast indication” message from NES cell gNB.

After the NES cell provide its UL WUS configuration to cell A, the NES cell can update Cell A if the current UL WUS configuration is changed or released. Meanwhile, the NES cell can activate or deactivate (switch to legacy way) OD SIB1 transmission at any time. If the NES cell is in SIB1 broadcast, the UE can receive SIB1 in legacy way; if the NES cell is in OD SIB1, the UE will request cell A for the UL WUS configuration first and then trigger the NES cell to send OD SIB1. Besides, RAN2 already agreed that “If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”. Therefore, we think it is no need to synchronize between OD SIB1 in NES cell and UL WUS configuration broadcast in Cell A, i.e., as stated in Option 1, after the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell can go to OD SIB1 Mode at any time and Cell A can start to broadcast UL WUS configuration via its SIB at any time.

If NES Cell already goes to OD SIB1 mode but Cell A has not started to broadcast UL WUS configuration, the NES cell is regarded as barred for UE. As normally NES cell is just used for capacity boost and there is always another coverage cell for UE, there will have not much impact on UE even if the NES cell is not applicable for a period of time.

If Cell A has started to broadcast UL WUS configuration but NES Cell is still in legacy SIB1 broadcast mode, as RAN2 agreed, UE will check the NES cell first and use the legacy SIB1 broadcasted from NES cell without requesting OD SIB1. Considering RAN2 agreed that “NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration)”, the new SIB for UL WUS configuration can include UL WUS configuration of multiple NES cells, so the signaling waste for broadcasting UL WUS configuration for a NES cell which is not in OD SIB1 mode is negligible.

Therefore, we think Option 1 can be adopted and an explicit start indication message no matter from NES cell or Cell A is not needed.

Proposal 5: To adopt Option 1 for UL WUS configuration provision and an explicit start indication message no matter from NES cell or Cell A is not needed.

NES cell needs to inform Cell A about the UL WUS configuration update/release, this can be achieved by reusing the new defined UL WUS Configuration Provision procedure. While considering the OD SIB1 activation/deactivation switch can be dynamic and frequent, NES cell may not need to informs cell A over Xn about the OD SIB1 activation/deactivation switch.

Proposal 6: NES cell does not inform cell A over Xn about the OD SIB1 activation/deactivation switch.

Considering there can be a scenario i.e. multiple NES cells under a single Cell A, it is beneficial to perform some coordination by Cell A on e.g. which NES cells under that Cell A can be activate OD SIB1 transmission, taking both energy saving and UE performance into account.

Proposal 7: Cell A can provide guidance to assist NES cell on OD SIB1 activation/deactivation.

In split architecture, gNB-CU is more knowledgeable on the SIB1 transmission status and load status of all NES cells and neighbor cells, so we prefer gNB-CU to decide OD SIB1 activation/deactivation of a NES cell, or at least gNB-CU should provide guidance to assist gNB-DU on OD SIB1 activation/deactivation of a NES cell.

Proposal 8: gNB-CU decides or assists gNB-DU on the OD SIB1 activation/deactivation of a NES cell, and sends the request to gNB-DU.

# Proposals

In this contribution, we analysed OD SIB1 transmission of a NES cell based on the latest agreements and the following proposals were drawn from the analysis.

Proposal 1: To define an UL WUS Configuration Provision procedure over Xn for UL WUS configuration provision from NES cell to Cell A.

Proposal 2: In case of multiple Cell A for a single NES cell, NES cell can choose which Cell A for UL WUS configuration provision.

Proposal 3: If one of the Cell A that received the UL WUS configuration for a NES cell accepts the request, the NES cell can go to OD-SIB mode.

Proposal 4: NES Cell DU sends UL WUS configuration to NES Cell CU in GNB-DU CONFIGURATION UPDATE message and F1 SETUP REQUEST message.

Proposal 5: To adopt Option 1 for UL WUS configuration provision and an explicit start indication message no matter from NES cell or Cell A is not needed.

Proposal 6: NES cell does not inform cell A over Xn about the OD SIB1 activation/deactivation switch.

Proposal 7: Cell A can provide guidance to assist NES cell on OD SIB1 activation/deactivation.

Proposal 8: gNB-CU decides or assists gNB-DU on the OD SIB1 activation/deactivation of a NES cell, and sends the request to gNB-DU.

# Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR